3GPP TSG-RAN WG4 Meeting # 99-e	R4-2111044

# Electronic Meeting, May 19 - 27, 2021

Source: 	Huawei

Title: 	TP to TR 38.860: coexistence with other services

Agenda Item:	10.4.3

Document for:	Approval

# Introduction

In this TP to TR38.860 we provide inputs to the co-existence analysis with non-3GPP services, based on the WF agreed last meeting in R4-2105421.

Referring to the WF on DTV, the following was captured:

Items for further study, e.g. - discuss the frequency separation between the lower edge of an NR band (DL) and DTV considered feasible for coexistence for each frequency arrangement, e.g. - consider/reuse the FCC frequency separation (7 MHz) for protection of Broadcast - or if this should be left for Administrations

For the frequency separation between the lower edge of the NR band (DL) and DTV channel it is proposed to conclude that due to various DTV channel arrangements across national regulations in APT, the frequency separation is considered to be decided by administrators.

It is proposed to agree on the attached TP to TR 38.860.

# TP to TR 38.860

The following TP to TR 38.860 is proposed for approval.

------------------------------ Modified section ------------------------------

## 6.3	Nearby non-3GPP services

### 6.3.1	Introduction

During the study item, multiple co-existence scenarios were discussed, with the aim to identify if dedicated requirements shall be considered in RAN4 for the possible protection of non-3GPP services.

### Coexistence between 600 MHz and TV Services

For all the frequency arrangements considered in this study item it was agreed that no specific emission limits are to be specified in RAN4 for the protection of DTV. It was decided sufficient to rely on the general BS spurious emissions limits.

Additionally, the frequency separation between the lower edge of the NR band (DL) and DTV channel was discussed. It was concluded that due to various DTV channel arrangements across national regulations, the frequency separation is considered to be decided by administrators.

The value of 7 MHz separation derived from the FCC decision on the protection separation of broadcast from the band 71 can be considered as reference.

### 6.3.3	Coexistence between 600 MHz Radio Astronomy Service (RAS)

For all the frequency arrangements considered in this study item, it was agreed that no BS spurious emissions limits be specified for protection of RAS services. Any possible protections measured required for the RAS services protection are considered to be under the responsibility of each administration.

## 6.4	Duplex filter implementation issues

----------------------------- End of modified section ------------------------------