3GPP TSG-RAN WG4 Meeting # 99e 	R4-2111262

# Electronic Meeting, May. 19-27, 2021

Source: 	vivo

Title: 	Discussion on intra-frequency measurement requirements for NR FR1 HST

Agenda Item:	9.7.2.1.2

Document for:	Discussion

Introduction

In RAN4 98e-bis, the WF R4-2105793 on FR1 HST RRM enhancements was agreed in [1].

In this paper our views on the issues related to intra-frequency measurements.

Discussion

Enhancement on CSSFoutside_gap,i for SCell measurement

In last meeting, the following issue was discussed.

- Enhancement on CSSFoutside_gap,i for SCell measurement

  - RAN4 has the following common understanding of the current non-HST specification:

    - For CSSFoutside_gap,i, both SCell(s) measured without MG and SCell(s) measured with MG are counted in NSCC_SSB.

      - Both activated and deactivated SCells are included

  - FFS the enhancement on NSCC_SSB for high speed scenario

Firstly, based on R16 version of TS 38.133, the CSSFoutsige_gap is not only for intra-frequency case but also for inter-frequency case. Therefore, we think it is not proper to list it as one enhancement for FR1 HST intra-frequency requirements.

Secondly, the CSSF calculation is not only related to FR1 only CA in NR SA, but also related to EN-DC, NR-DC, NE-DC, and CA scenarios including FR2 for NR SA. However, the WID scope of FR1 HST does not cover that many scenarios. Therefore, if there are any conclusions to enhance it in FR1 HST WI, probably the enhanced requirements only apply to the case of FR1-only CA in NR SA.

Thirdly, from UE implementation perspective, it would be very difficult set a different value of CSSF factor to some of the CA band combinations it supported, while others remain unchanged. It is also difficult to set such CSSF factor according to the network indication, e.g. in SIB, if specified in R17. Technically, it is also not reasonable to only preclude the SCells to be measured within gap from NSCC_SSB of the CSSFoutside_gap in FR1-only CA HST scenario.

On the other hand, the CSSF issue is not only related to SCells measurement, but also for spCells. Therefore, RAN4 could be careful in revising R15 spec for this issue. However, although it is almost impossible to revise R15 spec for this issue at this late stage, we see this issue is better be solved in TEI16, while UE capability is needed to keep backward compatibility. The UE feature list may also be revised accordingly. Please check our companion paper on this issue in [2],[3].

Proposal 1  Further discuss the issue of CSSFoutside_gap issue under TEI16.

Kp for deactivated SCells

In last meeting, the following was discussed.

  - PSS/SSS detection, time index detection, and measurement period for deactivated Scell

    - M2 defined in Rel-16 HST is reused for PSS/SSS detection delay requirements for deactivated Scell

    - M2 defined in Rel-16 HST is reused for time index detection delay requirements for deactivated Scell

    - Enhancement to PCell measurement in R16 HST can be reused for measurement period enhancement on deactivated Scell

    - FFS whether Rel-16 Kp requirements modifications (if any) shall also apply for R17 HST

The Kp issue is also discussed in R15/R16 maintenance. If the revision is done in R15, then there is no need to keep backward compatibility for this issue in R16. However, similar to the previous issue, the Kp issue can also be discussed in TEI16.

Although Kp is actually a relaxation for the case where partial-overlapping between SMTCs and measurement gaps exists, we see for R17 HST, a reasonable UE behaviour may still need to deal with this overlapping issue, and this relaxation is needed. Therefore, for deactivated SCells measurement, Rel-15/Rel-16 Kp requirements modification (if any) shall also apply for R17 HST.

Proposal 2  Rel-15/Rel-16 Kp requirements modification (if any) shall also apply for R17 HST.

Conclusions

Based on above analysis, we have following proposals.

Proposal 1  Further discuss the issue of CSSFoutside_gap issue under TEI16.

Proposal 2  Rel-15/Rel-16 Kp requirements modification (if any) shall also apply for R17 HST.

- References

[1]  R4-2105793  WF on FR1 HST RRM, CMCC.  RAN4 #98e-bis

[2]  R4-2111259  Discussion on addition of UE feature on enhanced CSSF for SCell measurements outside gaps, vivo.  RAN4 #99e

[2]  R4-2111257  CR on CSSF for SCell measurements outside gaps, vivo.  RAN4 #99e