3GPP TSG RAN WG4 Meeting #99-e	R4-2111312

Electronic Meeting, May 19-27, 2021

Agenda Item:	9.10.2.3

Source: 	Ericsson

Title:	Further analysis of requirements for network controlled small gap

Document for:	Discussion

- Introduction

In the last RAN4 meeting a WF capturing agreements and open issues related to network controlled small gap (NCSG) was approved [1].

In this contribution we further analyze the open issues related to the network controlled small gap (NCSG) identified in the last meeting.

# Use cases and scenarios for using NCSG pattern

The following was agreed regarding use cases and scenarios for NCSG [1].

- The use cases of NR NCSG can be

  - Eliminate/reduce interruption rate and interruption length/duration due to measurements on [deactivated Scell, Scell with dormant BWP or unused RF chain

  - FFS on

    - intra-frequency measurements with MG,

    - inter-frequency measurements with MG,

    - inter-RAT measurements,

Intra-frequency and inter-frequency measurements are almost always used. In EN-DC and NE-DC, inter-RAT measurements are quite crucial. Therefore, in our view NCSG should also be used for intra-frequency, inter-frequency and inter-RAT measurements.

We therefore support the following basic scenarios for using NCSG to avoid interruptions in NR:

- Observation # 1: Intra-frequency and inter-frequency measurement scenarios are fundamental.

- Observation # 2: Inter-RAT measurements are crucial for EN-DC and NE-DC operations.

- Proposal # 1: NCSG is used for the following measurement scenarios:

    - intra-frequency measurements with MG,

    - inter-frequency measurements with MG,

    - inter-RAT measurements

# NCSG patterns

The following was agreed regarding the NCSG patterns [1].

- The general NCSG design principle :

  - Option 1. Define NCSG patterns for All 26 MG patterns in Rel16

  - Option 2 Define NCSG patterns for subset of the legacy MG patterns in [TS38.133 v16.5.0]

  - FFS on which subset of legacy MG patterns

- The NCSG gap patterns are defined based on the absolute RF retuning time or not?

  - Option 1: NO, based on a generic interrupted duration [FFS]

    - Option1-1: ML=MGL-VIL1-VIL2

    - Option1-2: ML=legacy MG window length

  - Option 2: Yes, based on the RF retuning time (RRT). ML=MGL-RRT1-RRT2

    - FFS on the length of RRT

One important principle for defining NCSG pattern is to ensure that the entire SMTC window is available during the ML of the NCSG for measurements. This will guarantee that the UE can perform the measurements and meet the measurement requirements like when they are performing using legacy measurement gaps.

We therefore support option 2 except that RRT1 = RRT2 = RF retuning time (RRT), which depends on the FR according to TS 38.133 (e.g. sections 9.2.1 and 9.3.1):

- RRT=0.5 ms for FR1 and

- RRT = 0.25 ms for FR2

Therefore, ML should be:

- ML = Legacy MGL – 1 ms for FR1 and

- ML = Legacy MGL – 0.5 ms for FR2

VIL1 and VIL2 are at least equal to the RF tuning time. But in practice, VIL1/VIL2 may be longer e.g. considering same value for different SCS, interruption requirements etc. Also, for example, VIL1 and VIL2 depends on whether the NCSG is used in synchronous or asynchronous operation. For asynchronous case the VIL/2 is typically longer than that in synchronous case. The comparison between legacy MG pattern and NCSG in synchronous and asynchronous is illustrated in figure 1:

Legacy MG

NCSG

(synchronous)

VIL1

ML=Legacy MGL-2*RRT

VIL2

VIL1

ML=Legacy MGL-2*RRT

VIL2

NCSG

(asynchronous)

RRT

RRT

Effective MGL

Figure 1: Comparison between legacy and NCSG gap patterns

Based on the interruption requirements (table 1), the VIL1/VIl2 for asynchronous case will be one slot longer than those for synchronous case.

Table 1: Interruption length at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |

To reduce number of patterns ee propose that VIL1 and VIL2 for FR1 and FR2 are defined agnostic to SCS. Furthermore, for FR2 only one value can be defined for synchronous and asynchronous cases. Therefore, VIL1 and VIL2 can be expressed as shown in table 2:

Table 2: Proposed values of VIL1 and VIL2 for FR1 and FR2

| FR | VIL1 |  | VIL2 |  |
| --- | --- | --- | --- | --- |
|  | Sync | Async | Sync | Async |
| FR1 | 1 ms | 2 ms | 1 ms | 2 ms |
| FR2 | 0.75 ms |  | 0.75 ms |  |

Another implication of VIL1/VIL2 for different scenarios is that the actual duration of the MGL in NCSG becomes longer than the MGL used in legacy gap pattern. We therefore suggest that NCSG patterns are defined only for legacy gaps with larger MGL e.g. 6 ms or 5.5 ms. We therefore support option 2. At least NCSG corresponding to legacy patterns # 0, # 1, #13 and #14 are defined. These patterns are also mandatory from Rel-15.

- Proposal # 2: Define NCSG patterns for subset of the legacy MG patterns in TS 38.133.

- Proposal # 3: Define selected NCSG patterns with larger MGL e.g. 5.5 ms-6 ms.

- Proposal # 4: Define NCSG patterns for synchronous and asynchronous operations corresponding to legacy gap patterns with ID # 0, # 1, #13 and # 14.

- Proposal # 5: VIL1 and VIL2 for FR1 and FR2 are defined agnostic to SCS to limit the NCSG patterns.

- Proposal # 6: VIL1 and VIL2 for FR1 and FR2 are defined as follows:

| FR | VIL1 |  | VIL2 |  |
| --- | --- | --- | --- | --- |
|  | Sync | Async | Sync | Async |
| FR1 | 1 ms | 2 ms | 1 ms | 2 ms |
| FR2 | 0.75 ms |  | 0.75 ms |  |

- Proposal # 7: Define different NCSG patterns for synchronous and asynchronous operations in FR1 and same NCSG patterns for synchronous and asynchronous operations in FR2.

- Proposal # 8: ML should be sufficiently long enough to contain SMTC window.

- Proposal # 9: ML is defined as follows:

      - ML = Legacy MGL – 2*RRT

      - Where: RRT = 0.5 ms for FR1 and 0.25 ms for FR2

# Impact on RRM requirements due to NCSG

The following was agreed regarding the impact of the NCSG pattern on RRM requirements [1].

- FFS on Interruption requirements

  - Option 1: The interruption requirements in TS38.133 and TS36.133 shall be revisited

  - Option 2: Existing interruption requirements for SCell activation/deactivation can serve as starting point for the study of VIL requirements

  - Option 3: the interruption is proposed as following

  - Option 4: RAN4 to further discuss the condition, capability and impacts to measurement requirements for UE to use NCSG to control interruptions due to measurement on deactivated SCC or Scell in dormancy

- FFS on CSSF when NCSG is configured

- FFS on the existing measurement mode requirements (effective MGRP, data scheduling depends on gap configuration) can be the baseline

- FFS on Per-UE or Per-FR capability support

  - Option 1:per UE and per FR NCSG for RRM measurement needs the specific UE capability.

  - Option 2:  No additional NCSG capability for per-UE and per-FR differentiation is needed

  - Others

We don’t see any reason to revisit the interruption requirements defined in TS38.133 and TS36.133 when the UE measures without gaps and causes interruption on serving cells. It is important that existing interruption requuirements are met otherwise the use of NCSG will be less attractive.

NCSG patterns will also be defined as per UE and per FR as the following was agreed regarding per FR and per UE NCSG:

Support both per FR and per UE NCSG patterns in Rel17

It is reasonable to reuse the existing per FR gap UE capability for per UE NCSG capability. This means UE indicating per FR gap capability also supports per NCSG gaps for that FR if the UE supports NCSG.

- Proposal # 10: Existing interruption requirements for SCell activation/deactivation can serve as starting point for the study of VIL requirements

- Proposal # 11: Per FR NCSG reuses the existing per FR UE capability

# Measurement applicability

The following was agreed regarding the measurement applicability when NCSG pattern is used [1].

- FFS on whether NCSG can be configured simultaneously with legacy gap pattern

  - Option 1. Yes.

    - FFS on how to apply the measurement requirements (e.g. CSSF) under this assumption

  - Option 2. In the first phase of the WI , No

    - FFS on how to apply the measurement requirements (e.g. CSSF) under this assumption

- FFS on RF combination limitation

  - Option 1. UE is not expected to measure 2 inter-freq/RAT layers in parallel even if UE reports the support of NCSG to both corresponding bands

  - Others

- FFS on Rx beam limitation

  - Option 1. : NW needs to be informed that the inter-frequency measurements with NCSG is CBM or IBM with serving cells in FR2.

  - Option 2: NCSG in FR2 should be deprioritized in current stage.

  - Others

- FFS on scheduling and measurement restriction

  - Option 1: When NCSG is configured then during the ML the existing scheduling restriction requirements defined in TS 38.133 shall also apply

  - Others

In the first phase of the WI the UE can be configured only with NCSG pattern. In the second phase of the WI the simultaneous configuration of the NCSG pattern and the legacy gap pattern can be considered.

When configured with NCSG then the UE should be able to perform measurements on serving carriers in parallel over their respective measurement periods. In this case no scaling factor (CSSF) is needed to scale the measurement periods.

If the UE is configured to measure on non-serving carriers using NCGS then the measurement periods for measurements on non-serving carriers need to be scaled by the scaling factor (CSSF) as done for measurement with gaps.

If the UE is configured to measure on serving and non-serving carriers using NCGS then NCSG gaps are split for between measurements on serving and non-serving carriers using the gap sharing parameter (X), which in turns impact the scaling factor (CSSF) determination.

In summary the UE is not expected to measure 2 inter-frequency/RAT layers in parallel even if the UE reports the support of NCSG to both corresponding bands.

We don’t agree to downpriortize NCSG for FR2. Whether the network needs to be informed by the UE that the inter-frequency measurements with NCSG is CBM or IBM with serving cells in FR2 can be further discussed when basic aspects of NCSG are progressed.

The existing scheduling requirements defined for FR1 and FR2 can apply also during the ML.

- Proposal # 12: In the first phase of the WI consider only the scenario in which the UE is configured only with NCSG pattern.

- Proposal # 13: The UE is not expected to measure 2 inter-frequency/RAT layers in parallel even if UE reports the support of NCSG to both corresponding bands

- Proposal # 14: NCSG pattern is also supported for FR2 i.e. NCSG is NOT downpriotized for FR2.

- Proposal # 15: When NCSG is configured then during the ML the existing scheduling restriction requirements defined in TS 38.133 shall also apply.

# Signaling aspects

The following was agreed regarding the signaling needed to introduce NCSG pattern [1].

- FFS on How to consider the relation between NCSG and ‘NeedForGap’?

  - Option 1 :The “NeefForGap” signaling structure can be reused for NR NCSG as a start point

  - Option 1a :Rel-17 NCSG capability is reported on top of existing RAN2 ‘NeedForGap’ signaling structure with a new component ‘NCSG’.

  - Option 1b: Extend the Rel-16 NeedForGap ignaling for UE to indicate

    - Need for normal MG, or

    - No need for normal MG but NCSG, or

    - No need for either normal MG or NCSG

  - Option 2 :Don’t reuse Rel-16 ‘NeedForGap’ signaling for NCSG

  - Option 3: Let RAN2 decide NCSG signaling details and any relation between NCSG and ‘NeedForGap’ based on RAN4 technical input on NCSG pattern design

  - Option 4 : FFS when the NCSG pattern design as well as NCSG applicability and UE capability support are finalized

As stated in our earlier contribution that it is not certain at this stage if the “NeedForGap” structure can be reused or some modification is needed e.g. to add separate option of NCSG or replace ‘no gap’ with NCSG [2]. The relation between NCSG and ‘NeedForGap’ signaling structure needs RAN2 expertise.

One important thing is that the introduction of NCSG signalling does not cause any backward compatibility problem. Therefore, NCSG is defined in a way that the existing signaling for need for gaps is maitined and is comprehensible for the legacy network. While signaling is up toRAN2, but in our view new separate signaling for NCSG is added without any change to the current signaling structure as shown below (NCSG signaling as shown by yellow text):

NeedForGapsInfoNR-r16 ::=        SEQUENCE {

intraFreq-needForGap-r16      NeedForGapsIntraFreqlist-r16,

interFreq-needForGap-r16      NeedForGapsBandlistNR-r16

}

NeedForGapsIntraFreqlist-r16 ::=          SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForGapsIntraFreq-r16

NeedForGapsBandlistNR-r16 ::=             SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16

NeedForGapsIntraFreq-r16  ::=                 SEQUENCE {

servCellId-r16                               ServCellIndex,

gapIndicationIntra-r16                       ENUMERATED {gap, no-gap}

}

NeedForGapsNR-r16  ::=                        SEQUENCE {

bandNR-r16                                   FreqBandIndicatorNR,

gapIndication-r16                            ENUMERATED {gap, no-gap}

}

NeedForGapsInfoNR-r17 ::=        SEQUENCE {

intraFreq-needForShortGap-r17   NeedForShortGapsIntraFreqlist-r17,

interFreq-needForShortGap-r17   NeedForShortGapsBandlistNR-r17

}

NeedForShortGapsIntraFreqlist-r17 ::=          SEQUENCE (SIZE (1.. maxNrofServingCells)) OF ServCellIndex

NeedForShortGapsBandlistNR-r17 ::=             SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR

The current need for gap signalling structure is not extensible. We therefore do not support options 1, 1a and 1b. RAN4 should provide RAN4 agreements on NCSG patterns and details and let RAN2 decide the NCSG signaling details.

- Observation # 2: NCSG capability signaling should not cause backward compatibility problem for legacy network not comprehending NCSG.

- Observation # 3: NeefForGap signaling structure is not extensible.

- Proposal # 16: NeefForGap signaling structure is not reused for NCSG

- Proposal # 17: Let RAN2 decide NCSG signaling details and any relation between NCSG and ‘NeedForGap’ based on RAN4 technical input on NCSG pattern design.

- Summary and Proposals

In this paper we have provided further analysis of using NCSG gaps for different use cases and scenarios based on the last WF [1]. Based on the analysis following are the main proposals:

Scenarios for NCSG patterns:

- Observation # 1: Intra-frequency and inter-frequency measurement scenarios are fundamental.

- Observation # 2: Inter-RAT measurements are crucial for EN-DC and NE-DC operations.

- Proposal # 1: If NCSG is used for the following measurement scenarios:

    - intra-frequency measurements with MG,

    - inter-frequency measurements with MG,

    - inter-RAT measurements

NCSG patterns:

- Proposal # 2: Define NCSG patterns for subset of the legacy MG patterns in TS 38.133.

- Proposal # 3: Define selected NCSG patterns with larger MGL e.g. 5.5 ms-6 ms.

- Proposal # 4: Define NCSG patterns for synchronous and asynchronous operations corresponding to legacy gap patterns with ID # 0, # 1, #13 and # 14.

- Proposal # 5: VIL1 and VIL2 for FR1 and FR2 are defined agnostic to SCS to limit the NCSG patterns.

- Proposal # 6: VIL1 and VIL2 for FR1 and FR2 are defined as follows:

| FR | VIL1 |  | VIL2 |  |
| --- | --- | --- | --- | --- |
|  | Sync | Async | Sync | Async |
| FR1 | 1 ms | 2 ms | 1 ms | 2 ms |
| FR2 | 0.75 ms |  | 0.75 ms |  |

- Proposal # 7: Define different NCSG patterns for synchronous and asynchronous operations in FR1 and same NCSG patterns for synchronous and asynchronous operations in FR2.

- Proposal # 8: ML should be sufficiently long enough to contain SMTC window.

- Proposal # 9: ML is defined as follows:

      - ML = Legacy MGL – 2*RRT

      - Where: RRT = 0.5 ms for FR1 and 0.25 ms for FR2

Impact on RRM requirements due to NCSG:

- Proposal # 10: Existing interruption requirements for SCell activation/deactivation can serve as starting point for the study of VIL requirements.

- Proposal # 11: Per FR NCSG reuses the existing per FR UE capability.

Measurement applicability:

- Proposal # 12: In the first phase of the WI consider only the scenario in which the UE is configured only with NCSG pattern.

- Proposal # 13: The UE is not expected to measure 2 inter-frequency/RAT layers in parallel even if UE reports the support of NCSG to both corresponding bands

- Proposal # 14: NCSG pattern is also supported for FR2 i.e. not NCSG is NOT downpriotized for FR2.

- Proposal # 15: When NCSG is configured then during the ML the existing scheduling restriction requirements defined in TS 38.133 shall also apply.

Signaling aspects:

- Observation # 2: NCSG capability signaling should not cause backward compatibility problem for legacy network not comprehending NCSG.

- Observation # 3: NeefForGap signaling structure is not extensible.

- Proposal # 16: NeefForGap signaling structure is not reused for NCSG.

- Proposal # 17: Let RAN2 decide NCSG signaling details and any relation between NCSG and ‘NeedForGap’ based on RAN4 technical input on NCSG pattern design.

- References

- R4-2105792, “WF on R17 NR MG enhancements – NCSG”, Intel.

- R4-2107152, “Analysis of requirements for network controlled small gap”, Ericsson