3GPP TSG-RAN WG4 Meeting #99-e                                          R4-2111450

Electronic Meeting, May 19-27, 2021

Title:	draft reply LS on on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities

Response to:	R2-2104550/ R4-2107620

Release:	Rel-15

Work Item:	NR_newRAT

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name: 	Liu Ye

E-mail Address: 	leo.liuye@huawei.com

Attachments:	N/A

1. Overall Description:

RAN4 would like to thank RAN2 for the LS on draft reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities.

The (NG)EN-DC/NE-DC BC types in RAN2 LS are cited below as a reference.

- Type 1: Intra-band (NG)EN-DC/NE-DC combination without additional inter-band NR and LTE CA component, e.g. DC 41A_n41A

- Type 2: Intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component, e.g. DC_25A_41A_n41A

- Type 3: Intra-band (NG)EN-DC/NE-DC combination without supporting UL in both the bands of the intra-band (NG)EN-DC/NE-DC UL part, e.g. DC_25A_41A_n41A

- Type 4: Inter-band (NG)EN-DC/NE-DC combination without Intra-band component, in short we call it as Inter-band (NG)EN-DC/NE-DC combination.

- Type 5: Inter-band (NG)EN-DC combination configurations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g., DC_B42_n77 and DC_B42_n78.

Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/pa-PhaseDiscontinuityImpacts/dualPA-Architecture/simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable?

Answer 1: The applicability of the capabilities fo the BC types are listed as following.

Table 1: Applicability of the capability

|  | Type 1 | Type 2 | Type 3 | Type 4 | Type 5 |
| --- | --- | --- | --- | --- | --- |
| ul-TimingAlignmentEUTRA-NR | √ | √ | × | × | × |
| pa-PhaseDiscontinuityImpacts | √ | √ | × | × | × |
| dualPA-Architecture | √ | √ | × | × | × |
| simultaneousRxTxInterBandENDC | × | √ | √ | √ | √ |
| asyncIntraBandENDC | √ | √ | × ? | × | × |

Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /ul-dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part?

Answer 2: The capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /ul-dualPA-Architecture are UL related for intra-band (NG)EN-DC/NE-DC. Since UL is not supported for Type 3 BC, these capabiliities are not applicable. In addition, asyncIntraBandENDC is a capability for intra-band EN-DC, if Type 3 BC without UL can still be considered as intra-band EN-DC combination, RAN4’s understanding is the the capability is applicable, otherwise, the capability may not be applicable. Which also needs further clarification by RAN2.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above information into account.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#100-e                      Aug 2021											  E-meeting

TSG-RAN4 Meeting#101-e                      Nov 2021											  E-meeting