3GPP TSG-RAN WG4 Meeting #99-e  		                                          R4-2111462

Electronic Meeting, 19th-27th of May, 2021

# Source:	Thales

# Title:	On the S-band NTN Scenarios and Parameters for Coexistence Study

# Type:	discussion

# Document for:	discussion

# Agenda Item:	9.12.2.1

Release:	Rel-17

# Introduction

RAN4 #98-bis-e Agreements (20th of April 2021, GTW session):

-------------Additional agreements--------------

Further update the table in slide 4 as following:

Table 1.4-2 Interference Table

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN - NTN | TN DL | NTN DL |  | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL |  | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL |  | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL |  | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 7 | TN with NTN | TN UL | NTN DL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 2 |
| 8 | TN with NTN | NTN DL | TN UL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 1 |
| 9 | NTN with NTN | NTN DL | NTN DL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |

During RAN4 #98-bis-e, the following agreement has been captured in the Chairman Notes on 20th of April 2021 during the GTW session, with respect to the contribution R4-2106104 (“WF on [308] NTN_Solutions_Part2” – coexistence studies):

[https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_98bis_e/Inbox/Chairman_Notes]

RAN4 #98-bis-e Agreements (20th of April 2021, GTW session):

- TN NR: 20MHz

- NTN Single carrier BW: [5MHz] (FRF=3), [15MHz/30MHz] (FRF=1)

- To consider FRF in 2 phases as following:

- FRF=1 in phase 1 for simplification.

- FRF=3 in phase 2 or it is found FRF=1 is too stringent.

Moreover, the following agreement has been also made:

RAN4 #98-bis-e Agreements (R4-2106104):

- The satellite max Tx power can be calculated by the equation as below:

maxTx power(dBm)=EIRP density (dBW/MHz)+30+10 log10NRB*SCS*12-Max Gain

- Bandwidth will be further discussed.

In R4-2106104 it can be also found that

Therefore, the goal of this contribution is to further clarify the coexistence scenarios to be considered by RAN4 studies, and their related simulation parameters.

# S-band NTN Coexistence Scenarios

Using the previous RAN4#98bis-e agreements together with R4-2107270 (“On the S-band NTN coexistence scenarios and simulation parameters”), is possible to further describe more in detail the interference table and the possible interferences types for adjacent S-band coexistence that could be handled by RAN4 work.

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 (i1) | TN with NTN | TN DL | NTN DL | Coexistence with FDD band. | Phase 1 |
| 2 (i2) | TN with NTN | TN UL | NTN UL | Coexistence with FDD band. | Phase 1 |
| 3 (i3) | TN with NTN | NTN DL | TN DL | Coexistence with FDD band. | Phase 1 |
| 4 (i4) | TN with NTN | NTN UL | TN UL | Coexistence with FDD band. | Phase 1 |
| 5 (i5) | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 6 (i6) | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 7 (i7) | TN with NTN | TN UL | NTN DL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 2 |
| 8 (i8) | TN with NTN | NTN DL | TN UL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 1 |
| 9 | NTN with NTN | NTN DL | NTN DL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |

Please note that the scenarios 1-6 (and interference types i1-i6) have been already discussed in previous meetings and also discussed in R4-2107270, as represented below in Figures 1 and Figure 2.

Figure 1: S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode

Figure 2: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n34)

However, scenarios 7 and 8 (interference types i7 and i8) have not been described before and are represented now in Figure 3 below.

Figure 3: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n41)

Proposal 1: RAN4 shall update “Simulation assumptions for NTN co-existence study” document with the Figure 1, the Figure 2 and the Figure 3.

# Updated S-band Simulation Parameters

The current simulation assumptions from R4-2106105 are:

The satellite max Tx power can be calculated by the equation as below:

maxTx power(dBm)=EIRP density (dBW/MHz)+30+10 log10NRB*SCS*12-Max Gain

Table 2.3-1 Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Channel bandwidth |  | [5]/[15]/[30]MHz | [5]/[15]/[30]MHz | [5]/[15]/[30]MHz |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 m |
| Payload characteristics for UL transmissions |  |  |  |  |
| G/T | 2 GHz | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |

We propose to update these simulation assumptions with a more explicit equation:

maxTx power[dBm]=EIRP density[dBWMHz]+30+10 log10NRB*SCSMHz*12-Max Gain[dBi]

and with the following table with respect to NRB configuration per BandWidth size and SCS:

| Configuration FR1 S-band | NRB (5MHz BW) | NRB (10MHz BW) | NRB (15MHz BW) | NRB (30MHz BW) |
| --- | --- | --- | --- | --- |
| SCS 15 kHz | 25 | 52 | 79 | 160 |
| SCS 30 kHz | 11 | 24 | 38 | 78 |

This will further result into with an updated table for the Satellite max Tx power in dBm as a function of BW size and SCS configuration:

Set-1 satellite parameters for co-existence study (TR 38.821)

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Satellite max TX power in dBm |  | SCS15: 44.53; 47.71; 49.53; 52.59dBm SCS30: 43.98; 47.37; 49.36; 52.48dBm | SCS15: 46.53; 49.71; 51.53; 54.59dBm SCS30: 45.98; 49.37; 51.36; 54.48dBm | SCS15: 40.53; 43.71; 45.53; 48.59dBm SCS30: 39.98; 43.37; 45.36; 48.48dBm |
| Channel bandwidth |  | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 km |

The satellite max Tx power can be calculated by the equation as below:

maxTx power[dBm]=EIRP density[dBWMHz]+30+10 log10NRB*SCSMHz*12-Max Gain[dBi]

NRB configuration per BandWidth size and SCS:

| Configuration FR1 S-band | NRB (5MHz BW) | NRB (10MHz BW) | NRB (15MHz BW) | NRB (30MHz BW) |
| --- | --- | --- | --- | --- |
| SCS 15 kHz | 25 | 52 | 79 | 160 |
| SCS 30 kHz | 11 | 24 | 38 | 78 |

Set-1 satellite parameters for co-existence study (TR 38.821)

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Satellite max TX power in dBm |  | SCS15: 44.53; 47.71; 49.53; 52.59dBm SCS30: 43.98; 47.37; 49.36; 52.48dBm | SCS15: 46.53; 49.71; 51.53; 54.59dBm SCS30: 45.98; 49.37; 51.36; 54.48dBm | SCS15: 40.53; 43.71; 45.53; 48.59dBm SCS30: 39.98; 43.37; 45.36; 48.48dBm |
| Channel bandwidth |  | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 km |

Proposal 2: RAN4 shall update “Simulation assumptions for NTN co-existence study” document with the following information:

If we consider a constant density of users per km2, then the previous table can be updated with the following information:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| NTN UE Assumptions (Distribution, Density/km2) for 15 MHz and 30 MHz configurations (FRF1) |  |  |  |  |
| Satellite Beam Diameter | 2GHz | 250 km | 90 km | 50 km |
| NTN UE Density |  | 0.1 NTN UEs/km2 | 0.1 NTN UEs/km2 | 0.1 NTN UEs/km2 |
| Satellite Spectrum Efficiency Assumption |  | 1.35bit/sec/Hz | 1.35bit/sec/Hz | 1.35bit/sec/Hz |
| Channel Bandwidth Assumption per Beam |  | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz |
| Maximum Throughput per Beam |  | 6.75; 13.5; 20.25; 40.5 Mbps | 6.75; 13.5; 20.25; 40.5 Mbps | 6.75; 13.5; 20.25; 40.5 Mbps |
| Number of NTN UEs per Beam |  | 4908 NTN UEs/beam | 636 NTN UEs/beam | 196 NTN UEs/beam |
| Mean Throughput Rate per (Active) NTN UE |  | 30 MHz assumption: 8.25 kbps 15 MHz assumption: 4.12 kbps | 30 MHz assumption: 63.68 kbps 15 MHz assumption: 31.84 kbps | 30 MHz assumption: 205.35 kbps 15 MHz assumption: 102,67 kbps |

Another approach would be to set up the minimum achievable throughput per service and derive the density of the users based on this approach. If we consider a service type with a (minimum) requirement of 250 kbps (as in TR 38.821), then we will obtain:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| NTN UE Assumptions (Distribution, Density/km2) for 15 MHz and 30 MHz configurations (FRF1) |  |  |  |  |
| Satellite Beam Diameter | 2GHz | 250 km | 90 km | 50 km |
| Mean Throughput Rate per (Active) NTN UE |  | 250 kbps | 250 kbps | 250 kbps |
| Satellite Spectrum Efficiency Assumption |  | 1.35bit/sec/Hz | 1.35bit/sec/Hz | 1.35bit/sec/Hz |
| Channel Bandwidth Assumption per Beam |  | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz |
| Maximum Throughput per Beam |  | 6.75; 13.5; 20.25; 40.5 Mbps | 6.75; 13.5; 20.25; 40.5 Mbps | 6.75; 13.5; 20.25; 40.5 Mbps |
| Number of NTN (Active) UEs per Beam |  | 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam | 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam | 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam |
| NTN UE Density per Given Service Type (for Simulation Purpose) |  | 30 MHz assumption: 0.0033 NTN UEs/km2 15 MHz assumption: 0.0016 NTN UEs/km2 | 30 MHz assumption: 0.0255 NTN UEs/km2 15 MHz assumption: 0.0127 NTN UEs/km2 | 30 MHz assumption: 0.0825 NTN UEs/km2 15 MHz assumption: 0.0412 NTN UEs/km2 |

Further, we propose to include the NTN UE density assumptions in the “Simulation assumptions for NTN co-existence study” document.

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| NTN UE Assumptions (Distribution, Density/km2) for 15 MHz and 30 MHz configurations (FRF1) |  |  |  |  |
| Satellite Beam Diameter | 2GHz | 250 km | 90 km | 50 km |
| Mean Throughput Rate per (Active) NTN UE |  | 250 kbps | 250 kbps | 250 kbps |
| Satellite Spectrum Efficiency Assumption |  | 1.35bit/sec/Hz | 1.35bit/sec/Hz | 1.35bit/sec/Hz |
| Channel Bandwidth Assumption per Beam |  | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz |
| Maximum Throughput per Beam |  | 6.75; 13.5; 20.25; 40.5 Mbps | 6.75; 13.5; 20.25; 40.5 Mbps | 6.75; 13.5; 20.25; 40.5 Mbps |
| Number of NTN (Active) UEs per Beam |  | 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam | 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam | 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam |
| NTN UE Density per Given Service Type (for Simulation Purpose) |  | 30 MHz assumption: 0.0033 NTN UEs/km2 15 MHz assumption: 0.0016 NTN UEs/km2 | 30 MHz assumption: 0.0255 NTN UEs/km2 15 MHz assumption: 0.0127 NTN UEs/km2 | 30 MHz assumption: 0.0825 NTN UEs/km2 15 MHz assumption: 0.0412 NTN UEs/km2 |

Proposal 3: RAN4 shall update “Simulation assumptions for NTN co-existence study” document with the following information:

Another observation is that the maximum BW size used for NTN coexistence simulation purpose in MSS S-Band should consider the same maximum configurable BW size as for TN n65.

Proposal 4: The maximum BW size used for NTN coexistence simulation purpose in MSS S-Band should consider the same maximum configurable BW size as for TN n65.

Note1: It should be considered that the TN UE operating in n65 has the capability to operate also in the considered NTN MSS S-Band, since the UE uses the same implementation for both TN and NTN connectivity. For this reason, for the coexistence analysis, same BW configuration should be probably used for both TN and NTN.

Note2: The TN maximum BW size in n65 is currently 20 MHz, and therefore NTN maximum supported BW size in MSS S-Band should also be 20 MHz.

Note3: Simulation assumptions may be further updated accordingly.

Further, it would be useful to differentiate the number of BSs or UEs (terrestrial or non-terrestrial) or their density to use for simulation, as a function of interference type. More precisely, depending on who is the aggressor, and who is the victim, the simulation approach may be different. For instance, for generating interference, we can consider multiple sources, but for measuring interference, we can use one destination at a time/in different locations (e.g. we randomly change position of the destination over the simulation time). This approach can be also generalized for both NTN and TN, as we could apply the same considerations also for TN.

Proposal 5: RAN4 should consider the number of TN UEs or their respective densities as a function of interference type.

Proposal 6: RAN4 should consider the number of NTN UEs or their respective densities as a function of interference type.

Proposal 7: Depending on who is the aggressor, and who is the victim, the simulation approach may be different. For generating interference, RAN4 simulations may consider multiple sources, but for measuring interference, RAN4 simulation may use one single destination in different locations.

For example, with respect to the NTN UEs required for the simulation, the simulator can be configured differently with respect to simulation purpose:

- NTN UE as destination: For NTN UE as a victim, the simulator could consider one NTN UE (that is considered in the network at several random positions, and then the results are averaged by Monte-Carlo simulations);

- NTN UE as source: For NTN UE as an aggressor, the simulator could consider an NTN UE density (or number of NTN users/beam);

In any case, NTN simulation assumptions should follow similar approach as for the TN simulation assumptions.

Proposal 8: NTN simulation assumptions should follow similar Monte-Carlo simulation approach as for the TN simulation assumptions.

Proposal 9: If TN simulation assumptions are using one single TN UE scheduled approach, NTN simulation assumptions should also use one single NTN UE scheduled approach.

Proposal 10: If TN simulation assumptions are using a density of TN UEs, NTN simulation assumptions should also use a density of NTN UEs.

Moreover, from the service perspective, a UE with both NTN and TN capabilities will most probably connect to the network providing higher QoS. For this reason, we expect less NTN UEs inside a TN network, and therefore the NTN UEs will probably be at the edge of the TN cell or even outside the TN cell.

Proposal 11: The simulation assumptions should consider UEs connecting with higher priority to the network providing higher QoS.

Proposal 12: The simulation assumptions could be further simplified with NTN UEs at the edge of TN network or outside TN network.

# Conclusions

Proposal 1: RAN4 shall update “Simulation assumptions for NTN co-existence study” document with the Figure 1, the Figure 2 and the Figure 3.

Figure 1: S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode

Figure 2: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n34)

Figure 3: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n41)

The satellite max Tx power can be calculated by the equation as below:

maxTx power[dBm]=EIRP density[dBWMHz]+30+10 log10NRB*SCSMHz*12-Max Gain[dBi]

NRB configuration per BandWidth size and SCS:

| Configuration FR1 S-band | NRB (5MHz BW) | NRB (10MHz BW) | NRB (15MHz BW) | NRB (30MHz BW) |
| --- | --- | --- | --- | --- |
| SCS 15 kHz | 25 | 52 | 79 | 160 |
| SCS 30 kHz | 11 | 24 | 38 | 78 |

Set-1 satellite parameters for co-existence study (TR 38.821)

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Satellite max TX power in dBm |  | SCS15: 44.53; 47.71; 49.53; 52.59dBm SCS30: 43.98; 47.37; 49.36; 52.48dBm | SCS15: 46.53; 49.71; 51.53; 54.59dBm SCS30: 45.98; 49.37; 51.36; 54.48dBm | SCS15: 40.53; 43.71; 45.53; 48.59dBm SCS30: 39.98; 43.37; 45.36; 48.48dBm |
| Channel bandwidth |  | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 km |

Proposal 2: RAN4 shall update “Simulation assumptions for NTN co-existence study” document with the following information:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| NTN UE Assumptions (Distribution, Density/km2) for 15 MHz and 30 MHz configurations (FRF1) |  |  |  |  |
| Satellite Beam Diameter | 2GHz | 250 km | 90 km | 50 km |
| Mean Throughput Rate per (Active) NTN UE |  | 250 kbps | 250 kbps | 250 kbps |
| Satellite Spectrum Efficiency Assumption |  | 1.35bit/sec/Hz | 1.35bit/sec/Hz | 1.35bit/sec/Hz |
| Channel Bandwidth Assumption per Beam |  | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz |
| Maximum Throughput per Beam |  | 6.75; 13.5; 20.25; 40.5 Mbps | 6.75; 13.5; 20.25; 40.5 Mbps | 6.75; 13.5; 20.25; 40.5 Mbps |
| Number of NTN (Active) UEs per Beam |  | 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam | 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam | 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam |
| NTN UE Density per Given Service Type (for Simulation Purpose) |  | 30 MHz assumption: 0.0033 NTN UEs/km2 15 MHz assumption: 0.0016 NTN UEs/km2 | 30 MHz assumption: 0.0255 NTN UEs/km2 15 MHz assumption: 0.0127 NTN UEs/km2 | 30 MHz assumption: 0.0825 NTN UEs/km2 15 MHz assumption: 0.0412 NTN UEs/km2 |

Proposal 3: RAN4 shall update “Simulation assumptions for NTN co-existence study” document with the following information:

Proposal 4: The maximum BW size used for NTN coexistence simulation purpose in MSS S-Band should consider the same maximum configurable BW size as for TN n65.

Note1: It should be considered that the TN UE operating in n65 has the capability to operate also in the considered NTN MSS S-Band, since the UE uses the same implementation for both TN and NTN connectivity. For this reason, for the coexistence analysis, same BW configuration should be probably used for both TN and NTN.

Note2: The TN maximum BW size in n65 is currently 20 MHz, and therefore NTN maximum supported BW size in MSS S-Band should also be 20 MHz.

Note3: Simulation assumptions may be further updated accordingly.

Proposal 5: RAN4 should consider the number of TN UEs or their respective densities as a function of interference type.

Proposal 6: RAN4 should consider the number of NTN UEs or their respective densities as a function of interference type.

Proposal 7: Depending on who is the aggressor, and who is the victim, the simulation approach may be different. For generating interference, RAN4 simulations may consider multiple sources, but for measuring interference, RAN4 simulation may use one single destination in different locations.

Proposal 8: NTN simulation assumptions should follow similar Monte-Carlo simulation approach as for the TN simulation assumptions.

Proposal 9: If TN simulation assumptions are using one single TN UE scheduled approach, NTN simulation assumptions should also use one single NTN UE scheduled approach.

Proposal 10: If TN simulation assumptions are using a density of TN UEs, NTN simulation assumptions should also use a density of NTN UEs.

Proposal 11: The simulation assumptions should consider UEs connecting with higher priority to the network providing higher QoS.

Proposal 12: The simulation assumptions could be further simplified with NTN UEs at the edge of TN network or outside TN network.

END