3GPP TSG-RAN4 Meeting #99e	R4-2109336

# Electronic Meeting, 19 - 27 May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1903 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on applicability of requirements to multi-TRxP - R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN4#96e it was agreed for Rel-16 eMIMO multi-TRxP transmission [2]: - No RRM core requirement impact identified on MRTD/MTTD values specified in Rel-15; - It is RAN4 common understanding that MRTD/MTTD requirements in clauses 7.5.3, 7.6.3 and 7.6.4 is sufficient for support the deployment with multi-DCI based and single-DCI based multi-TRxP transmission. The above conclusion is not captured in the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added applicability of requirements to Multi-TRxP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear whether exisiting requirements are applicable to multi-TRxP deployment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

### 3.6.11	Applicability of MRTD/MTTD requirements in intra-band DC/CA

Unless explicitly stated otherwise the Maximum Transmission Timing Difference (MTTD) and Maximum Receive Timing Difference (MRTD) requirements in clauses 7.5.3, 7.6.3 and 7.6.4 for co-located deployment are applicable when

- When UE is configured to receive multiple PDCCH

- When UE is configured by repetitionScheme set to one of ' fdmSchemeA', ' fdmSchemeB' and 'tdmSchemeA'

End of Change 1