3GPP TSG-RAN WG4 Meeting # 99-e 												R4-2107963

Electronic Meeting, 19th – 27th May, 2021

Agenda item:			10.5.1

Source:	Moderator (China Unicom)

Title:	Email discussion summary for [99-e][153] FS_NR_PC2_UE_FDD

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

Topic #1: PC2 for NR FDD band

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2110163 | Apple | Observation 1: In FDD bands, there is no concept of duty cycle and contiguous UL transmission has always been assumed during SAR characterization. Observation 2: To support HPUE in FDD bands, 3GPP may need to accept the duty-cycled UL transmission for FDD bands. Observation 3: To support HPUE for FDD bands, duplexer power handling capability needs to be evaluated. Observation 4: For FDD bands with narrow duplex distance, further UL RB allocation restriction may be needed under HPUE operation to prevent REFSENS degradation which could render a countereffect on having HPUE to improve UL throughput near the cell edge as motivated by this study item. Observation 5: Half-duplex operation in FDD bands by nature is duty-cycled which has inherently resolved the SAR issue under HPUE scenario. Observation 6: Half-duplex operation allows bypassing the high insertion loss duplexer and avoids REFSENS impact from transmit leakages which can save UL from RB allocation restriction for FDD bands with narrow duplex distance. Proposal: Introduce half-duplex operation for NR FDD bands under HPUE scenario to enable a quick adoption for PC2 in NR FDD bands. |
| R4-2110197 | Xiaomi | Observation 1: The evaluation period when deriving the UL dutycycle shall be carefully defined. Observation 2: To guarantee BS and UE has the same knowledge of UL dutycycle, defining a unified evaluation period between BS and UE is necessary. Observation 3: Only one set of MSD value by considering the worst case between two different RF architectures is specified. |
| R4-2110829 | OPPO | Observation 1: To make sure UE capability can be guaranteed, NW need to calculate the duty cycle capability based on per frame window length with 1 symbol moving step. Observation 2: It is not meaningful to continue discussing how to align the understanding of calculation window between UE and NW since the duty cycle capability probably will never be considered in scheduling. Proposal 1: It is proposed to conclude that the FDD duty cycle capability is only required to be applied by UE itself and complete the discussion of duty cycle based SAR solution. |
| R4-2109700 | vivo | Proposal: After the connection setup, the last MSG1/MSGA of RACH before connection setup is proposed as the starting point of evaluation period. Observation: Before the connection setup, the SAR impact of MSG1/MSGA is negligible. And UE can use P-MPR to meet SAR limits. |
| R4-2110433 | ZTE | Observation 1: With the existing assumptions, no MSD degradation for band n1 and band n3 for separate PA architecture. Observation 2: There were no agreements on the new assumptions. Observation 3: Taking more aggressive duplexer assumption into account, no additional REFSEN degradation might need to be considered for PC2 band n1. Observation 4: REFSEN degradation will become more severer for PC2 band n3, especially for 40/50MHz. Nevertheless, new assumptions should be discussed and agreed first. |
| R4-2111446 | Huawei, HiSilicon | Observation 1: From the simulation results, performance gain can be observed for both cell average and cell edge cases. And extremely obvious performance gain on cell average throughput is observed. Proposal 1: It is proposed to make a conclusion on system performance gain for FDD HUPE and focus on other remaining issues for the SI. |
| R4-2109699 | vivo | Observation 1: The performance gain of FDD HPUE has been observed for both cell average and cell edge, but it does not match the actual situation in which only part of UE can perform high power. Observation 2: The data density of the current traffic model is too low to reflect the interference of HPUE. Proposal 1: It should be more reasonable to limit the proportion of HPUE in the simulation. Proposal 2: The higher data density traffic model is also needed for the performance evaluation. |
| R4-2109763 | ZTE | Observation: From the simulation results, obvious performance gains on cell average throughput and cell edge throughput can be observed for the case of 10 kB packet size. |
| R4-2109998 | LG Electronics | TP on UE implementation for PC2 FDD band |
| R4-2110798 | Qualcomm | TP to TR38.861: Simulation results for FDD HPUE |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 SAR Scheme(s)

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1-1: How to handle evaluation period for duty cycle solution

- Proposals

  - Option 1: Using UE implementation based method to handle duty cycle capability. (i.e. FDD duty cycle capability is only required to be applied by UE)

  - Option 2: UE and network using unified evaluation period, with the last MSG1/MSGA of RACH before connection setup is proposed as the starting point of evaluation period.

  - Option 3: Reuse the existing duty cycle capability as other HPUEs since R15 and R16.

- Recommended WF

  - TBA

Discussion:

Huawei: we prefer Option 3. We use the similar mechanism as HPUE in other WIs. Option 2 is to define the specific UE behaviour.

Ericsson: We do not prefer to introduction of any artificial approach. We prefer Option 1. We have test case to ensure P-MPR works and average value is kept within a certain value.

Vivo: Support Option 3. Option 3 and Option 2 do not conflict. Even if we have duty cycle we still have problem to sync network and UE.

Skyworks: For FDD PC2, we have comments that not only duty cycle but also how long. P-PMR may be used with duty cycle.

LGE: both P-MPR and duty cycle can be used for PC2 FDD. Option 1 is more preferable approach.

Huawei: P-MPR is agreed to be as baseline. We do not repeat discussion on P-MPR here. Option 1 is similar to P-MPR solution. What is additional benefit that Option 1 can provide?

Oppo: we are also OK with Option 1. Duty cycle can only be implemented by UE. For Option 3, it is quite confusing. How the exact duty cycle can be evaluated.

Apple: regarding Option 3, the existing duty cycle capability, if UE reports it to network, what does network do? Network may not do anything according to previous comment.

Ericsson: To Huawei, Oppo actually answer the question. Network does not have exact value of output power. It is artificial approach if network only consider duty cycle without considering power.

Huawei: Duty cycle capability is introduced from Rel-15. It is used in many other WIs. If network is willing to address the SAR issue, network can do it by using duty cycle.

Ericsson: Option 3 is not feasible.

Vivo: Duty cycle is still needed. It will break the link if network has no information but UE has fallback.

Oppo: What is the meaning for “left to UE implementation”.

Discussion points:

- How to handle SAR issue is left to UE implementation.

- Using UE implementation based method to handle duty cycle capability. (i.e. FDD duty cycle capability is only required to be applied by UE)

  - The existing duty cycle solution defined in other WIs can apply to FDD.

Issue 1-1-2: Half-duplex operation

- Proposal: Introduce half-duplex operation for NR FDD bands under HPUE scenario to enable a quick adoption for PC2 in NR FDD bands.

  - Option 1: Not to introduce half-duplex operation for FDD PC2, as there are significant performance loss for DL operation.

  - Option 2: Introduce half-duplex operation for FDD PC2.

- Recommended WF

  - TBA

Sub-topic 1-2 Interference

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2: MSD evaluation

- Moderator observation: MSD values provided by companies based on existing assumptions for n1 and n3 is not a limiting factor for FDD HPUE.

- Proposals:

  - Option 1: To evaluate MSD values based on available RF components and existing assumptions, the specific values can be discussed based on available inputs.

  - Option 2: New assumptions (based on new RF component capability), if proposed and agreed in the group, can be used for MSD evaluation.

- Recommended WF

  - TBA

Sub-topic 1-3 System Performance Evaluation

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2: Simulation results

- Proposal 1: There are performance gain observed based on simulation results from agreed assumptions.

- Recommended WF

  - To conclude that there is performance gain observed based on simulation results.

Companies views’ collection for 1st round

Open issues

Sub topic 1-1 SAR Scheme(s)

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 1-1-1: For option 1, how would this be any different from P-MPR? Option 2 is an interesting idea but needs further investigation. Option 3 doesn’t appear to work. Issue 1-1-2: The proposal for half duplex is to enable quick adoption, but we fear it might have the opposite effect. In order to enable half duplex, the networks have to support it and we think that may take more time than solving the UE problems related to full duplex PC2. But we would be interested to hear the perspectives from other companies. |
| Huawei | Issue 1-1-1: Option 3. We think the solution based on UL dutycycle is the same as those for other HPUE WI. Essentially option 1 is UE implementation, which is similar to P-MPR solution. For option 2, we don’t think that the UE capability should have impact on BS behavior. Issue 1-1-2: Option 1. The proposal of half duplexer solution is aiming to solve the SAR issue, however, the performance loss introduced by the solution could too high than the purpose of introducing FDD HPUE. |
| Skyworks | Issue 1-1-1: As discussed in previous meeting duty cycle alone might not be sufficient, a max 100% duty cycle duration is needed for RF front end components to have a known thermal and reliability behavior. The UE shall at least be able to use PMPR when the 100% duty cyle duration is too long. Issue 1-1-2: Question for clarification: is this HD-FDD proposed as an option on top of FD-FDD or the only operation mode for PC2 FDD? From the diagram it seems that FD-FDD is supported too up to 23dBm and then HD-FDD is used only above 23dBm. Is this the right understanding? |
| Xiaomi | Issue 1-1-1: Needs to better understanding how option 3 work in FDD operation. For option 1, does it means only P-MPR solution is considered? Issue 1-1-2: The proposed half duplexer solution is beneficial to avoid desense. If the desense is very severe for specific FDD band, this proposal can be considered. Thus, we are open to discuss this proposal of half duplexer solution. |
| Vivo | Issue 1-1-1: option 2 Issue 1-1-2: it seems difficult to sync between NW and UE about when to use half-duplex. |
| ZTE | Issue 1-1-1: Option 1 is UE implementation, like P-MPR method, and it seems the fallback behoviour (i.e. fallback to PC3) might not feasible. Option 3, The Tx control and duty cycle calculation might be different with HPUE TDD. Issue 1-1-2: Half duplex can solve some issue such as SAR and interfering from Tx to Rx. However, it might need switching time between Tx<->Rx or guard band to protect the UL or DL symbols, like GP in TDD, otherwise the performance loss would be seen. |
| Ericsson: | Issue 1-1-1: this is essentially the same as a proprietary P-MPR method. But the P-MPR method is preferred to a duty-cycle reporting method with a PC3 (?) fallback that may not be necessary, the actual output power not considered. The P-MPR method can be tested: use an RMC with a specific duty cycle e.g. 50%. Then the average output power should be 23 dBm. Issue 1-1-2: HD will be covered by other device types with reduced capability. |
| Apple | Issue 1-1-1: Option 1 and Option 3 does not seem to relate to “how to handle evaluation period for duty cycle solution”. Issue 1-1-2: The half-duplex operation not only mitigates the SAR issue, it also completely avoids the REFSENS impact to DL. Another advantage is that duplexer can be bypassed to reduce the UL insertion loss which is a further boost for UL power. The overall throughput impact can be further analyzed. In half-duplex operation, UL can take the advantage of full RB allocations and achieve the same UL throughput with a shorter UL burst (as compared to full-duplex operation with UL RB restriction and duty cycle). With a shorter UL burst, more time can be allocated to DL. |
| LGE | Issue 1-1-1: Option 1 and Option 3 is feasible for PC2 FDD UE. For Option 3, the Tx control and duty cycle calculation might be different with HPUE TDD. Issue 1-1-2: Option 1. Half-duplexer can be avoid the Rx/Tx interference and satisfy the SAR requirmeents. But, it was not analyzed in SLS evaluation so, if to introduce the half-duplexer mode, then T-put gain significantly degraded. The REFSENS problem can be solved by RB position & size restriction with small UL/DL Gap. |
| OPPO | Issue 1-1-1: How to handle evaluation period for duty cycle solution Option 1. In our view this is the most practical outcome of the FDD duty cycle discussion since there is no practical way to align NW/UE on the evaluation period, but of course this is common issue as other HPUE not just FDD HPUE. Issue 1-1-2: Half-duplex operation Option 1. |
| China Unicom | Issue 1-1-1: Option 3. The TDM patterns for FDD band used for FDD+TDD EN-DC and ongoing UL CA PC2 can be aligned. We can further investigate the Tx control and duty cycle calculation issues for FDD PC2. Issue 1-1-2: HD operation might be beneficial to solve DESENS issues, but we have concerns on performance loss for NR FDD. |

Sub topic 1-2 Interference

| Company | Comments |
| --- | --- |
| Qualcomm | Option 2 |
| Huawei | Option 1. Based on the analysis in these two meetings, we already see that the MSD for n1 and n3 due to HPUE is not very large. The requirements can be determined in WI stage. |
| Skyworks | We do not agree with the analysis for n3 and thus the moderator and huawei observation that the MSD is not very different: In R4-2109998 there is only 3dB increase in PA noise between n1 and n3, this is totally ignoring the fact that at 40-45MHz CBW, the DL is subject to IMD5 of wanted and image (duplex distance is 95MHz and BW up to 45MHz). In fact the interference level can be as high as -30dBm/MHz based on MPR. With 50dB isolation, the worst case MSD is >10dB. As ZTE mentions in their paper the n3 REFSENS degradation is large while for n1 with the large duplex distance it is negligible. Since this PC2 FDD discussion is guided by the use of wider CBW to be supported vs LTE. It is very important that the worst case MSD is understood. At least as a first step the normal assumptions shall be used with the allocation in the worst position, carrier, image, CIM3,CIM5 TRX impairments as per current 3GPP assumptions. It should be noticed that the TRX impairments actually dominates this issue as the PA only generates the unwanted IMDs in the DL because of the presence and level of the TRX impairments. We are not against looking at options that would make PC2 FDD REFSENS similar to PC3 but we should start from the baseline PC3 test points. These options should include improvements of TRX impairments |
| Xiaomi | It may depend on case by case |
| ZTE | There are two different RF architecture, one 26dBm PA and 2*23dBm PA. So we think for 2*23dBm RF architecture, the existing assumptions would be reused (Option 1), while for one 26dBm PA, New assumptions would be needed (Option 2.). |
| Apple | We share the same views with Skyworks. For FDD bands with narrow duplex distance, substantial UL RB restriction may be needed to avoid REFSENS impact. For n3 under PC3, even with UL RB restriction down to 50 RBs, we can still see significant REFSENS degradation for 45MHz DL BW. |
| LGE | Option 2 is more generic to determine the simulation assumptions and change the RX position and RB size restriction, it shall be considered the enhanced RF component feasibility such as duplexer and PA linearity enhancement. |

Sub topic 1-3 System Performance Evaluation

| Company | Comments |
| --- | --- |
| Huawei | Based on the simulations in these two meetings, we already see the performance gain by FDD HPUE. Thus we think that the conclusion on system performance evaluation can be made in this meeting. |
| vivo | The interference of high power is not analyzed in the FTP traffic mode, because of the low resource usage, the uplink transmission of multi-UE happens at the same time are very rare. In the other traffic mode, such full buffer traffic mode, the defect of the high power can be observed. |
| ZTE | Agree with the recommended WF. |
| OPPO | Share similar view as vivo, the interference should be modeled in a more practical way to see how much interference would happen in high load, middle load, low load situations. And the vivo simulation shows that the performance gain would be small or even degraded. |
| China Unicom | Agree with the WF. From the simulation results submitted by companies in the current and previous meeting based on agreed assumptions, performance gain in both cell average and cell edge can be observed. Regarding the simulation based on full buffer model, the full buffer traffic is very unlikely to be seen in the commercial network, especially for UL operation, so practically full buffer is not a good representation for UL data traffic. And there is no need to include it in the study. |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2109998 (TP on UE implementation for PC2 FDD band) - LGE | Skyworks: we do no agree on the MSD analysis for n3 which does not account for the critical IMD cases for large CBW which is the main argument for PC2 FDD. LGE: We think that RAN4 just evaluate the MSD degradation impact in SI phase. If RAN4 agree to necessity the MSD analysis for wider CBW in SI phase, then we can provide expected MSD degradation in the wider CBW in next meeting. So, TP can be approved in this meeting and updated with wider CBW in next RAN4 meeting. |
|  | ZTE: It seems only 10MHz CBW is evaluated. However, for other CBW, espcially for big CBW, CIM issues needs to be considered. such as for 40/50MHz for band n3. LGE: We think that RAN4 just evaluate the MSD degradation impact in SI phase. If RAN4 agree to necessity the MSD analysis for wider CBW in SI phase, then we can provide expected MSD degradation in the wider CBW in next meeting. So, TP can be approved in this meeting and updated with wider CBW in next RAN4 meeting. |
| R4-2110798 (TP to TR38.861: Simulation results for FDD HPUE) - Qualcomm | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 SAR Scheme(s) | Tentative agreements: None Candidate options: Recommendations for 2nd round: Further discuss viable SAR solution(s) other than P-MPR. |
| Sub-topic #2 Interference | Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss remaining issues for MSD method. |
| Sub-topic #3 System performance evaluation | Tentative agreements: - There is performance gain observed based on simulation results from agreed assumptions. - Large traffic models (e.g. full-buffer) in UL is hardly observed in the practical network, there is no need to include it in the study. Candidate options: Recommendations for 2nd round: Agree on the tentative agreements, and make conclusion on performance evaluation. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

WF on PC2 for NR FDD band

Companies could provide comments for draft WF on PC2 for NR FDD band shared in [99e][153] FS_NR_PC2_UE_FDD Round 2 draft folder

| Company | Comments |
| --- | --- |
| Apple | Thanks China Unicom for preparing the WF on PC2 for NR FDD band. In this meeting, we brought in the concept of half-duplex operation for PC2 FDD band which can not only mitigate the SAR issue, but also provide the benefit of avoiding further REFSENS impact under HPUE yet without the need for any UL resource allocation restriction. Additionally, bypassing the duplexer can help reduce 1 to 2dB UL insertion loss and avoid the potential duplexer performance degradation due to short-term and long-term thermal stress under HPUE operation. Though there were some concerns during the first round discussions on the DL throughput impact under half-duplex operation, since we are still in the study phase, we think any potential viable solution should be given an opportunity to be studied, especially when we do not yet have a clear picture on how the SAR issue can be properly mitigated. On the other hand, using Band n3 as an example, for 40MHz and above channel BW, we have seen substantial REFSENS degradation even under PC3 with UL resource restricted to 50 RBs which only accounts 1/4 or less of the full channel BW. By taking into account 50% UL duty cycle for PC2, with half-duplex operation and full UL allocation, UL only needs to occupy 1/8 of the total time which leaves 7/8 of total time to DL and that is not far from full-duplex operation for DL. Furthermore, half-duplex operation is only needed when the UL power exceeds 23 dBm and for most of the time the UE may still operate under normal full-duplex mode. Therefore, before further detailed analysis, it would be too early to conclude about the throughput impact. Based on the above assessment, we suggest to include the half-duplex operation into the WF for consideration. |
| Huawei | Half duplex solution would have severe performance loss for the system especially n1 and n3 are main FDD NR operating bands. The solution also could have impact to RAN1, which is out of the scope of the RAN4 SI. Our preference is not to include this solution in the WF. |
| Vivo | SAR scheme: Only UE implementation-based method may have issues. Most of time, UE is at the cell edge when reaching MOP. If there is no any indication and expectation of UE power fallback timing, the link may break. Half duplex operation: Half duplex may have many issues, but is a possible solution. In the SI phase, we support to further analysis. System performance evaluation: We agree that full buffer in UL is not a typical traffic model and the performance gain can be observed in low traffic model. But from the system evaluation point of view, all aspects of FDD HP UE should be analysed. After comparing pros and cons, we get a concrete conclusion. |
| China Unicom | We appreciate the effort from Apple to bring a new potential solution to accommodate SAR issues, and we think further analyses could be brought in the next meeting. However, in this meeting companies have concerns that half-duplex operation will introduce DL performance degradation, and there might be impacts to other WGs. So we suggest not to include the half-duplex in the WF in this meeting. For system performance evaluation, we can observe performance gain from both system level and Monte Carlo based simulations. In commercial network, if the resource utilization ratio exceeds certain threshold, operator will extend the network capacity (e.g. deploy new carriers, etc.), so generally the utilization rate will be maintained at a low level. The conclusion part was revised to not precluding further simulations based on other traffic models. |
| Qualcomm | We have a concern on the WF. Under the slide titled “SAR Scheme”, the agreement is UE implementation based to handle duty cycle. Since there are no other bullets listed for SAR Scheme, this might be understood that duty cycle is the only approach, but it has already been agreed that Tx power backoff (P-MPR) is also allowed. Also, if the Tx power or duty cycle is purely UE implementation, then what is the need to introduce duty cycle? We see the value of duty cycle when there is reporting (but that is FFS) so that the scheduler might take that into consideration. Without this, we don’t see much value in designating a UE implementation method that has no bearing that we can see to the specifications. |

TP comments collection

| CR/TP number | Comments collection |
| --- | --- |
| Revision of R4-2109998 (TP on UE implementation for PC2 FDD band) - LGE | LGE : the sensitivity degradation study results is for general sensitivity degradation by 3dB high max output power in FDD band in example NR band n1 and n3. The CIMD impact with wider CBW in n3 by ZTE and SKW is band specific impact with small frequency gap between UL and DL. So the CIMD with wider CBW can be further discussed in next rAN4 meeting. And can captured the MSD issues in TR. So the analysis results can be approved in this meeting. |
|  | China Unicom: Thanks for the revised TP. Some of the parts may still need revisions based on the revised version. For instance, the title of section 6.1.1, 6.1.2, 6.1.3, the term “Dorminent factor” is used, is it intended to mean “Dominant factor”? In section 7.1, the tables are in format of figures. And some grammar issues like “filter vendor need to enhanced the Tx/Rx isolation level to …” We think these issues need to be clarified or fixed before implementing to the TR. |
|  | Skyworks: the Revised TP at least points at the real aspect of PC2 in band n3: PC2 FDD is studied to allow the same cell range for BW>20MHz vs LTE in n3 it goes up to 50MHz. for large BW the interference to the DL channel is no longer related to the PA+TRX noise analyzed in the TP but due to low order IMDs of the wanted and the Image falling into the DL channel resulting in larger MSD. Current PC3 values for REFSENS are For PC3 and will be worse for PC2: REFSENS for 40MHz is -81.3 dBm REFSENS for 45MHz is -82.3dBm REFSENS for 50MHz is still discussed ~-80dBm This is clearly up to >10dB desense compared to PC2 TP that says WC is -93.7dBm For next meeting we need to properly evaluate those cases. |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| WF on PC2 for NR FDD band | China Unicom |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2110163 | Half duplex operation for PC2 FDD bands | Apple | Noted |  |
| R4-2110197 | Discussion on HP UE for FDD bands | Xiaomi | Noted |  |
| R4-2110829 | R17 FDD HPUE | OPPO | Noted |  |
| R4-2109700 | Discussion on SAR scheme of FDD HPUE | vivo | Noted |  |
| R4-2110433 | Discussion on interference for HPUE FDD band | ZTE | Noted |  |
| R4-2111446 | Further system level simulation for FDD HPUE | Huawei, HiSilicon | Noted |  |
| R4-2109699 | System performance evaluation on FDD HPUE | vivo | Noted |  |
| R4-2109763 | System performance evaluation on FDD HPUE | ZTE | Noted |  |
| R4-2109998 | TP on Sensitivity degradation in NR n3 for PC2 UE in FDD band | LGE | Revised | Depending on 2nd round discussion |
| R4-2110798 | TP to TR38.861: Simulaiton results for FDD HPUE | Qualcomm | Agreeable |  |
| R4-2108866 | TR 38.861 v0.1.0 FS_NR_PC2_UE_FDD | China Unicom | Post-meeting approval | To capture agreed TPs in this meeting |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |
| R4-2107742 | WF on PC2 for NR FDD band | China Unicom | Agreeable | Further check recommendation in GTW session |
| R4-2107896 | TP on Sensitivity degradation in NR n3 for PC2 UE in FDD band | LGE | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents