3GPP TSG-RAN WG4 Meeting # 99e 												R4-2108441

Electronic Meeting, 19th – 27th May, 2021

Agenda item:			9.16.1, 9.16.2

Source:	Moderator (Samsung)

Title:	Email discussion summary for [99-e][316] NR_eIAB

Document for:	Information

Introduction

This is Rel-17 enhancement on IAB based on Rel-16 WI with latest WID agreed as RP-210758. RAN4 scope on core part is to define necessary RF and RRM according to Rel-17 extension. This thread continues the discussion for Rel-17 eIAB RF impact based on scope WF agreed in May

List of candidate target of email discussion for 1st round and 2nd round

- 1st round:

  - Clarification on work plan if needed

  - collect view on each topics agreed to be discussed further in RAN4

- 2nd round: discuss on tentative WF regarding detail RF impact on topic discussed during 1st round

Topic #1: Work plan

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2110002 | Samsung,Qualcomm | Overall work plan shared in this contribution with coordination on all related working groups for RAN4 information. As pointed out in work plan RAN4 perf part will be included back after related TU and scope clear. |

Open issues summary

Sub-topic 1-1: work plan

Sub-topic description:

The work plan is just for information with the same style and skeleton as shared before last meeting with more detail suggestion on topics to be discussed in May meeting. And it has been shared in RAN4 reflector before submission. There is no comment received so far. Hence it is supposed this one can be noted. But if any further clarification and/or comment especially on RRM perspective in the 1st round would be still allowed and welcome.

Please note that this work plan is not aligned the typical RAN4 work plan style. But it is supposed that 3GPP would be tolerant enough for this level variety.

It is suggested not to continue on discussion for work plan in the 2nd round. Furthermore, if no further comment on workplan style and no comment to be received before meeting, the work plan will not be addressed in thread discussion in future meetings.

Issue 1-1: Work plan on Rel-17 eIAB

- Proposals: work plan just provides the suggested topic to be studied for coming meeting based on existing allocated TU.

- Recommended WF

  - Work plan is just for information which is recommended to be noted.

  - Clarification and comment can be collected during the 1st round discussion.

Companies views’ collection for 1st round

Open issues

Sub topic 1-1 work plan

| Company | Comments |
| --- | --- |
| Ericsson | ok |
| Nokia, Nokia Shanghai Bell | Impact on RF conformance testing with simultaneous DU and MT operations should be added to the WID in next RAN. |
| Samsung | To Nokia: yes this point as agreed in April meeting is planned to updated in June RAN-P meeting |

CRs/TPs comments collection

NA for this meeting

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #1: work plan | Tentative agreements: Workplan can be noted and agreement on update WID will be addressed in RAN-P meeting Recommendations for 2nd round: No discussion needed in 2nd round |

CRs/TPs

NA for this meeting

Discussion on 2nd round (if applicable)

It is suggested not to continue on discussion for work plan in the 2nd round.

Topic #2: Rel-17 eIAB impact on RAN4 RF core

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109754 | ZTE Corporation | Proposal 1: When IAB node in Case-6 timing mode transmits UL with other IAB nodes or Access Ues in the same slot, the impacts of Rx timing misalignment should be studied. To support case-7 timing with shared RF chain between MT and DU, it is proposed: Proposal 2: RAN 4 send an LS to consult with RAN1 on the possible Rx power difference between IAB-DU and IAB-MT. |
| R4-2109834 | Nokia, Nokia Shanghai Bell | FDM operation: Observation 1: FDM of IAB-MT and IAB-DU transmissions may impact RF performance, especially if IAB-MT transmission is power controlled. Proposal 1: Further investigate specification impact of FDM of IAB-MT and IAB-DU transmissions, taking into account at least if emissions requirements can be met and impact to signal quality Observation 2: FDM of IAB-MT and IAB-DU transmissions may impact IAB-MT signal quality, if relatively large timing advance is applied to IAB-MT. It is not clear if there is specification impact. SDM operation: Proposal 2: No specification impact from SDM-operation when two panels are simultaneously receiving or simultaneously transmitting. |
| R4-2110003 | Samsung | Observation and proposal for scenario 1 and 2: Observation 1: For scenario 1 and scenario 2, IAB-MT and IAB-DU can be recognized to be same as TDD synchronization operation among gNBs belong to same site. Observation 2: Unsynchronized TDD operation discussed in legacy release has no impact on BS RF core requirement. Proposal 1: there is no RF core requirement impact due to enable IAB simultaneous operation in scenario 1 and scenario 2. Observation and proposal for scenario 3 and 4: Observation 3: for IAB-type 1-H Intra-system transmitter intermodulation has already been defined. Observation 4: for IAB type 1-O only colocation IM has been defined which is considered as worst case with higher interference signal level. Observation 5: following BS agreement, no IAB type 2-O IM defined due to high isolation. Proposal 2: if no evidence shown isolation decreased significantly between interference and aimed transmitter, no new TX intermodulation requirement is needed to support scenario 3 and scenario 4. Proposal 3: For IAB which is able for simultaneous operation in scenario 3 and scenario 4, the receiver RF requirements shall be met with the transmitter unite(s) ON. |
| R4-2110004 | Samsung | Observation 1: more RAN1 input is needed to proceed RAN4 discussion on requirement impact due to switching between Case 1, Case 6 and Case 7 timing is supporting. Observation 2: To support timing case #1, only RRM requirement was implicated in rel-16 study without any impact on RF requirement. Observation 3: To support timing case#6, no RF requirement impact is anticipated. Observation 4: To support timing case#7, no RF requirement impact is anticipated. |
| R4-2110005 | Samsung | Observation: existing IAB-MT capability ignalling design is also forward compatible to support existing Rel-15 band comb related feature as optional ones. Proposal: no LS to RAN2 needed at least at current stage regarding DC feature related capability. |
| R4-2111184 | Ericsson | Observation 1: The RF requirements for simultaneous transmission of both MT and DU within an IAB node are specified in Rel-16 TS 38.174. Proposal 1: No RF specification impact in Rel-17 for simultaneous Tx on MT and co-located DU. Observation 2: MT TX/DU RX or MT RX/DU Tx simultaneous operation requires enough isolation between MT and DU transceiver which could possibly be achieved through site deployment. High level of needed isolation could be possibly achieved by increasing the antenna isolation by large physical separation between antennas. Observation #3: Simultaneous MT Tx/DU RX or MT Rx/DU TX even when enough isolation within one operator operating band is achieved may result in interference in adjacent operator´s network. Proposal 2: No RF specification impact in Rel-17 for MT RX/ DU Tx or MT TX/DU RX simultaneous operation. Observation 4: MT RX/DU RX simultaneous receiving does not necessitate the additional RF requirement. Observation 5: MT RX/DU RX simultaneous receiving imply child IAB-MT transmitting only in downlink time slot as the parent IAB-DU can only transmit in down time slot. Proposal 3: There is no RF specification impact for MT RX/DU RX receiving. |
| R4-2111185 | Ericsson | Observation#1: Parent IAB-DU does not need to be aware about the TAE between its DL timing and the DL timing of child IAB-DU for case#6 timing operation. Observation-2: For the case of child IAB-MT synchronizing with co-located child IAB-DU, Parent IAB-DU needs to be aware about the TAE between its DL timing and the DL timing of child IAB-DU for case#6 timing operation. So the correct setting of the receiving timing on parent IAB-DU will be possible Observation-3: Parent IAB-DU set its receiving timing differently depending on the child IAB-MT synchronization implementation. Observation-4: The TAE between DL TX of child IAB-DU and parent IAB-DU could be signalled to parent IAB-DU. Alternatively, the parent IAB-DU receiving timing needs to tolerate the maximum TAE. Proposal-1: RAN4 discuss two aspects for Case#6 timing. - TAE impact: pre-known by parent IAB-node or be tolerated on parent IAB node for implementation (b) - Signalling aspect: whether the synchronization implementation option (a) or (b) should be signalled to parent IAB. Proposal-2: Focus Case 7 timing discussion for the shared receiver for IAB-MT and IAB-DU case. |

Open issues summary

As common understanding in RAN4, for Rel-17 Eiab discussion in RAN4 will highly depend on RAN1 progress which is reported as 30% completion level in Mar RAN-R SR(RP-2100443). Then it may be premature to make decision on all aspects in RAN4 based on current status.  Consequently, RAN4 discussion in this meeting is based on WF in R4-2106115 agreed in last meeting for below topic:

- Simultaneous operation of IAB-node’s child and parent links

- Timing enhancement impact on RF

- DC further clarification

Sub-topic 2-1: Simultaneous operation of IAB-node’s child and parent links

Sub-topic description:  WF agreed in R4-2106115 on this topic as below

Agreements: Declaration is agreed to be considered for IAB-node to indicate whether to support simultaneous operation and/or new requirement to be introduced in Rel-17. - FFS on details Way forward: There is no agreement on whether the RF core requirement will be impact due to introduction of simultaneous operation between IAB-DU and IAB-MT. While it is identified that RF conformance testing need to be reviewed anyway. - Further input on necessity on RF impact is encouraged in next meeting to justify the necessity. - Include the conformance testing spec(TS38.716-1/2) in updated WID and address perf TU on RF session

There are three contributions provide discussion on this topic. The view on simultaneous TX/RX by SDM is aligned well. For this aspect it’s supposed that the tentative agreement can be confirmed in 1st round with discussion on minor detail.

In addition, it is proposed to study further the RF requirement impact due to simultaneous TX/RX by FDM, for which more clarification is suggested by moderator to facilitate further discussion.

Open issues and candidate options before e-meeting:

Issue 2-1-1: for simultaneous operation of IAB-node’s child and parent links by SDM

- Proposal:

  - Proposal 1: no RF specification impact identified on simultaneous MT TX/DU TX

  - Proposal 2: no RF specification impact identified on simultaneous MT RX/DU RX

  - Proposal 3: for both simultaneous MT TX/DU RX and simultaneous MT RX/DU TX

    - Option 1: No any RF specification impact as isolation can be achieved by site deployment.

    - Option 2: the receiver RF requirements shall be met with the transmitter unite(s) ON which to be addressed simply in general sub-clause without impact on individual RF requirement.

- Recommended WF

  - View on each proposal or alternative option are encouraged to be shared in the 1st round discussion

Issue 2-1-2: for simultaneous operation of IAB-node’s child and parent links by FDM

- Proposal:  according to suggestion to discuss this aspect further, it is suggested to clarify below points in 1st round

  - Question 1: whether IAB-MT and IAB-DU simultaneous operation is expected to be achieved purely by FDM without SDM(e.g., by single beam)?

  - Question 2: co-location deployment between different BS classes is not assumed in legacy BS specification, whether this is applicable for IAB node?

  - Question 3: for simultaneous operation of IAB-MT and IAB-DU by FDM, whether the isolation by site deployment can be applied?

- Recommended WF

  - Views on each question are encouraged to be shared in the 1st round discussion

Sub-topic 2-2: Timing enhancement impact on RF

Sub-topic description: case 6 and case 7 timing mode are agreed to be supported on top of case 1 which is agreed for Rel16. Furthermore, RAN1 is working on the detail regarding switching among case 1, case 6 and case7.  It’s obvious that further RAN4 study needed regarding the additional timing mode and switching between cases, which is not easy without full RAN1 conclusion. However, the study on case 6 and case 7 seems doable with respect to purely RF requirement of own IAB node.

There are three contribution provide analysis on this topic. And two of them provide more analysis on case#6 impact on donor node as when Case#6 is applied, donor cannot follow the existing TA mechanism any more. Some alternatives solutions have already discussed since SI phase for Rel-16 IAB as captured in TR38.874 in RAN1 as below. For this aspect there is no conclusion in RAN1 as well. It’s suggested to avoid duplicated discussion parallel in RAN4 on RAN1-led feature.

The use of Case 6, if supported, at the IAB-node should be under control of the parent or network. To enable alignment of DL transmissions among IAB-nodes, the following examples of solutions have been identified: - Alt. 1: The IAB-node may need to carry out parallel (always time multiplexed) case #1 and case #6 uplink transmissions; - Alt 2: Signalling between the parent and IAB-node of the time difference of the DL Tx and UL Rx timing at the parent node in order to correct potential misalignment of the DL Tx timing at the child node: - The child IAB-node compares the corresponding difference of its own DL Tx timing and BH Rx timing; if the signalled difference of the parent node is larger than measured at the child node, the child node advances its TX timing, if smaller the TX timing is delayed. - Note: Alt 1 & Alt 2 may require maintenance of separate Rx timings at the parent node for Case 6 UL transmissions from different child nodes. Case #7 is compatible with Rel-15 Ues by introducing an “effective” negative TA, and TDM between child IAB-nodes/Rel-16 Ues which support the new TA values and child IAB-nodes/Ues which do not support the new TA values. To enable alignment between DL and UL reception within the IAB-node the following examples of solutions have been identified: - Alt 1: Introduce negative initial time alignment (TA) for IAB-nodes, to be applied to child nodes of the IAB-node applying case #7 timing; - Alt 2: Apply a positive TA that enables symbol alignment, but not slot alignment, between the DL reception and the UL reception at the IAB-node; - Alt. 3: Signalling of a relative offset w.r.t the most recent TA value, to be applied to child nodes of the IAB-node applying case #7 timing to achieve an effective negative TA. In addition to OTA synchronization, other techniques such as GNSS and PTP, can be used to achieve synchronization across IAB-nodes.

Open issues and candidate options before e-meeting:

Issue 2-2-1: timing case#6

- Proposal:

  - Proposal 1: no RF requirement impact identified for IAB which supports timing case#6 except TAE

  - Proposal 2: company's view can be shared on below options:

    - Option 1:  FFS on whether parent IAB node needs be aware of TAE between its DL timing and the DL timing of child IAB-DU for case#6 timing operation

FFS on whether the synchronization implementation should be signalled to parent IAB

    - Option 2: Donor BS/parent IAB node implication due to its child IAB operating as timing case#6 is postponed to be discussed based on RAN1 conclusion when it's available.

    - Option 3: TBA

- Recommended WF

  - Views on each proposal are encouraged to be shared in the 1st round discussion

Issue 2-2-2: timing case#7

- Proposal:

  - Proposal 1: no RF requirement impact identified at least for IAB node which supports timing case#7 by separated RF chains between its own MT and DU.

  - Proposal 2: for IAB node supports  timing case#7 with shared RF chain solution, to clarify in 1st round:

    - The necessity on LS to RAN1on received power difference suggested by R4-2109754

    - Whether there is RF requirement impact or not for IAB node in this case

- Recommended WF

  - Views on each proposal are encouraged to be shared in the 1st round discussion

Sub-topic 2-3: DC scenario clarification

Sub-topic description: WF agreed in R4-2106115 on this topic as below

Agreement: All companies provide feedback on this topic share the same understanding as: Existing rel-16 IAB RF specification is compatible for DC scenario. - This is not precluded further discussion on DC if new feature/scenario agreed in RAN1/2/3. Way forward: Further review needed in next meeting regarding: - DC scenario impact on IAB RRM requirement to be discussed in RRM session - Capability signaling for DC feature to be reviewed in next RAN4 meeting to see whether LS needed

Open issues and candidate options before e-meeting:

Issue 2-3: DC scenario clarification

- Proposal:  LS to RAN2 is not needed at least at current stage regarding DC feature related capability.

- Recommended WF: confirm above understanding

Companies views’ collection for 1st round

Open issues

Sub topic 2-1: Simultaneous operation of IAB-node’s child and parent links

Issue 2-1-1: for simultaneous operation of IAB-node’s child and parent links by SDM

| Company | Comments |
| --- | --- |
| Ericsson | P1 is ok, P2is ok , P3-option 1 is ok to us. |
| Nokia, Nokia Shanghai Bell | Proposal 1 and Proposal 2 are ok. For proposal 3 we would be fine with option 1. The isolation would be site dependent and it is challenging to define associated RAN4 requirement. For observation 3 in R4-2111184, how to ensure interference in adjacent operator´s network is acceptable by testing simultaneous MT Tx/DU RX or MT Rx/DU TX operation? |
| Samsung | Fine with proposal 1 and 2. Regarding proposal 3, our preference is option 2 which is also not has any impact on RF requirement. |

Issue 2-1-2: for simultaneous operation of IAB-node’s child and parent links by FDM

| Company | Comments |
| --- | --- |
| Ericson | Q1: it is up to configuration , but should not be a single beam as IAB-MT target to fixed node, while IAB-DU target to mobility UE. Or Q1 also means the conductive IAB node? Q2: what is the question related to the FDM operation? Different IAB class co-located or? Q3: FDM should be the same with SDM, no RF impact is expected, site isolation should be enforced to not desensitizing receiver. |
| Nokia, Nokia Shanghai Bell | Question 1: whether IAB-MT and IAB-DU simultaneous operation is expected to be achieved purely by FDM without SDM(e.g., by single beam)? Using single beam is not precluded so requirements should be based on this most stringent scenario. Question 2: co-location deployment between different BS classes is not assumed in legacy BS specification, whether this is applicable for IAB node? Our understanding is that same approach should be used as e.g. BS-UE or IAB-DU distance stays the same for same site, I.e. it would be logical that only same class can be co-located. Further considerations are needed if/how this applies to IAB-MT. Question 3: for simultaneous operation of IAB-MT and IAB-DU by FDM, whether the isolation by site deployment can be applied? Is this meant for isolation between different operators? This will have the same issues as observed in the rel-16 CLI study. For isolation between the same operator while it is possible to have isolation in the site, IAB-MT and IAB-DU isolation cannot be assumed to apply always as same antenna array may be shared. Overall, we think that at least the impact of power imbalance and timing difference should be further investigated for FDM-operation. |
| Samsung | Question 1: In RAN4 spec we only consider 1-H, 1-O and 2-O for IAB node, and it’s highly possible that the beam direction would be different between IAB-MT and IAB-DU. If it is confirmed maybe no need to differentiate the FDM and SDM approach for simultaneous operation between IAB-MT and IAB-DU. Question 2: We believe logically the IAB node should be collocated between IAB-MT and IAB-DU with kind of similar class(even though the MR is not defined for IAB-MT). Even though the IAB power is declaration based for same class it’s supposed that the TX power difference should be limited. This should be taken into account if we study the power difference scenario in FDM based operation further. Question 3: It is assumed that the isolation can be improved by site deployment. However, if that is the same panel that may be different assumption which is needed further discussion. |

Sub topic 2-2:  Timing enhancement impact on RF

Issue 2-2-1: timing case#6

| Company | Comments |
| --- | --- |
| Ericsson | P1 is ok, timing aspects only impact timing related requirent, so TAE is potential impacted RF requirement. P2 option 1: as the case 6 timing has a wish to time alignment between IAB-MT and IAB-DU, the TAE requirement need to further discussed. We observe that such TAE may not be needed if it is not shared architecture as the IAB-MT uplink timing is totally controlled by its parent node, so such TAE requirement may only apply for certain architecture or specifically certain clock solution. We are also ok to wait more progress with RAN1. Signaling aspect can be postponed until more progress on the RAN4 requirement discussion. |
| Nokia, Nokia Shanghai Bell | The described operation in R4-2111185 seems to differ somewhat from the assumptions in RAN1. In Case#6 (like in other timing options) IAB-DU can start transmitting first when the timing synch has been achieved (with given accuracy requirement) - in the description it looked that IAB-DU would TX having a timing error (TAE). Timing advance control is not the same as in Case#1&7. RAN1 is still discussing the Case#6 timing and slightly different views on the operation still exist --> RAN4 should also wait for RAN1 completion before making final agreements. In R4-2111185, TAE information only needs for implementation when IAB-MT is synchronized together with its IAB-DU. Regarding proposal 1, naturally the RF impacts of power imbalance are not removed by supporting timing case 6. For proposal 2, we agree but it is not limited to donor but could be any parent-node. |
| ZTE | UL Rx timing at parent node should be considered when child IAB operates as timing case#6. RAN1 do not reach a common understanding for the TAE between the DL Tx and UL Tx when an IAB operates as timing case#6, RAN4 can discuss the TAE range between DL Tx and UL Tx of an IAB when it operates as timing case#6 and further discuss the impact for UL Rx timing for parent node. |
| Samsung | Regarding proposal 1 our understanding it that case 6 is just simultaneous TX for MT and DU hence the RF impact should be the same as discussed in subtopic 2-1. For TAE as indicated in our paper, if it is for time alignment error, traditionally this is only define within one BS node (or IAB-DU). It is not expected to define TAE between different nodes, which more likes synchronization requirement and would be out of RF requirement scope. Proposal 2, our preference is option 2 , RAN4 should wait for RAN1 progress for further study based on whole picture on how the system operate. |

Issue 2-2-2: timing case#7

| Company | Comments |
| --- | --- |
| Ericsson | Issue 2-2-2: we think there may be some similarity between case 6 and 7 timing aspects. So RF requirement discussion could cover both case6 /7 timing. We are not sure the receiver power difference question should be Ran1 answer, it quite depends on receiver blocking /ACS. |
| Nokia, Nokia Shanghai Bell | RAN1 has already agreed alt.2 to use symbol alignment without need for negative TA. There may be no specification changes needed due to Case#7 timing: Normal TA loop is used for UL TX with T_delta can be used for synchronisation. IAB-DU UL RX is aligned with TA control of the IAB-node itself. We agree proposal 1, but we think the power imbalance would be for RAN4 to evaluate and LS to RAN1 is not needed. |
| ZTE | When the Rx power imbalance from parent link and child link is quite large, the IAB node may have problem with simultaneous Rx since it is hard to select the AGC level to achieve simultaneous Rx from both link. We do not tend to spend time doing simulation in RAN4, if all agree to do it in RAN4 we are OK. |
| Samsung | Agree with proposal 1 For the power imbalance, first of all it should be RAN4 scope rather than RAN1. Hence no LS to RAN1 needed. In addition, it is not precluded further study if preferred by other companies would like to have more assessment. But it is not quite sure whether power imbalance should be defined in spec due to specific implementation case or we just simply follow the RF architecture agnostic way. If for certain deployment scenario the power imbalance from parent and child is out of capability of certain IAB node, the infra vendor may consider other type of IAB type rather than putting limitation on scenario. |

Sub topic 2-3: DC scenario clarification

| Company | Comments |
| --- | --- |
| Ericsson | Issue 2-3, we prefer to discuss signaling aspect not in this meeting. |
| Samsung | Agree with the recommendation |

CRs/TPs comments collection

NA for this meeting

Summary for 1st round

Open issues

According to 1st round discussion it seems that the main remaining issue for this meeting is that the feasibility or limitation on shared RF chain/antenna array IAB to supporting simultaneous operation between IAB-MT and IAB-DU(timing case#6 and timing case#7 are simultaneous TX and simultaneous RX if considered from RF perspective). In below summary table the discussion status is still provided for each issue to align with round1 discussion order for clarity, which may have kind of overlapping and/or redundancy.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1: Issue 2-1-1: for simultaneous operation of IAB-node’s child and parent links by SDM | Tentative agreements: according to feedback during 1st round below proposals are agreeable: Proposal 1: no RF specification impact identified on simultaneous MT TX/DU TX Proposal 2: no RF specification impact identified on simultaneous MT RX/DU RX Candidate options: according to companies’ feedback 2 companies prefer option while one company prefers option2 Proposal 3: for both simultaneous MT TX/DU RX and simultaneous MT RX/DU TX - Option 1: No any RF specification impact as isolation can be achieved by site deployment. - Option 2: the receiver RF requirements shall be met with the transmitter unite(s) ON which to be addressed simply in general sub-clause without impact on individual RF requirement. Recommendations for 2nd round: Capture agreeable proposals in WF Check the proposal 3 further in 2nd round |
| Sub-topic#2-1 Issue 2-1-2: for simultaneous operation of IAB-node’s child and parent links by FDM | Tentative agreements: Companies shared views on several baseline understanding with respect the clarification questions in 1st round. And according to feedback it seems that the possible scenario needs further discussion on simultaneous operation by FDM way is that IAM-MT and IAB-DU share same antenna array at different TX power conditions. Even though there is also comment that for FDM solution the enough isolation can be achieved by sit deployment. Furthermore, since it seems common understanding that the co-located node should be the same class at least for BS and IAB-DU with similar power capability. The applicability for IAB-MT is FFS but this issue could be taken into account regarding the power TX power imbalance discussion. In addition, timing difference is pointed in 1st comment for further discussion. Recommendations for 2nd round: Discuss further in 2nd round regarding IAB simultaneous operation by FDM way includes but not limits to: View on scenario of IAB-MT and IAB-DU share the same antenna array to support simultaneous operation by FDM way: - Power imbalance and isolation between MT and DU - Timing difference if any impact |
| Sub topic 2-2: Timing enhancement impact on RF Issue 2-2-1: timing case#6 | Tentative agreements: According to discussion it seems below proposal is aggregable: Proposal 1: no RF requirement impact identified for IAB which supports timing case#6 except TAE Candidate options: Regarding the TAE within IAB: FFS whether TAE between MT UL TX and DU DL TX needs to be defined especially for shared RF chain IAB in 2nd round Regarding implication on donor BS and parent IAB: postpone the discussion for RAN1 input PS: the TX power imbalance issue can be covered in issue 2-1-2. Recommendations for 2nd round: Confirm tentative agreement and discuss further based on candidate options in 2nd round |
| Sub topic 2-2: Timing enhancement impact on RF Issue 2-2-2: timing case#7 | Tentative agreements: According to 1st round discussion below two proposals seems agreeable Proposal 1: no RF requirement impact identified at least for IAB node which supports timing case#7 by separated RF chains between its own MT and DU. Proposal 2: for IAB node supports timing case#7 with shared RF chain solution, regarding RX power imbalance no LS to RAN1 needed. Candidate options: To discuss further whether there is RF impact due to shared RF chain IAB to supporting timing case#7 Recommendations for 2nd round: Confirm tentative agreement and discuss further on candidate option in 2nd round |
| Sub topic 2--3: DC scenario clarification Issue 2-3: DC scenario clarification | Tentative agreements: No LS needed at current stage regarding this issue Recommendations for 2nd round: No discussion and action needed in 2nd round. RAN4 can discuss this issue in later stage. |

CRs/TPs

NA for this meeting

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on simultaneous operating and timing case#6/7 for enhanced IAB | Samsung |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2110002 | Workplan for Rel-17 IAB | Samsung,Qualcomm | Noted |  |
| R4-2109754 | Discussion on IAB timing related issues | ZTE Corporation | Noted |  |
| R4-2109834 | IAB Rel.17 – RF requirements | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2110003 | Discussion on simultanous TX/RX for IAB node | Samsung | Noted |  |
| R4-2110004 | Discussion on timing mode for IAB | Samsung | Noted |  |
| R4-2110005 | Discussion on Dual-connectivity scenario for IAB | Samsung | Noted |  |
| R4-2111184 | RF impact analysis for Simultaneous operation of DU and MT | Ericsson | Noted |  |
| R4-2111185 | IAB MT /DU case 6/7 timing | Ericsson | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents