3GPP TSG-RAN WG4 Meeting # 99-e	                                                                  R4-2108452

Electronic Meeting, May. 19-27, 2021

Agenda item:			9.7.3

Source:	Moderator (CMCC)

Title:	Email discussion summary for [99-e][327] NR_HST_FR1_Demod

Document for:	Information

- Introduction

This email discussion focuses on UE demodulation for Rel-17 NR HST, including agenda 9.7.3.1~9.7.3.3. Two topics are included in total, including PDSCH requirements for CA scenarios and enhanced transmission schemes. The agreed way forward in previous meeting is in R4-2106098.

The targets of email discussion for 1st round and 2nd round are:                    m8sw 09-	---------923

- 1st round: discuss the open issues and strive to minimize the open issues

- 2nd round: according to 1st round discussion, discuss left open issues for 2nd round, and strive to minimize the open issues, and strive to approve the WF.

Topic #1 PDSCH requirements for CA scenarios

Agenda  9.7.3.2

Companies’ contributions summary

| TDoc | Title | Source | Proposals/ Observations |
| --- | --- | --- | --- |
| R4-2109212 | Views on FR1 HST CA PDSCH performance requirements | Intel Corporation | Proposal #1: Consider Option 1 or New Option 4 that propose combinations of CA duplex modes for requirements definition and applicability rules between them. Proposal #2: Reuse applicability rule from DPS single carrier to CA and define new one as: UE can skip single carrier test case if it has passed corresponding CA test case. Proposal #2: Do not define UE capabilities for HST-DPS and HST-SFN CA. Proposal #3: Consider Option 1: If UE supports demodulationEnhancement-r16, only HST-SFN JT requirements shall apply, otherwise HST-DPS requirements shall apply for CA. Proposal #4: UE skip single carrier test case if it explicitly passes corresponding CA test case. Proposal #5: Add clarification on Doppler frequency for the link from the second nearest RRH for HST-DPS channel model. Proposal #6: Do not add path loss and propagation delay for transmitted signals from each RRH for HST DPS channel model. Proposal #7: Define HST CA demodulation requirements in release independent manner only if corresponding RRM requirements will be release independent. Align exact release number with RRM requirements if such approach will be agreed. |
| R4-2109213 | Simulation results for FR1 HST CA | Intel Corporation |  |
| R4-2109356 | Discussion on PDSCH CA Requirements in HST | Apple | Proposal #1: Do not introduce requirements in HST for FDD 15KHz + TDD 15KHz CA and TDD 15 kHz + TDD 30 kHz CA. Use applicability rule on CA duplex mode as CA CQI requirements. Proposal #2: If UE supports demodulationEnhancement-r16, for PDSCH CA requirements only HST-SFN JT requirements shall apply, otherwise HST-DPS requirements shall apply. Proposal #3: Do not introduce additional UE capability for CA scenario. Proposal #4: Extend the visibility for SSB and CSI-RS for HST-DPS to -Ds to Ds. Observation #1: With time varying path delay alone, performance delta is not significant. Observation #2: With time varying path delay and power, the performance delta is significant. Observation #3: We don’t see motivation for TCI state switch at mid-point of RRHs if same signal power is received along all UE position along the track. Observation #4: Considering time varying path power needs further discussion in RAN4. Observation #5: Assuming 0 path delay for all UE position would be impractical and show optimistic results. Proposal # 5: Update HST-DPS channel model to at least model time varying path delay. Proposal #6: Further discuss if time varying path power should be considered for HST-DPS. |
| R4-2109357 | Simulation results for HST CA scenarios | Apple |  |
| R4-2109466 | Views on FR1 HST PDSCH CA Tests | Qualcomm Incorporated | Proposal 1: If UE passes the FR1 HST PDSCH CA tests for HST-SFN scheme, tests with HST-DPS scheme can be skipped. Proposal 2: Do not define FR1 HST PDSCH CA tests for Duple/SCS combinations involving TDD 15kHz. Proposal 3: Reuse CA CQI applicability rule on CA duplex modes for testing: If UE supports both FDD 15 kHz + TDD 30 kHz and FDD 15 kHz + FDD 15 kHz CA duplex modes, FDD 15kHz + FDD 15kHz CA tests can be skipped. |
| R4-2109513 | Discussion on FR1 HST UE demodulation for CA scenario | CMCC | Proposal 1: it is not necessary to introduce new UE capabilities for HST-DPS CA and HST-SFN CA • for HST-DPS CA, no need to introduce UE capability • for HST-SFN CA, the existing UE capability introduced in Rel-16 NR HST can be reused Proposal 2: More discussion is needed on how to define the applicability rule between HST-SFN joint transmission scheme and DPS transmission scheme considering they are different transmission scheme and have different demodulation performance and different receiver algorithm. SCS configuration Proposal 3: for SCS configuration for CA scenario to be considered and applicability rule for SCS configuration, we are OK with following two options: • Option 2: Introduce requirements in HST for FDD 15 kHz + TDD 15 kHz CA and TDD 15 kHz + TDD 30 kHz CA, and the applicability rule between CA scenario with TDD 15 kHz SCS and CA scenario with TDD 30 kHz SCS specified in Rel-16 can be reused • Option 3: Do not introduce requirements in HST for FDD 15 kHz + TDD 15 kHz CA and TDD 15 kHz + TDD 30 kHz CA, and no applicability rule for FDD 15 kHz + FDD 15 kHz CA, TDD 30 kHz + TDD 30 kHz CA and FDD 15 kHz + TDD 30 kHz CA (i.e. not to reuse CA CQI applicability rule to PDSCH CA normal demodulation requirements) Release independent Proposal 4: it is proposed that Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15. |
| R4-2109519 | Simulation results for HST-SFN joint transmission for CA scenario | CMCC |  |
| R4-2109765 | Discussion on PDSCH requirements for CA in FR1 HST | ZTE Corporation | Proposal 1: Existing UE capability can cover CA scenario. |
| R4-2110148 | Views on HST CA tests for FR1 | NTT DOCOMO, INC. | Observation 1: Option 2 (i.e. Define applicability rule that UE has passed DPS CA requirements can skip SFN CA requirements) indicate that HST-SFN JT CA tests will always be skipped. Proposal 1: If RAN4 agree to define applicability rule between HST-SFN CA requirements and DPS CA requirements, we prefer to define Option 1 (i.e. If UE supports demodulationEnhancement-r16, only HST-SFN JT requirements shall apply, otherwise HST-DPS requirements shall apply for CA). |
| R4-2110526 | Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon | Observation 1: Very limited application scenario for normal HST-SFN. Proposal 1: Only define FDD 15kHz + FDD 1 kHz, TDD 30kHz + TDD30 kHz, FDD 15kHz +TDD30kHz requirements − Test #1: FDD 15 kHz + TDD 30 kHz > FDD 15 kHz + FDD 15 kHz − Test #2: TDD 30 kHz + TDD 30 kHz Proposal 2: Define two UE capabilities for HST-DPS CA and HST-SFN CA, UE perform the test only when UE supports it. if UE supports both, test one scheme: − If UE has passed HST-DPS CA tests, HST-SFN CA tests can be skipped − If UE has passed HST-SFN CA tests, HST-DPS CA tests can be skipped Proposal 3: Do not modify the HST-DPS channel model. Proposal 4: Further study whether HST PDSCH CA requirements can be release independent from Rel-15. |
| R4-2110527 | Simulation results on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon |  |
| R4-2110640 | Update of simulation results for CA PDSCH with HST | Ericsson |  |
| R4-2110641 | PDSCH demodulation requirements for CA with HST-SFN scenario | Ericsson | Proposal 1: For PDSCH CA demodulation in HST (both SFN JT and DPS), RAN4 defines the following combinations: FDD SCS=15kHz + FDD SCS=15kHz, TDD SCS=30kHz + TDD SCS=30kHz, and FDD SCS=15kHz + TDD SCS=30kHz. RAN4 don’t define PDSH CA requirements in HST (both SFN JT and DPS) for FDD 15KHz + TDD 15KHz CA and TDD 15 kHz + TDD 30 kHz CA. Proposal 2: No additional UE capability signaling is introduced for PDSCH CA demodulation requirements for both HST-SFN JT and HST-DPS. Proposal 3: Define the following applicability rule for HST-SFN JT and HST-DPS requirements for CA. • If UE is capable of CA (TS 38.101-4 5.1.1.7) and UE is capable of demodulationEnhancement-r16, the HST-SFN JT requirements for CA shall apply. o If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16. • If UE is capable of CA (TS 38.101-4 5.1.1.7) and UE is not capable of demodulationEnhancement-r16, the HST-DPS requirements for CA shall apply. o If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16. Proposal 4: No need to update the HST-DPS channel model. |

Open issues summary

Applicabiliy rule

Issue 1-1-1: SCS configurations and applicability rules for SCS configuration

- Agreements in RAN4#98bis-e meeting:

  - Option 1:  Do not introduce requirements in HST for FDD 15KHz + TDD 15KHz CA and TDD 15 kHz + TDD 30 kHz CA. If UE supports both FDD 15 kHz + TDD 30 kHz and FDD 15 kHz + FDD 15 kHz CA duplex modes, apply requirements only to the first one (i.e. use the same applicability rule on CA duplex mode for HST CA as CA CQI requirements).

  - Option 2: Introduce requirements in HST for FDD 15 kHz + TDD 15 kHz CA and TDD 15 kHz + TDD 30 kHz CA, and the applicability rule between CA scenario with TDD 15 kHz SCS and CA scenario with TDD 30 kHz SCS specified in Rel-16 can be reused

  - Option 3: Do not introduce requirements in HST for FDD 15 kHz + TDD 15 kHz CA and TDD 15 kHz + TDD 30 kHz CA, and no applicability rule for FDD 15 kHz + FDD 15 kHz CA, TDD 30 kHz + TDD 30 kHz CA and FDD 15 kHz + TDD 30 kHz CA (i.e. not to reuse CA CQI applicability rule to PDSCH CA normal demodulation requirements)

- Proposals in RAN4#98bis-e meeting:

|  | Number of CA Duplex modes | Number of required tests if UE supports all duplex modes |
| --- | --- | --- |
| Option 1 (Intel, Apple, Qualcomm, Huawei, Ericsson) | 3 (FDD 15+FDD15, FDD 15+TDD 30, TDD 30+TDD 30) | 2 (FDD 15 + TDD 30; TDD 30 + TDD 30) |
| Option 2 (CMCC) | 5 (FDD 15 + FDD 15; FDD 15 + TDD 15; FDD 15 + TDD 30; TDD 30 + TDD 30, TDD 15+TDD 15) | 4 (FDD 15 + FDD 15; FDD 15 + TDD 15; FDD 15 + TDD 30; TDD 30 + TDD 30) |
| Option 3 (CMCC, Ericsson) | 3 (FDD 15+FDD15, FDD 15+TDD 30, TDD 30+TDD 30) | 3 (FDD 15 + FDD 15; FDD 15 + TDD 30; TDD 30 + TDD 30) |
| Option 4* (Intel) | 5 (FDD 15 + FDD 15; FDD 15 + TDD 15; FDD 15 + TDD 30; TDD 30 + TDD 30, TDD 15+TDD 15) | 2 (FDD 15 + TDD 30; TDD 30 + TDD 30) TDD 15 is tested if FDD 15+TDD 30 is not supported |

- Recommended WF

  - One new option 4 is proposed in this meeting. More discussion is needed considering the total number of test cases and the number of performed test cases. Compromise is highly encouraged.

Issue 1-1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme

- Agreements in RAN4#98bis-e meeting:

  - Option 1: If UE supports demodulationEnhancement-r16, only HST-SFN JT requirements shall apply, otherwise HST-DPS requirements shall apply for CA.

  - Option 2: Define applicability rule that UE has passed DPS CA requirements can skip SFN CA requirements

  - Option 3:  Define two UE capabilities for HST-DPS CA and HST-SFN CA, UE perform the test only when UE supports it. if UE supports both

    - Option 3a: Test both schemes

    - Option 3b: Test one scheme

      - If UE has passed HST-DPS CA tests, HST-SFN CA tests can be skipped

      - If UE has passed HST-SFN CA tests, HST-DPS CA tests can be skipped

- Proposals in RAN4#99-e meeting:

  - Option 1 (Intel, Apple, Qualcomm, DOCOMO): If UE supports demodulationEnhancement-r16, only HST-SFN JT requirements shall apply, otherwise HST-DPS requirements shall apply for CA.

  - Option 2: Define applicability rule that UE has passed DPS CA requirements can skip SFN CA requirements

  - Option 3 (Huawei): Define two UE capabilities for HST-DPS CA and HST-SFN CA, UE perform the test only when UE supports it. if UE supports both, test one scheme:

    - If UE has passed HST-DPS CA tests, HST-SFN CA tests can be skipped

    - If UE has passed HST-SFN CA tests, HST-DPS CA tests can be skipped

- Recommended WF

  - This issue is related to the UE capabilities. Discuss after RAN4 reach agreements on the UE capabilities.

Issue 1-1-3: Applicability rule between single carrier and CA

- Agreements in RAN4#98bis-e meeting:

  - Discuss applicability rule between single carrier and CA later based on the conclusion of applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme.

- Proposals in RAN4#98bis-e meeting:

  - Option 1 (Intel, Ericsson): UE skip single carrier test case if it explicitly passes corresponding CA test case. i.e.

    - If UE is capable of CA (TS 38.101-4 5.1.1.7) and UE is capable of demodulationEnhancement-r16, the HST-SFN JT requirements for CA shall apply.

      - If UE pass the HST-SFN JT requirements for CA, UE can skip HST-SFN JT requirements for single carrier defined in Rel-16.

    - If UE is capable of CA (TS 38.101-4 5.1.1.7) and UE is not capable of demodulationEnhancement-r16, the HST-DPS requirements for CA shall apply.

      - If UE pass the HST-DPS requirements for CA, UE can skip HST-DPS requirements for single carrier defined in Rel-16.

- Recommended WF

  - Option 1

UE capability and network-assisted signalling

Issue 1-2-1: UE capability and network-assisted signalling

- Agreements in RAN4#98bis-e meeting:

- Option 1: Existing UE capability can cover CA scenario

- Option 2: Define UE capability signaling for UE supporting HST-SFN CA and HST-DPS CA

- Proposals in RAN4#99-e meeting:

  - Option 1 (Intel, Apple, CMCC, ZTE, Ericsson): Do not define UE capabilities for HST-DPS and HST-SFN CA.

  - Option 2 (Huawei): Define UE capability signaling for UE supporting HST-SFN CA and HST-DPS CA

- Recommended WF

  - Option 1

Release independent

Issue 1-3-1: Release independent

- Agreements in RAN4#98bis-e meeting:

  - FFS whether HST PDSCH CA requirements can be release independent from Rel-15

- Proposals in RAN4#98bis-e meeting:

  - Option 1 (Intel): Define HST CA demodulation requirements in release independent manner only if corresponding RRM requirements will be release independent. Align exact release number with RRM requirements if such approach will be agreed.

  - Option 2 (CMCC): it is proposed that Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15.

  - Option 3 (Huawei): Further study whether HST PDSCH CA requirements can be release independent from Rel-15.

- Recommended WF

  - Discuss after RAN4 reach agreements on the UE capabilities.

Channel Model

Issue 1-4-1: Channel model for HST-DPS

- Agreements in RAN4#98bis-e meeting:

- FFS whether to update the HST-DPS channel model

- FFS on adding path loss and propagation delay for transmitted signal from each RRH.

- FFS on clarification of propagation conditions for SSB, TRS(second TCI state) transmitted from the second nearest RRH

  - Option 1: Add clarification on another Doppler frequency for this link comparing to the signal from the nearest RRH.

  - Option 2: Add clarification on another Doppler frequency, propagation delay and Rx power comparing to the signal from the nearest RRH.

- Proposals in RAN4#99-e meeting:

  - Option 1 (Intel): Add clarification on Doppler frequency for the link from the second nearest RRH for HST-DPS channel model.

  - Option 2 (Apple):

    - Extend the visibility for SSB and CSI-RS for HST-DPS to -Ds to Ds.

    - Update HST-DPS channel model to at least model time varying path delay.

    - Further discuss if time varying path power should be considered for HST-DPS.

  - Option 3 (Huawei, Ericsson): Do not modify the HST-DPS channel model.

- Recommended WF

  - More discussion is needed.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-1-1: SCS configurations and applicability rules for SCS configuration We prefer Option 1, since this method is used in CA CQI and there is no test coverage issue found. Issue 1-1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme & Issue 1-2-1: UE capability and network-assisted signalling To achieve better performance under HST-SFN, DL Doppler pre-compensation is usually applied for NR Rel-17 UE. Also DPS can achieve better performance comparing to SFN, so we prefer Option 2. UE needs to track 2 active TCI states simultaneously for DPS 1b while SFN need to track multi taps, both of them have complete different processing algorithms and higher complexity under HST CA scenario, UE capability is needed. Also, the main purpose of definition the applicability rule is to minimize the testing burden, with the UE capability definition and assumption of test applicability between CA and single carrier, the total number of tests will be limited to 2 as listed below, this will not increase the test burden. Case; UE capability; DPS 1a CA case; DPS 1b CA case; SFN CA case; Number of tests. 1; None; √; DPS 1a CA + DPS 1b CC + SFN CC. 2; DPS 1b CA; √; DPS 1b CA + SFN CC. 3; SFN CA; √; DPS 1b CC + SFN CA. 4a; DPS 1b CA and SFN CA; Both; DPS 1b CA + SFN CA. 4b; DPS 1b CA and SFN CA; Either; DPS 1b CA + SFN CC or DPS 1b CC + SFN CA. Issue 1-1-3: Applicability rule between single carrier and CA CA cases have higher processing complexity than single carrier cases, so we prefer to define the applicability that UE skip single carrier test case if it explicitly passes corresponding CA test case. Issue 1-3-1: Release independent OK with the recommended WF. Issue 1-4-1: Channel model for HST-DPS Considering the necessary to track TCI in advance for DPS 1b, we are OK to extend the RRH visibility to [-Ds, Ds]. We don’t think it necessary to vary the propagation delay and Rx power. With current configuration, there is no big issue and it is convenient for the test. Also, we don’t think the requirements for DPS need to be re-simulated. |
| Qualcomm | Issue 1-1-1: Prefer Option 1 since both FDD 15kHz and TDD30kHz requirements get tested with the applicability rule and TDD 15kHz is not a typical scenario. Issue 1-1-2: Prefer Option 1. Issue 1-1-3: Ok with recommended WF. Issue 1-2-1: Ok with recommended WF. Issue 1-3-1: Ok with recommended WF. Issue 1-4-1: We are ok to extend the visibility for SSB and TRS. Modeling time varying path delay should not impact the performance much because TTL will keep correcting it at every TRS occasion. So, we don’t have any strong preference on time varying delay. We don’t agree to model time varying path power because it will be difficult to define the SNR in that case. Even in case of HST-SFN, SNR along the track is kept constant for the same reason. |
| Intel | Issue 1-1-1: SCS configurations and applicability rules for SCS configuration We are fine with Option 1 and also can compromise to Option 4 to address concerns from operators on test coverage. Issue 1-1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme & Issue 1-2-1: UE capability and network-assisted signalling We support Option 1. We do not support definition of UE capabilities just to select requirements for testing. If we need to reduce test burden, we should test advanced receiver rather than baseline processing that is tested in other test cases. Issue 1-1-3: Applicability rule between single carrier and CA Support the recommended WF. Issue 1-3-1: Release independent Support the recommended WF. Issue 1-4-1: Channel model for HST-DPS Support Option 1. We have prepared corresponding CR this meeting (R4-2109205) please double check email thread 320. The question to companies whether we need to clarify propagation conditions from -Ds to Ds/2 or from -Ds to Ds? The first option is technically proper but not symmetric. |
| Ericsson | Issue 1-1-1: Firstly we don’t want to specify the requirements with TDD 15kHz. Then our preference is option 1. If operators do not reduce the number of tests, we are also fine with option 3 as a compromise. Issue 1-1-2: Support Option 1. Issue 1-1-3: Option 1. Support the moderators recommended WF. Issue 1-2-1: Option 1. Support the moderators recommended WF. Issue 1-3-1: Support the moderators recommended WF. Issue 1-4-1: We are ok to extend the RRH visibility as Intel proposed. We have no strong view whether to extend to Ds/2 or Ds. It is not necessary to consider path delay and receiving power in the channel model. It is not the purpose of DPS test when RAN4 discussed. |
| Apple | Issue 1-1-1: SCS configurations and applicability rules for SCS configuration We still prefer option 1. We need not specify requirements for TDD 15KHz for CA scenario. Issue 1-1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We support option 1. We don’t see the motivation to introduce UE capability for meeting requirements. Issue 1-1-3: Applicability rule between single carrier and CA We are fine with Option 1/ recommended WF. Issue 1-2-1: UE capability and network-assisted signalling Option 1/ recommended WF. We don’t support introducing additional UE capability for meeting requirements. Issue 1-3-1: Release independent We support option1. Issue 1-4-1: Channel model for HST-DPS We think visibility of SSB/CSI-RS should be extended to -Ds to Ds. We don’t think asymmetric visibility assumption is realistic. |
| CMCC | Issue 1-1-1: SCS configurations and applicability rules for SCS configuration We are OK with option 2, 3, 4 Issue 1-1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We do not see the necessity to introduce new UE capability, DPS is mandatory support from Rel-15, no UE capability is needed. For HST-SFN, the capability introduced in Rel-16 can be reused. As for the applicability rule between HST-SFN CA and DPS CA, considering advanced receiver is required for HST-SFN, we slightly prefer option 1. Issue 1-1-3: Applicability rule between single carrier and CA We are not OK with option 1. The test cases for single carrier are for the functionality verification. It is not preferred to define applicability rule between single carrier and CA. Issue 1-2-1: UE capability and network-assisted signalling Option 1. We do not see the necessity to introduce new UE capability. DPS is mandatory support from Rel-15, no UE capability is needed. For HST-SFN, the capability introduced in Rel-16 can be reused. Issue 1-3-1: Release independent Option 2, and we are OK have further discussion. But we are not OK with option1 to mix with RRM. It is not a good way to mix the discussion between RRM and demodulation. For demodulation session, it is suggested to focus on the discussion from demodulation point of view and conclude whether it can be release independent from demodulation point of view firstly. Once we have conclusion separately in demodulation session and RRM session, we can further consider whether the whole feature can be release independent taking the conclusions from both sessions into account. |
| docomo | Issue 1-1-1: SCS configurations and applicability rules for SCS configuration Our preference is Option3. Issue 1-1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme We support Option1. As for Option2, we don't agree Option 2 because HST-SFN CA requirements are always skipped. Issue 1-2-1: UE capability and network-assisted signalling We support Option1. Issue 1-3-1: Release independent OK with recommended WF. In addition, we have a similar view as CMCC. We prefer to focus on the UE demodulation perspective. |

CRs/TPs comments collection

| CR tdoc number | Comments collection |
| --- | --- |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1-1: SCS configurations and applicability rules for SCS configuration | Companies’ views are summarized in the following table. Number of CA Duplex modes; Number of required tests if UE supports all duplex modes. Option 1 (Intel, Apple, Qualcomm, Huawei, Ericsson); 3 (FDD 15+FDD15, FDD 15+TDD 30, TDD 30+TDD 30); 2 (FDD 15 + TDD 30; TDD 30 + TDD 30). Option 2 (CMCC); 5 (FDD 15 + FDD 15; FDD 15 + TDD 15; FDD 15 + TDD 30; TDD 30 + TDD 30, TDD 15+TDD 15); 4 (FDD 15 + FDD 15; FDD 15 + TDD 15; FDD 15 + TDD 30; TDD 30 + TDD 30). Option 3 (CMCC, Ericsson , DOCOMO); 3 (FDD 15+FDD15, FDD 15+TDD 30, TDD 30+TDD 30); 3 (FDD 15 + FDD 15; FDD 15 + TDD 30; TDD 30 + TDD 30). Option 4* (Intel, CMCC); 5 (FDD 15 + FDD 15; FDD 15 + TDD 15; FDD 15 + TDD 30; TDD 30 + TDD 30, TDD 15+TDD 15); 2 (FDD 15 + TDD 30; TDD 30 + TDD 30) TDD 15 is tested if FDD 15+TDD 30 is not supported. Since only 1 company support option2, moderator recommends to remove option 2 and continue to discuss the other options in 2nd round. Recommended WF for 2nd round discussion: Continue to discuss option 1, option 3 and option 4 in 2nd round. |
| Issue 1-1-2: Applicability rule for HST-SFN joint transmission scheme and DPS transmission scheme & Issue 1-2-1: UE capability and network-assisted signalling | - Option 1 (Intel, Apple, Qualcomm, DOCOMO, CMCC): If UE supports demodulationEnhancement-r16, only HST-SFN JT requirements shall apply, otherwise HST-DPS requirements shall apply for CA. - Option 2 (Huawei): Define applicability rule that UE has passed DPS CA requirements can skip SFN CA requirements - Option 3 (Huawei): Define two UE capabilities for HST-DPS CA and HST-SFN CA, UE perform the test only when UE supports it. if UE supports both, test one scheme: - If UE has passed HST-DPS CA tests, HST-SFN CA tests can be skipped - If UE has passed HST-SFN CA tests, HST-DPS CA tests can be skipped Majority companies support option1, and also do not agree to introduce UE capabilities for HST-DPS and HST-SFN CA. Recommended WF for 2nd round discussion: Encourage the proponent company provide more justification on the necessity of introducing UE capabilities. |
| Issue 1-1-3: Applicability rule between single carrier and CA | Majority companies agree with option 1, while 1 company did not agree with option1. Recommended WF for 2nd round discussion: Continue to discuss in 2nd round. |
| Issue 1-3-1: Release independen | - Option 1 (Intel, Apple): Define HST CA demodulation requirements in release independent manner only if corresponding RRM requirements will be release independent. Align exact release number with RRM requirements if such approach will be agreed. - Option 2 (CMCC, DOCOMO): it is proposed that Rel-17 FR1 HST PDSCH CA requirements are release independent from Rel-15. - Option 3 (Huawei): Further study whether HST PDSCH CA requirements can be release independent from Rel-15. Tentative agreements: Discuss after RAN4 reach agreements on the UE capabilities. |
| Issue 1-4-1: Channel model for HST-DPS | - Option 1 (Intel, Ericsson): Add clarification on Doppler frequency for the link from the second nearest RRH for HST-DPS channel model. - Option 2 : - (Huawei, Qualcomm, Apple) Extend the visibility for SSB and CSI-RS for HST-DPS to -Ds to Ds. - Update HST-DPS channel model to at least model time varying path delay. - Further discuss if time varying path power should be considered for HST-DPS. 2 companies support option1, and 3 companies support to extend the visibility for SSB and TRS. Recommended WF for 2nd round discussion: Further discuss the following options in 2nd round: - Option 1: Add clarification on Doppler frequency for the link from the second nearest RRH for HST-DPS channel model. - Option 2: Extend the visibility for SSB and CSI-RS for HST-DPS to -Ds to Ds. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

Discussion on 2nd round (if applicable)

Open issues summary

Open issues

| Company | Comments |
| --- | --- |

Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Topic #2: Enhanced transmisison schemes

Agenda  7.9.3.3

Companies’ contributions summary

| TDoc | Title | Source | Proposals/ Observations |
| --- | --- | --- | --- |
| R4-2109214 | Views on FR1 HST PDSCH performance requirements for multi-DCI based Tx scheme. | Intel Corporation | Proposal #1: Do not define performance requirements for multi-DCI based Tx scheme in HST-SFN scenario. |
| R4-2110528 | Discussion on enhanced transmission schemes for NR UE HST FR1 performance requirements | Huawei, HiSilicon | Observation 1: For the fixed MCS, almost same maximum throughput can be calculated in theory for multi-DCI based multi-TRP comparing to DPS/SFN. Observation 2: Maximum throughput cannot achieved for multi-DCI based multi-TRP with MCS 17 and TDD 30 kHz regardless the value of timing offset Observation 3: For the fixed MCS, there is no performance gain for multi-DCI based multi-TRP comparing to HST-DPS all the time. Observation 4: Same UE capability of the maximum frequency offset tracking for all three transmission schemes. Observation 5: There is no any performance gain for multi-DCI based multi-TRP using link adaption under HST scenario. Proposal 1: Do not define requirements for multi-DCI based multi-TRP under HST scenario. |
| R4-2110529 | Simulation results on enhanced transmission schemes for NR UE HST FR1 performance requirements | Huawei, HiSilicon |  |
| R4-2110642 | PDSCH demodulation requirements with enhanced transmission schemes in HST scenario | Ericsson | Proposal: RAN4 do not define requirements for transmission scheme 2 (multi-DCI based transmission scheme) with HST scenario. |
| R4-2110939 | Discussion on multi-DCI transmission scheme for FR1 HST | MediaTek inc. | Proposal 1: Do not define requirements for transmission scheme 2. |

Open issues summary

Issue 2-1: Whether to specify PDSCH requirements for multi-DCI transmission scheme

- Agreements in RAN4#98bis-e meeting:

- Option 1: Do not define requirements for transmission scheme 2

- Option 2: Continue Evaluation of transmission scheme 2

- Companies are suggested to provide the performance evaluation of transmission scheme 2 (i.e., multi-DCI based transmission scheme) for following cases

  - Case  1: Fix the MCS along the track

  - Case  2: Vary the MCS along the track

- Note: To vary SNR along the track consider HST-SFN channel model from TS 38.101-4 without normalization, i.e.

- Proposals in RAN4#99-e meeting:

  - Option 1 (Intel, Huawei, Ericsson, MTK): Do not define performance requirements for multi-DCI based Tx scheme in HST-SFN scenario.

- Recommended WF

  - 4 companies provide simulation results, 1 company shows the performance evaluation also with varying MCS, and no performance gain is observed.

  - Can we agree with option 1?

Companies views’ collection for 1st round

| Company | Comments |
| --- | --- |
| Huawei | OK with Option 1. |
| Qualcomm | Ok with Option 1. |
| Intel | Support Option 1. |
| Ericsson | OK with Option 1. |
| Apple | We support option 1. |

CRs/TPs comments collection

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1: Whether to specify PDSCH requirements for multi-DCI transmission scheme | Tentative agreement: Do not define performance requirements for multi-DCI based Tx scheme in HST-SFN scenario. |

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

Discussion on 2nd round (if applicable)

Open issues summary

Open issues

| Company | Comments |
| --- | --- |

Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on NR HST demodulation | CMCC |  |
| Summary for FR1 HST demodulation results | Ericsson |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents