3GPP TSG-RAN WG4 Meeting # 99-e	R4-2108776

Electronic Meeting, May. 19-27, 2021

Source:	ZTE Corporation

Title:	On test cases for beam management in NR-U

Agenda item:	6.1.6.3.13

Document for:	Discussion

# 1	Introduction

In RAN4 98-bis-e, the third meeting to discuss performance part of NR-U, companies had some discussions on the scope of test cases, test configurations and the work plan. Company views on test cases for link recovery were captured in the WF [1] as shown below.

CCA probabilities for link recovery test cases The following options have been discussed during the meeting: - Option 1: Set the CCA parameters in the link recovery tests for NR-U as follows. For DL LBT parameters, RAN4 should wait for the conclusion of CCA models for NR-U RRM performance requirements. CCA parameters in link recovery tests for NR-U T1; T2; T3; T4; T5. PCCA,DL; semi-static channel access; 1.0; FFS; FFS; FFS; FFS. dynamic channel access; 1.0; FFS; FFS; FFS; FFS. PCCA,UL; 1.0; 1.0; 1.0; 1.0; 1.0. - Option 2: Set PCCA_DL=100% and PCCA_UL=100% during T1 for L1-RSRP measurement reporting tests. - Option 3: Set PCCA_DL=75% and PCCA_UL=75% during T2 for L1-RSRP measurement reporting tests. - Option 4 (new, based on comments on the 1st round): Discuss and agree with the general principles of CCA modelling and the default probabilities before agreeing on test specific CCA probabilities.

- This paper provides our view on the topic.

2	Discussion

First of all we think that Option 4 should be fine, which is to wait till the general principles of CCA models. However, as RAN4 is completing Release 16 work, we should try to also accelerate the process by making progress in parallel. Thus, we suggest to agree on Option 1, which specifies the probability of DL and UL CCA success rate under non-controversial scenarios and then further study the parameters which are not certain at this stage.

Taking Option 1 as a starting point, we can try to complete the table during this meeting and then we will have a quite complete set of test configurations so that the draft CRs for test cases can start to be drafted.

Agree on Option 1 in the WF [1].

3	Conclusion

Proposal 1: Agree on Option 1 in the WF [1].

References

- R4-2105709