3GPP TSG-RAN WG4 Meeting # 99-e	R4-2108781

Electronic Meeting, May. 19-27, 2021

Source:	ZTE Corporation

Title:	Discussion on impact of CSSF to positioning measurements

Agenda item:	6.5.1.4

Document for:	Discussion

# 1	Introduction

- The topic on CSSF was discussed during the RAN4 98-bis-e meeting without a conclusion. The current status is summarized in the WF [1] as shown below.

- This paper discusses the open issue listed above in the WF and provide our view.

2	Discussions

After analyzing the problem and all options, we think that we should not complicate the definition of CSSF just because of positioning. Sticking to R15 definition and make some extra clarifications sounds like the most reasonable and effective approach. As long as the MO on one PFL can be covered by the MG, the current CSSF definition is fine and we don’t see any new technical issue.

CSSF is derived in Rel-15 approach, and any PFL is counted as a candidate for a MG occasion as long as at least one PRS resource on that PFL is fully covered by the MGL excluding RF switching time.

The other options have some drawbacks in our view. For instance, calculating multiple CSSF values would complicate UE implementation and might also cause misalignment.

3	Conclusion

Proposal 1: CSSF is derived in Rel-15 approach, and any PFL is counted as a candidate for a MG occasion as long as at least one PRS resource on that PFL is fully covered by the MGL excluding RF switching time.

References

- R4-2105851