3GPP TSG-RAN WG4 Meeting # 99-e	R4-2108782

Electronic Meeting, May. 19-27, 2021

Source:	ZTE Corporation

Title:	Measurement Accuracy Requirements for UE Rx-Tx time difference

Agenda item:	6.5.2.2.2.3

Document for:	Discussion

# 1	Introduction

- The topic on accuracy requirements for UE Rx - Tx measurements was discussed during the RAN4 98-bis-e meeting without a conclusion. The current status is summarized in the WF [1] as shown below.

- This paper discusses the open issue listed above in the WF and provide our view.

2	Discussions

For the applicability under TA change, first of all, we don’t see a critical difference between UE autonomous change or network configured change. TA change will interrupt the measurement of UE Rx - Tx in a similar way for both reasons, thus, we suggest to treat these two scenarios jointly.

When the uplink transmission time changes for the UE, the requirements shall not apply since the Rx - Tx measurement is interrupted.

UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment or based on network-configured TA.

3	Conclusion

Proposal 1: UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment or based on network-configured TA.

References

- R4-2105750