3GPP TSG-RAN WG4 Meeting#99      	R4-2108934

E-meeting, 19th – 27th  May, 2021

Title:	TP to TR 38.717-03-02: CA_n5-n30-n66

Source:	Nokia, AT&T

Agenda item:	8.13.2

Document for:	Approval

# Introduction

This is a TP into TR 38.717-03-02 to introduce CA_n5-n30-n66

********************** Start of TP *********************************

### 5.1.X	CA_n5A-n30A-n66A

#### 5.1.X.1	Operating bands for CA

Table 5.1.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n5-n30-n66 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 5.1.X.2	Channel bandwidths per operating band for CA

Table 5.1.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CA_n5A-n30A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n30A-n66A | CA_n30A-n66A | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n5A-n66A | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  | CA_n5A-n30A | n5 | See CA_n5(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n30A-n66(2A) | CA_n30A-n66A | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  | CA_n5A-n66A | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |

#### 5.1.X.3	UE co-existence studies

There is 5th order IMD from uplink band CA_n5-n30 to band n66 DL.

#### 5.1.X.4	REFSENS requirements

Table 5.1.X.4-1 shows the required MSD levels for the CA configuration, it is proposed to reuse the MAS value of DC_12A-66A_n25A in TS 38.101-3.

Table 5.1.X.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n30A-n66A CA_n5A-n30A-n66(2A) | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n30 | 2307.5 | 5 | 25 | 2352.5 | N/A | FDD | N/A |
|  | n66 | 1725 | 5 | 25 | 2125 | 4 | FDD | IMD5 |

********************** End of TP *********************************