3GPP TSG-RAN WG4 Meeting # 99-e		R4-2109061

Electronic Meeting, May. 19-27, 2021

Agenda item:	9.7.2.1.1

Source: 	CATT

Title: 	General discussion on RRM requirements for NR FR1 HST

Document for:	Discussion

Introduction

In RAN4#98-bis-e meeting, a way forward on RRM requirements for NR FR1 HST was approved in [1].

In this contribution, we provide the further consideration of several open issues of RRM requirement for FR1 HST and give our proposals.

Discussion

- Scell link recovery

In way forward [1], it is listed as below:

- Option 1 (HW, MTK): For Scell link recovery, RAN4 needs to study how many band(s) is supported in R17 HST in FR1 - Option 2 (Ericsson, vivo, CATT, CMCC, Nokia): no need to have this limitation in the spec

The beam failure detection can be applied for SCell. In our view, it depends on network. There is no need to have the limitation in the spec.

Proposal 1: For SCell link recovery, it depends on network. There is no need to have the limitation in the spec.

- CSSF

In way forward [1], it is listed as below:

- Option 1(HW, MTK): For CSSF, RAN4 needs to study how many Scell(s) is supported in R17 HST in FR1 - Option 2 (Ericsson, Apple, CATT, CMCC, Intel): no need to have this limitation in the spec

We agree that using few numbers of SCells can enhance the performance. But in our view, it depends on network. There is no need to have the limitation in the spec.

Proposal 2: For CSSF, it depends on network. There is no need to have the limitation in the spec.

- Release independent

In Rel-16 HST, it is agreed that Rel-16 NR HST RRM enhanced requirements are release independent from Rel-15. Rel-17 is the enhancement of carrier aggregation. No new change from RAN1/RAN2 side. Rel-17 NR HST RRM enhancement can be release independent from Rel-15.

Proposal 3: Rel-17 NR HST RRM enhancement can be release independent from Rel-15.

Conclusion

In this contribution, we provide the further consideration of several open issues of RRM requirement for FR1 HST and give our proposals as follow:

Proposal 1: For SCell link recovery, it depends on network. There is no need to have the limitation in the spec.

Proposal 2: For CSSF, it depends on network. There is no need to have the limitation in the spec.

Proposal 3: Rel-17 NR HST RRM enhancement can be release independent from Rel-15.

Reference

[1] R4-2105793, “WF on FR1 HST RRM”, CMCC