3GPP TSG-RAN WG4 Meeting # 99-e		R4-2109062

Electronic Meeting, May. 19-27, 2021

Agenda item:	9.7.2.1.2

Source: 	CATT

Title: 	Discussion on intra-frequency measurement for NR FR1 HST RRM enhancement

Document for:	Discussion

Introduction

In RAN4#98-bis-e meeting, a way forward on RRM requirements for NR FR1 HST was approved in [1].

In this contribution, we provide the consideration of intra-frequency measurement for FR1 HST and give our proposals.

Discussion

In last two meetings, for intra-frequency measurement, it has been agreed for activated SCell: re-use PCell enhancement in Rel-16. Therefore, in such requirements in Table 9.2.5.1-1, Table 9.2.5.1-3, Table 9.2.5.2-5, Table 9.2.6.2-1, Table 9.2.6.2-3, Table 9.2.6.3-3, the additional note can be added as:

“NOTE: When RRM enhancement for high speed is configured for CA, the requirements apply to measurements of a secondary component carrier with active SCell.”

Proposal 1: Add additional note can be added in Table 9.2.5.1-1, Table 9.2.5.1-3, Table 9.2.5.2-5, Table 9.2.6.2-1, Table 9.2.6.2-3, Table 9.2.6.3-3 as “NOTE: When RRM enhancement for high speed is configured for CA, the requirements apply to measurements of a secondary component carrier with active SCell.”

For deactivated SCell for intra-frequency measurement, it has been agreed: re-use similar principle as Rel-16 HST:

In way forward [1], it is listed as below:

- PSS/SSS detection, time index detection, and measurement period for deactivated Scell - M2 defined in Rel-16 HST is reused for PSS/SSS detection delay requirements for deactivated Scell - M2 defined in Rel-16 HST is reused for time index detection delay requirements for deactivated Scell - Enhancement to PCell measurement in R16 HST can be reused for measurement period enhancement on deactivated Scell DRX cycle; T SSB_measurement_period_intra. No DRX Note 2; 5 x measCycleSCell x CSSFintra. DRX cycle≤ 160ms; 5 x max(measCycleSCell, M2 Note 2xDRX cycle) x CSSFintra. 160ms < DRX cycle≤ 320ms; 4 x max(measCycleSCell, M2 Note 2xDRX cycle) x CSSFintra. DRX cycle>320ms; Y Note 3 x max(measCycleSCell, DRX cycle) x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms. - FFS whether Rel-16F - FFS whether Rel-16 Kp requirements modifications (if any) shall also apply for R17 HST

Proposal 2: For deactivated SCell, reuse similar principle as Rel-16 HST and add additional note for SCells in such requirements.

For the remained issue of Kp, in current spec, Kp is used for the case which SMTC is partially overlapping with measurement gaps. For deactivated SCell, there is no Kp in Rel-15 for non-HST.  It is reasonable to keep Kp. But it’s better to conclude Kp for non-HST firstly.

Proposal 3: For deactivated SCell Kp issue, it is reasonable to keep Kp. But it’s better to conclude Kp for non-HST firstly.

Conclusion

In this contribution, we provide the consideration of intra-frequency measurement for FR1 HST and give our proposals as follow:

Proposal 1: Add additional note can be added in Table 9.2.5.1-1, Table 9.2.5.1-3, Table 9.2.5.2-5, Table 9.2.6.2-1, Table 9.2.6.2-3, Table 9.2.6.3-3 as “NOTE: When RRM enhancement for high speed is configured for CA, the requirements apply to measurements of a secondary component carrier with active SCell.”

Proposal 2: For deactivated SCell, reuse similar principle as Rel-16 HST and add additional note for SCells in such requirements.

Proposal 3: For deactivated SCell Kp issue, it is reasonable to keep Kp. But it’s better to conclude Kp for non-HST firstly.

Reference

[1] R4-2105793, “WF on FR1 HST RRM”, CMCC