3GPP TSG-RAN4 Meeting #99-e	R4-2109090

# Electronic meeting, May 19 – 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1871 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PRS RSTD measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Some agreements on RSTD measurement period are not captured in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture the agreements on RSTD measurement period (including TPRS,i, Nmuting, LPRS,i, requirements applicability) into the specification. - Some editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RSTD measurement requirement is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not include the corrections in the resubmitted CR R4-2109089. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.9.2.5	Measurements Period Requirements

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure and report multiple (up to the UE capability specified in Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

When measurement gaps and processing time T have overlap between different PRS frequency layers, TRSTD,Total is defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

Editor’s note:	FFS the RSTD measurement period when measurement gaps and processing time T do not have overlap between different PRS  frequency layers.

Where ,

i is the index of PRS frequency layer,

L is total number of PRS frequency layers, and

Teffect,i is the periodicity of PRS-RSTD measurement in PRS frequency layer i as defined further in this clause.

TPRS-RSTD,i is the measurement period for PRS RSTD measurement in PRS frequency layer i as specified below:

TPRS-RSTD,i=Nmuting*CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast ,

where:

Nmuting is the scaling factor when muting option 1 is applied defined as:

Nmuting = X * dl-prs-MutingBitRepetitionFactor

In which,

- X is the size of NR-MutingPattern-r16 for mutingOption1-r16

- Nmuting upper bound value is FFS

NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1; and in FR2,  NRxBeam,i = [8].

CSSFPRS,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in frequency layer i as defined in clause 9.1.5.2.

LPRS,i should be calculated by aggregating the duration of all the PRS resources that fall within MGs and are not muted. The observation window for LPRS is FFS.

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [34].

Nsample is the number of PRS RSTD samples and Nsample= 4.

Tlast is the measurement duration for the last PRS RSTD sample, including the sampling time and processing time, Tlast = Ti + LPRS,i ,

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i , is the periodicity of PRS RSTD measurement in frequency layer i.

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi.

MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the PRS frequency layer i.

TPRS,i is the periodicity of DL PRS resource on frequency layer i. If more than one PRS periodicities are configured in PRS frequency layer i, the least common multiple of PRS periodicities among all DL PRS resource sets is used to derive the measurement period of thatPRS frequency layer i.

- Note: For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

  - Definition of a PRS resource being fully or partially covered by the MG is FFS.

LPRS,i

NPRS,islot

{N,T}

N’

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration LPRS.

When PRS-RSRP is configured for DL-TDOA, RSTD and RSRP are performed over the same measurement period.

If handover occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements. The UE shall also meet the RSTD measurement requirements in this clause and measurement accuracy requirements in clause 10.1.23. However, in this case the RSTD measurement period TRSTD,total.HO shall be as follows:

TRSTD, total,HO=TRSTD, Total+K*Teffect+THO

Where,

-	K is the number of times handover occurs during TPRS-RSRPRSTD,total.HO;

-	Teffect is the largest Teffect,i among all PRS layers;

-	THO is the time during which the RSTD measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

-	THO is the time during which the RSTD measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

<End of Change 1>