3GPP TSG-RAN WG4 Meeting #99-e                                                               R4-2109109

Electronic Meeting, May. 19 - 27, 2021

Source: 	CATT

Title: 	Discussion on the UE RX RF requirements for 1024QAM for NR FR1

Agenda item:	9.6.3

Document for:	Approval

- Introduction

In RAN4 #98-bis-e, RAN4 agreed to use 0dB relaxation for maximum input level for 1024QAM compared to 256QAM and introduce an appropriate note in the maximum input level tables in TS 38.101-1 referring to new RMCs defined in annex A(RMCs to be settled during performance part of the WI)[1]. The remaining issues include how to introduce an appropriate note in the maximum input level tables in TS 38.101-1 and how to settle the RMCs for maximum input level for 1024QAM.

In this contribution, we provide our views on these issues for 1024QAM for NR FR1.

- Discussion

Maximum input level tables for 1024QAM

Taking a look at the maximum input levels in the Table 7.4-1 and Table 7.4A.1-1 in TS 38.101-1, the maximum input level for 64QAM and 256QAM were defined in different rows of tables with note 2 and note 3 respectively. The changes required for 1024QAM are shown in the following table 1 and table 2.

Table 1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -252 |  |  |  | -242 | -232 | -222 | -212 | -202 |  |  |  |  |
|  |  | -273 |  |  |  | -263 | -253 | -243 | -233 | -223 |  |  |  |  |
|  |  | -274 |  |  |  | -264 | -254 | -244 | -234 | -224 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Reference measurement channel is FFS for1024 QAM. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2: Maximum input level for Intra-band contiguous CA

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -232 | -232 | -252 |  |
|  |  | -253 | -253 | -273 |  |
|  |  | -254 | -254 | -274 |  |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Reference measurement channel is FFS for 1024 QAM. |  |  |  |  |  |

Proposal 1: It is proposed to adopt the changes for maximum input level for 1024QAM as in Table 1 and Table 2 for 38.101-1.

RMCs for maximum input level for 1024QAM

Referring to the RMCs in TS 36.101[2], the target coding rate for 256QAM and 1024QAM are same and both are 4/5.  In TS 38.101-1[3], the MCS index for 256QAM is 23 and target coding rate for 256QAM is also 4/5.  So the same target coding rate 4/5 for 1024QAM can be adopted. Referring to the the agreement in RAN1 #104[5], MCS indexs for 1024QAM in 1024QAM mcs table are listed in the following table. The target code rate of  MCS index 23 is 805.5/1024, which is colse to 4/5.  So the MCS index 23 in 1024QAM mcs table can be adopted for RMCs for maximum input level for 1024QAM.

| Table 5.1.3.1-4: MCS index table 4 for PDSCH |  |  |  |
| --- | --- | --- | --- |
| MCS Index | Modulation Order | Target code Rate R x [1024] | Spectral |
| IMCS | Qm |  | efficiency |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.377 |
| 2 | 2 | 449 | 0.877 |
| 3 | 4 | 378 | 1.4766 |
| 4 | 4 | 490 | 1.9141 |
| 5 | 4 | 616 | 2.4063 |
| 6 | 6 | 466 | 2.7305 |
| 7 | 6 | 517 | 3.0293 |
| 8 | 6 | 567 | 3.3223 |
| 9 | 6 | 616 | 3.6094 |
| 10 | 6 | 666 | 3.9023 |
| 11 | 6 | 719 | 4.2129 |
| 12 | 6 | 772 | 4.5234 |
| 13 | 6 | 822 | 4.8164 |
| 14 | 6 | 873 | 5.1152 |
| 15 | 8 | 682.5 | 5.332 |
| 16 | 8 | 711 | 5.5547 |
| 17 | 8 | 754 | 5.8906 |
| 18 | 8 | 797 | 6.2266 |
| 19 | 8 | 841 | 6.5703 |
| 20 | 8 | 885 | 6.9141 |
| 21 | 8 | 916.5 | 7.1602 |
| 22 | 8 | 948 | 7.4063 |
| 23 | 10 | 805.5 | 7.8662 |
| 24 | 10 | 853 | 8.3301 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 27 | 2 | reserved |  |

Proposal 2: The MCS index 23 in 1024QAM mcs table can be adopted for RMCs for maximum input level for 1024QAM.

- Conclusions

This contribution provides analysis on maximum input level for NR FR1 1024QAM. The following proposals are concluded:

Proposal 1: It is proposed to adopt the changes for maximum input level for 1024QAM as in Table 1 and Table 2 for 38.101-1.

Proposal 2: The MCS index 23 in 1024QAM mcs table can be adopted for RMCs for maximum input level for 1024QAM.

- References

- R4-2105416,WF on UE RF requirements for DL 1024QAM,Ericsson, [CATT, Huawei, HiSilicon, CMCC, China Unicom, Nokia, Nokia Shanghai Bell, Qualcomm],RAN4#98-bis-e

- 3GPP TS 36.101 V17.0.0 (2020-12)

- 3GPP TS 38.101-1 V17.0.0 (2020-12)

- R4-2109110, Draft CR for 38.101-1: Introduction of maximum input level for 1024QAM for NR FR1, CATT, RAN4#99-e

- Session notes for 8.16 (Introduction of DL 1024QAM for NR FR1), Ad-hoc chair (Samsung), RAN1#104-e