3GPP TSG-RAN WG4 Meeting #99-e                                                               R4-2109114

Electronic Meeting, May. 19 - 27, 2021

Source: 	CATT

Title: 	Discussion on the BS TX RF requirements for 52.6-71GHz

Agenda item:	9.15.5.1

Document for:	Approval

- Introduction

In RAN4#98bis-e, a WF [1] on BS RF TX requirements for 52.6 – 71 GHz was approved. The remaining issues in WF include output power accuracy values (EIRP and TRP), TX Off power, etc.

In this contribution, we provide our views on these issues for 52.6-71GHz.

- Discussion

EIRP accuracy and TRP accuracy

As per the WF [1], the agreements and the remaining issues concerning EIRP accuracy and TRP accuracy are shown as below:

- The BS output power for NR operation in 52.6 – 71 GHz range is declared by the manufacturer. Additional regional requirements can be added to specification to align with regulatory requirements. - Consider further detailed output power accuracy values (EIRP and TRP), taking into account also whether and/or how fractional bandwidth is applied.

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode | Fractional bandwidth |
| --- | --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD | 5.4% |
| n258 | 24250 MHz – 27500 MHz | TDD | 6.3% |
| n259 | 39500 MHz – 43500 MHz | TDD | 4.8% |
| n260 | 37000 MHz – 40000 MHz | TDD | 3.9% |
| n261 | 27500 MHz – 28350 MHz | TDD | 1.5% |

The factors [2] which influence EIRP and TRP accuracy are but not limited to the following:

1) Transceiver unit accuracy;

2) Array accuracy, including sub-array gain accuracy, mismatch errors and insertion loss variations;

3) Steer accuracy, including phase shift accuracy among transceiver units, and phase shifter accuracy.

Generally, the transceiver unit, array, and phase shifter are designed based on centre frequency and fractional bandwidth (FBW) of operating band. Fractional bandwidth is important factor affecting transceiver unit accuracy, array accuracy and steer accuracy. For FR2, the maximum fractional bandwidth of operating band in FR2 is ~ 6% listed as above table. And if fractional bandwidth of operating band is larger than 6%, two rated carrier EIRP may be declared by manufacturer. So when fractional bandwidth of operating band is less than 6%, the EIRP accuracy (±3.4dB) and the TRP accuracy (±3dB) is required. Referring to TR 37.842 and TR 37.843, EIRP accuracy and TRP accuracy can be expressed as equation 1 and equation 2 respectively.

[dB]                                                                                                     (Equation 1)

[dB]                                                                                                            (Equation 2)

From implementation point of view.  is important factor for EIRP accuracy, for FR2, the , 1.6dB means that amplitude deviation of steering relative to 1 can reach , which is feasible. So the can be applicable for 52.6-71GHz.

For microwave antenna at frequency range 57-66GHz[3], the gain deviation is about 0.5dB, and its fractional bandwidth of 57-66GHz is 7.32%. So for fractional bandwidth of operating band less than 6%,  can be applicable for 52.6-71GHz. Whenfor TRP accuracy equals 3.0dB, the, which can be applicable for 52.6-71GHz. So the existing FR2 EIRP accuracy (±3.4dB) and the TRP accuracy (±3dB) for FR2 do not make implementation to restrictive, and they can be reused for 52.6-71GHz.

Proposal 1: The existing FR2 EIRP accuracy (±3.4dB) and the TRP accuracy (±3dB) for FR2 can be reused for 52.6-71GHz.

OTA transmit OFF power

As per the WF [1], the agreements and the remaining issues concerning TX OFF power are shown as below:

- Further evaluate TX Off power, considering also whether existing FR2 requirement -36dBm/MHz can be valid.

For FR2, the OFF power -36dBm/MHz for FR2 was calculated using the following equation 3 on basis of the inter site interference propagation case with 128 transceivers, 28GHz typical frequency, and minimum path loss with 100m distance etc.

(Equation 3)

For 52.6-71 GHz, we can assume except NF and typical frequency, other parameters of inter site interference propagation case can keep.  The typical frequency of 52.6-71 GHz is 60GHz, so the path loss  of 60GHz is increased by ~6.6dB relative to 28GHz. Meanwhile the NF of 60GHz is increased by ~ 3dB relative to 28GHz. So  for 60 GHz is increased by ~9.6dB relative to 28GHz and reaches -26.4dBm/MHz. If OFF power is less than -36dBm/MHz, it must be less than -27dBm/MHz. So the existing FR2 requirement -36dBm/MHz is valid for 52.6-71 GHz.

Proposal 2: The existing FR2 OFF power requirement -36dBm/MHz is valid for 52.6-71 GHz.

- Conclusion

This contribution provides analysis on BS TX RF requirements for 52.6-71GHz for licensed operation. The following proposals and observations are concluded as follows:

Proposal 1: The existing FR2 EIRP accuracy (±3.4dB) and the TRP accuracy (±3dB) for FR2 can be reused for 52.6-71GHz.

Proposal 2: The existing FR2 OFF power requirement -36dBm/MHz is valid for 52.6-71 GHz.

- References

- R4-2106113,WF on BS RF TX requirements for 52.6 – 71 GHz, Nokia, Nokia Shanghai Bell,RAN4#98-bis-e

- R4-1714118,WF on TRP and EIRP accuracy requirements for NR BS type 2-O,Nokia, Nokia Shanghai Bell, Huawei, ZTE, Ericsson, NEC, RAN4#85

- TYA06U060S, www.tycc.cn