3GPP TSG-RAN WG4 Meeting # 99-e 												              R4-2109118

Electronic Meeting, 19th – 27th May, 2021

Agenda item:			9.12.2.1

Source:	CATT

Title:	Updated simulation assumptions for NTN co-existence

Document for:	Approval

- Introduction

In RAN4#99e meeting, simulation assumption has been approved with some open issues to be further studied [1]. This document presents an updated version to solve the open issue. (Updated part is marked in cyan)

- Discussion

Co-existence simulation scenarios

It is proposed to have a phase-by-phase approach to conduct co-existence study considering scenarios.

The proposed scenarios for coexistence study are in the following table.

Table 2.1-1 Scenarios for NTN-NTN/TN co-existence

| FR1: 2GHz |  |  | Set 1 |  |  | Set 22 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GEO | LEO 600km | LEO 1200km | GEO | LEO 600km | LEO 1200km |
| NR / NB-IoT | Rural |  | X | X | X | X | X | X |
|  | Urban macro |  | X | X | X | X | X | X |
|  | Dense Urban |  | X | X | X | X | X | X |
|  | Indoor |  | X | X | X | X | X | X |
| NTN1 | GEO3 | Set 1 | X | X | X | N/A | N/A | N/A |
|  | LEO 1200km |  | X | X | X | N/A | N/A | N/A |
|  | LEO 600km |  | X | X | X | N/A | N/A | N/A |
|  | GEO | Set 22 | N/A | N/A | N/A | X | X | X |
|  | LEO 1200km |  | N/A | N/A | N/A | X | X | X |
|  | LEO 600km |  | N/A | N/A | N/A | X | X | X |
| Note 1: Start with Earth Fixed beam first, Earth Moving Beams could be further discussed Note 2: Use Set 1 satellite antenna as the starting point for co-existence study. Set 2 might be used if any worst case in associate with Set 2 is found. Note 3: GEO and LEO only operate at adjacent channel. Note 4: Use GEO and LEO@600km when TN is victim. Note 5: Further check the possibility to remove LEO 1200km cases in future RAN4 meetings. |  |  |  |  |  |  |  |  |

The aggressor and victim combination is list in Table 2.1-2.

Table 2.1-2 Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL |  | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL |  | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL |  | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL |  | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with Band 34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with Band 34 TDD. | Phase 1 |
| 7 | TN with NTN | TN UL | NTN DL |  | Phase 2 |
| 8 | TN with NTN | NTN DL | TN UL |  | Phase 1 |
| 9 | NTN with NTN | NTN DL | NTN DL | LEO-LEO | Phase 1 |
|  |  |  |  | GEO-GEO | Phase 1 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | LEO-LEO | Phase 1 |
|  |  |  |  | GEO-GEO | Phase 1 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |

The proposed frequency and bandwidth are listed as table 2.1-3.

Table 2.1-3.  Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Rural | 2 GHz | 20MHz | FDD, TDD | [1] |
| TN Urban macro | 2 GHz | 20MHz | FDD, TDD | [1] |
| TN Dense Urban | 2 GHz | 20MHz | FDD, TDD | [1] |
| GEO | 2 GHz | [5] MHz for FR1 | FDD | [3] |
| LEO | 2 GHz | [5] MHz for FR1 | FDD | [3] |
| HAPS | 2 GHz | TBD | FDD | [1] |

Network layout model

Cellular cell structure is considered for both NTN and TN network layout.

Co-existence between NTN and TN

Coordination System

As illustrated in [2], there is no need to consider the curvature of earth for layout, assuming one satellite beam for the simulation. The distances for LEO-600, LEO-1200 and GEO can be assumed as 600km, 1200km and 35786km separately for any point under the 3dB satellite beam.

Simulation Methodology

Following simulation steps can be used for NTN-TN co-existence study.

- Generate aggressor and victim networks.

- [NTN central beam is at satellite nadir, surrounded with 6 co-frequency beams. NTN FRFs higher than 1 need to be considered. Assume one NTN aggressor as default.

- TN centre is randomly generated within the NTN central beam.

- For following two cases, more TN sites might be needed due to large coverage per beam of NTN.. The number of TN networks is FFS. Figure 2.2.1-1 gives an illustration of the deployment.

| No. | Combination | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | TN - NTN | TN DL (TN BS) | NTN UL (NTN satellite) |
| 2 | TN - NTN | TN UL (TN UE) | NTN UL (NTN satellite) |
| 3 | To be added |  |  |

Figure 2.2.1-1 Heterogeneous network layout

- For other cases, 19-cell with wrap around will be used.]

- UE associations

- TN UEs are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss.

- Deployment of NTN UE

NTN UE should be randomly generated within the NTN area. How does it co-locate with TN network depends on how we place the 2 networks.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering ACI as below:

- ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

- Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

- RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

Co-existence between NTN and NTN

The following 2 cases should be considered.

- One satellite carries two neighbour carriers, where the footprints of the 2 carriers are the same and coordinated see figure 2.2-1.

- Two satellites (GEO and LEO) operate on two neighbour carriers but at different height, see figure 2.2-2. The number of LEO satellite and footprints are FFS.

Figure2.1-1 Layout for coexistence between NTN and TN (TBD)

Figure 2.2-2 Layout for coexistence between NTN systems

Figure 2.2-3 Layout for coexistence between NTN systems (different height satellites)

Simulation parameters

NTN parameters

Satellite parameters

Two sets of satellite parameters are shown in Table 2.3-1 and Table 2.3-2 according to TR 38.821.

The satellite max Tx power can be calculated by the equation as below:

maxTx power(dBm)=EIRP density (dBW/MHz)+30+10 log10NRB*SCS*12-Max Gain

Table 2.3-1 Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Channel bandwidth |  | [5] MHz | [5]MHz | [5] MHz |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 m |
| Payload characteristics for UL transmissions |  |  |  |  |
| G/T | 2 GHz | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |

Table 2.3-2 Set-2 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 |
| Channel bandwidth |  | [5] MHz | [5] MHz | [5] MHz |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter |  | 450 km | 190 km | 90 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| G/T | 2 GHz | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |

Table 2.3-3 Other parameters for NTN

| Parameters | NTN | Remark |
| --- | --- | --- |
| Carrier frequency | 2GHz |  |
| The number of active UE (UL) (Note 1) | [10] |  |
| The number of active UE (DL) (Note 1) | [10] |  |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 2.6.2 |  |
| NTN system Noise figure in dB | 5 |  |
| Handover margin | [0]/[3]dB |  |
| Note 1: Further down-scoping is needed |  |  |

UE parameters

UE parameters are shown in Table 2.3-4

Table 2.3-4 UE characteristics for system level simulations

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 9 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi per element |

HAPS parameters

Refer to R4-2106106.

TN parameters

Table 2.3-5 Simulation assumptions of TN respectively based on NB-IoT and NR

|  | NB-IoT standalone | NR |
| --- | --- | --- |
| Carrier frequency in GHz | 2 | 2 |
| Size of each nominal channel BW in MHz | 0.2 | 20 |
| Transmission bandwidth in MHz | 0.18 | 9 |
| Environment | Urban macro Sub-urban Rural | Urban macro Sub-urban Rural |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA | 500 for 2GHz band for UMA FFS for other scenarios |
| System loading and activity | Full buffer 100% | Full buffer 100% |
| Network location | FFS | The centre of TN network is aligned with the centre of centre beam of NTN |
| DL subcarrier spacing | 15kHz | 15kHz |
| UL | See RP-152284 | OFDMA |
| DL power control | No | No |
| UL power control | 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB | 36.942 |
| Frequency reuse | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | 1 |
| Number of scheduled UE per cell (UL) | 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone | 3 |
| UE antenna height in meter | 1.5 | 1.5 |
| UE TX power in dBm | -40 to 23 | -40 to 23 |
| UE antenna gain in dBi | 0 | 0 |
| Building penetration loss | 45.820 Annex D.1 | In Pathloss model, TR 38.901 |
| Cell selection margin in dB | 3 | 3 |
| BS-MS min couple loss in dB | 70 | 70 for outdoor scenario in Table 2.1-1. 45 for indoor scenario in Table 2.1-1. |
| BS noise figure in dB | 5 | 5 |
| UE noise figure in dB | 9 | 9 |
| BS-UE path-loss model | TR36.942 macro urban | TR38.803 |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 | Deployment scenario related, referring to TR 38.901 |
| Shadowing correlation | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 |
| Link-level performance model |  | section 2.9 |
| UE distribution |  |  |
| Evaluation metrics | SINR vs ACS (as victim) | Throughput loss (refer to section 2.8) |

Table 2.3-6 Deployment-related parameters of TN (2 GHz)

|  | Urban Macro | Suburban Macro | Rural Macro | Remarks |
| --- | --- | --- | --- | --- |
| Cell radius in meters | 500 | 1000 | 5000 | ITU-R Report M.2292 |
| BS Antenna height in meters | 25 | 30 | 30 |  |
| Base Station Antenna Characteristics |  |  |  |  |
| Antenna Pattern | For AAS, see TR 37.842 Section 5.3.3 |  |  | TR 37.842 |
| Element Gain in dBi | 6.4 | 7.1 | 7.1 | 3GPP LS to ITU-R WP5D RP-200559 and ITU-R WP5D [IMT_Parameters] |
| H and V 3dB beamwidth of single element in degree | 90º for H 65º for V | 90º for H 54º for V | 90º for H 54º for V |  |
| H and V front-to-back ratio in dB | 30 for both H/V | 30 for both H/V | 30 for both H/V |  |
| Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |  |
| Antenna array configuration (Row × Column) | 8 x 8 elements | 8 x 8 elements | 8 x 8 elements |  |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.9 of wavelength for V | 0.5 of wavelength for H, 0.9 of wavelength for V |  |
| Conducted power per antenna element in dBm | 25 | 25 | 25 |  |
| Mechanical downtilt in degree | 10 | 6 | 3 |  |
| UE Parameters |  |  |  |  |
| UE Outdoor/indoor | 100% Outdoor |  |  |  |
| UE height in meter | 1.5 | 1.5 | 1.5 | 3GPP LS to ITU-R WP5D RP-200559 and ITU-R WP5D [IMT_Parameters] |
| Minimum BS-UE distance in meter | 35 | 35 | 35 |  |

Non-AAS antenna parameters will also be considered. Refer to Section 2.4.2.

Table 2.3-7 ACLR/ACS for TN  (2GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 45 dB | 45 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) | 37 |
|  | ACS | 33 | 28 |

Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.821.

Satellite antenna pattern

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered

1

where J1(x) is the Bessel function of the first kind and first order with argument x, a is the radius of the antenna's circular aperture, k = 2f/c is the wave number, f is the frequency of operation, c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam. Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

The normalized gain pattern for a = 10 c/f (aperture radius of 10 wavelengths) is shown in Figure 2.4.1-1.

Figure 2.4.1-1: Satellite antenna gain pattern for aperture radius 10 wavelengths, a=10 c/f

Satellite and UE beam forming pattern

The following table is agreed for the beam layout definition for a single satellite simulation in S-Band.

Table 2.4.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |
| --- | --- |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |
| UV plane illustration (extracted from [19]) |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS = sqrt(3) x sin(HPBW/2 [rad]) |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point |
| Frequency re-use factor | Option 1: 1 Option 2: 3 Option 3: 2 if polarization re-use is enabled |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UE distribution | The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |
| UE configuration | S-band: Handheld |
| UE orientation | Handheld: Random |
| UE attachment | RSRP |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). The atmospheric absorptions loss shall be considered. |  |

TN BS and UE antenna and beam forming pattern modeling

BS antenna

For AAS antennas and non-AAS antennas, refer to the ITU LS reply in R4-20xxxxx.

For UE antenna, an Omni-directional radiation pattern with antenna gain 0dBi is assumed

UE antenna

For UE antennas, an omni-directional radiation pattern with antenna gain 0dBi is assumed.

Propagation model

Propagation model between NTN and UE

Propagation model between NTN and UE could be referred to section 6.6 in TR 38.811.

Propagation model between TN BS and UE

Propagation model between TN BS and UE could be referred to section 7.4 in TR 38.901.

Propagation model between NTN BS and TN BS

Propagation model between NTN BS and TN BS should reference to TS 38.811 which is used for DL-UL cross link interference for S band.

Transmission power control model

TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

NTN UL TPC

Adopt the same TPC model of TN for NTN UL scenarios but needs to revise CLx-ile to align with UE UL power control parameters used in TR38.821.

DL TPC

For downlink scenario, no power control scheme is applied.

Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where,

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

Performance metric

For NR,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

For NB-IOT,

The average throughput loss and SNR loss, 5%-ile throughput loss and SNR loss should be according to 36.802

For NTN,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803 as the SINR-Throughput performance metrics, but , SNIRMIN, and SNIRMAX need to be further studied and decided for NR NTN.

- Conclusion

It is proposed to use the simulation assumptions in this paper as the starting point for NTN co-existence study.

- Reference

[1] R4-2106105, Simulation assumptions for NTN co-existence study, Samsung, CATT

[2] R4-2109119, comparison of co-existence performance w/w.o. consideration on earth curvature, CATT