3GPP TSG-RAN WG4 Meeting #99-e	R4-2109203

# Electronic, 19th– 27th April, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0189 | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-4: Performance requirements for single-DCI based multi-TRP Repetition Tx schemes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Perf |  |  |  |  |  | Date: |  |  | 2021-04-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Performacne requriements for single-DCI based multi-TRP Repetition Tx schemes are defined with square brackets |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove square brackets from performacne requriements for single-DCI based multi-TRP Repetition Tx schemes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performacne requriements for single-DCI based multi-TRP Repetition Tx schemes will be with square brackets |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1, 5.2.2.2, 5.2.3.1, 5.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF 1st CHANGE

Table 5.2.2.1.13-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 | R.PDSCH.1-2.5 FDD | 10 / 15 | 16QAM, 0.54 | TDLA30-10 | 2x2, ULA Low | 70 | 17.4 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |

END OF 1st CHANGE

START OF 2nd CHANGE

Table 5.2.2.1.13-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition (Note 1) | Correlation matrix and antenna configuration Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | BLER (%) | SNR (dB) (Note 4) |
| 1-1 | R.PDSCH.1-11.2 FDD | 10 / 15 | 16QAM, 0.54 | TDLA30-10 | 2x2, ULA Low | 1 (Note 3) | 2.6 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. Note 4: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |

END OF 2nd CHANGE

START OF 3rd CHANGE

Table 5.2.2.2.13-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 | R.PDSCH.2-2.5 TDD | 40 / 30 | 16QAM, 0.54 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 70 | 17.6 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |  |

END OF 3rd CHANGE

START OF 4th CHANGE

Table 5.2.2.2.14-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR (dB) (Note 4) |
| 1-1 | R.PDSCH.2-16.2 TDD | 40 / 30 | 16QAM, 0.54 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 1 (Note 3) | 2.5 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. Note 4: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |  |

END OF 4th CHANGE

START OF 5th CHANGE

Table 5.2.3.1.13-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 | R.PDSCH.1-2.5 FDD | 10 / 15 | 16QAM, 0.54 | TDLA30-10 | 2x4, ULA Low | 70 | 11.0 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |

END OF 5th CHANGE

START OF 6th CHANGE

Table 5.2.3.1.13-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | BLER (%) | SNR (dB) (Note 4) |
| 1-1 | R.PDSCH.1-11.2 FDD | 10 / 15 | 16QAM, 0.54 | TDLA30-10 | 2x4, ULA Low | 1 (Note 3) | -1.1 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. Note 4: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |

END OF 6th CHANGE

START OF 7th CHANGE

Table 5.2.3.2.13-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 | R.PDSCH.2-2.5 TDD | 40 / 30 | 16QAM, 0.54 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 70 | 10.5 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |  |

END OF 7th CHANGE

START OF 8th CHANGE

Table 5.2.3.2.13-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR (dB) (Note 4) |
| 1-1 | R.PDSCH.2-16.2 TDD | 40 / 30 | 16QAM, 0.54 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 1 (Note 3) | -1.5 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. Note 4: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |  |

END OF 8th CHANGE