3GPP TSG-RAN WG4 Meeting # 99-e                	R4-2109286

Electronic Meeting, May. 18-28, 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	NR-U PUSCH discussion and simulation results

Agenda item: 	6.1.7.4.2

Document for: 	Discussion

Introduction

This paper presents discussion and simulation results for interlaced PUSCH including GC-UCI considering the agreed simulation parameters on [1].

Discussion

In RAN4#98-bis-e there was an initial agreement on the fixed pattern for information bits [2].

Agreement: Tentative agreements: set each field in the CG-UCI to specific values: - Use a bit pattern {c0,c1,…,c17} that consists of : - HARQ process number: [c0,c1,c2,c3] = [0 0 0 1] - RV sequence: [c4,c5] = [0 0] - NDI: [c6] - Option 1: [c6] = [1] - Option 2: [c6] = toggle for every new transmission, e.g. 0 for even transmissions and 1 for odd ones COT sharing information field: [c7,c8,…c17] = [0,0,0,0,0,0,0,0,0,0,0]

There was a discussion on the feasibility to specify a fixed pattern for the tests. In this contribution we provide results comparing two patterns, a random pattern (Pattern 1) and a fixed pattern (Pattern 2).

|  | Pattern 1 | Pattern 2 |
| --- | --- | --- |
| HARQ process number | [0 0 0 0] | [0 0 0 1] |
| RV sequence | [0 0] | [0 0] |
| NDI | toggle between 0 and 1 for every new transmission | toggle between 0 and 1 for every new transmission |
| COT | Random | [0 0 0 0 0 0 0 0 0 0 0] |

The UE should normally toggle the NDI field (1 bit) for each new transmission. In this case, as the maximum number of HARQ transmissions for NR-U CG-UCI multiplexed on PUSCH is 1, the NDI was proposed to be fixed during the tests. However, depending on implementation, the gNB could consider an unexpected NDI as an error if this field is fixed while performing the tests.

Define performance requirements for CG-UCI toggling NDI for every new transmission (Option 2).

PUSCH Simulation results

Preliminary simulation results are presented for interlaced PUSCH in this section. The simulation parameters are shown in Table 1, and the results are shown in Table 2.

Table 1 Simulation assumptions for interlaced and contiguous PUSCH analysis

| Parameter | Value |
| --- | --- |
| Waveform | CP-OFDM |
| Allocation length | 14 symbols |
| Number interlaces | 1 |
| BW | 20 MHz |
| SCS | 15 and 30 kHz |
| TDD pattern | 7D2S1U S=6D:4G:4U for 30kHz SCS 3D1S1U S=10D:2G:2U for 15kHz SCS |
| DM-RS configuration | 1+1 |
| Mapping type | Type A and Type B |
| MCS (Table 1, namely “qam64”) | MCS 20 |
| Propagation condition | TDLA30-10 |
| Antenna config | 1x2 |
| Maximum number HARQ retransmissions | 4 |
| RV sequence | {0,2,3,1} |
| Test metric | SNR@70% TPUT |

Table 2 Simulation results for interlaced PUSCH for MCS 20

| Bandwidth | DMRS Type | SCS | RV seq. | SNR@70% TPUT |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ideal | Impairment |
| 20 | TypeA | 15 | {0.2.3.1} | 9.9 | 12.4 |
|  |  | 30 | {0.2.3.1} | 9.8 | 12.3 |
|  | TypeB | 15 | {0.2.3.1} | 9.9 | 12.4 |
|  |  | 30 | {0.2.3.1} | 9.8 | 12.3 |

GC-UCI simulation results

Preliminary simulation results are presented for GC-UCI in this section. The simulation parameters are shown in Table 3 and the simulation results are presented in Tables 4.

Table 3 CG-UCI simulation parameters

| Parameter |  | Value |
| --- | --- | --- |
| Channel model |  | TDLA30-10 |
| Bandwidth |  | 20 MHz |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| MCS |  | MCS 20 |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID=0 |
| Time domain | PUSCH mapping type | A,B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | 1 interlace |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
|  | Number GC-UCI information bit payload | 18 |
|  | Scaling | 1 |
| UCI | betaOffsetGC-UCI | 8 |
|  | UCI partition for frequency hopping | Disabled |

Table 4 CG-UCI simulation results

| Pattern | SCS | Mapping type | CG-UCI SNR@BLER=1% |  | PUSCH SNR@BLER=10% |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ideal | impairment | ideal | impairment |
| Pattern 1 (random COT info field) | 15 kHz | A | 3.2 | 5.7 | 14.2 | 16.7 |
|  |  | B | 3.3 | 5.8 | 14.2 | 16.7 |
|  | 30 kHz | A | 3.8 | 6.3 | 14.9 | 17.4 |
|  |  | B | 3.9 | 6.4 | 14.8 | 17.3 |
| Pattern 2 (fixed COT info field) | 15 kHz | A | 3.1 | 5.6 | 14.2 | 16.7 |
|  |  | B | 3.8 | 6.3 | 14.2 | 16.7 |
|  | 30 kHz | A | 3.9 | 6.4 | 14.9 | 17.4 |
|  |  | B | 4.6 | 7.1 | 14.8 | 17.3 |

Conclusion

In this contribution we provided interlaced PUSCH simulation results for NR-U. Among the discussion points, we derive the following observations and proposals:

- Define performance requirements for CG-UCI toggling NDI for every new transmission (Option 2).

References

- R4-2106010, WF on NR-U BS demodulation requirements, 3GPP TSG-RAN WG4 Meeting #98-bis-e, Huawei

- R4-2106134, Email discussion summary for [98-bis-e][316] NR_unlic_Demod_BS, 3GPP TSG-RAN WG4 Meeting #98-bis-e, Moderator (Huawei, HiSilicon)