3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2109287

# Electronic Meeting, May. 19-28, 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	NR-U PUCCH discussion and simulation results

Agenda item: 	6.1.7.4.3

Document for: 	Discussion

Introduction

This paper presents discussion on open points and simulation results of PUCCH with enhanced formatted for operation on unlicensed bands [1].

Discussion

In RAN4#98-bis-e there was an initial agreement on the fixed patterns for information bits [2].

Agreement: Issue 2-1-1: Test metric for PF3 Tentative agreements: Use Prob(DTX->ACK)<=1% and Prob(ACK miss)<=1% as test metric for PF3 Issue 2-2-1: Pattern for information bits Agreements from GTW: - PF0: [0] - PF1: [0 1] - for PF3: [0 0 0 0] Candidate options: Information bit pattern for PF2 - Option 1: [0 1 0 1… 0 1] including HARQ-ACK and CSI part 1 - Option 2: Random (random selection of information bits, same as LTE)

Based on agreements, the performance requirements of PUCCH formats 1 and 3 are defined with ACK missed detection metric: SNR@ACK missed <= 10-2 with SNR@Prob(DTX->ACK)≤ 10-2.

The performance requirement of PUCCH formats for ACK missed detection is determined by the probability of false detection of the ACK and the probability of detection of ACK. Therefore, the proposed bit patterns defined as all zeros (NACK) should be redefined.

PF0 and PF3 bit patterns are defined as all +0’s, and the test metric to define the performance requirements is ACK-missed detection.

Use bit patterns [1] and [1 1 1 1] for PF0 and PF3, respectively.

Simulation assumptions

Table 1 Simulation assumptions for PUCCH format 0 analysis

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of symbol | 1 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 Low |
| Test metric | SNR at ProbPUCCH DTX→Ack bits ≤ 10-2 Prob(Ack missed detection) ≤ 10-2 |
| Bit pattern | [1] including HARQ-ACK information only |

Table 2 Simulation assumptions for PUCCH format 1 analysis

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of symbols | 14 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| OCC index (timeDomainOCC) | 0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 Low |
| Test metric | SNR at ProbPUCCH DTX→Ack bits ≤ 10-2 Prob(Ack missed detection) ≤ 10-2 SNR at ProbPUCCH DTX→Ack bits ≤ 10-2 ProbPUCCH NACK→ACK bits ≤ 10-3 |
| Bit pattern | [0 1] including HARQ-ACK information only |

Table 3 Simulation assumptions for PUCCH format 2 analysis

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Intra-slot frequency hopping | N/A |
| Number of symbols | 1 |
| The number of UCI information bits | 22 bits |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 Low |
| OCC-Length-r16 | Not configured |
| Test metric | SNR @ ProbUCI block BLER=10-2 |
| Bit pattern | [0 1 … 0 1] |

Table 4 Simulation assumptions for PUCCH format 3 analysis

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Additional DM-RS configuration | No additional DM-RS |
| Number of symbols and UCI information bits | 4 bits and 4 symbols |
| Channel bandwidth | 20MHz |
| SCS | 15kHz 30kHz |
| Antenna configuration | 1T2R |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 Low |
| Index of OCC | n0 |
| Length of OCC | n1 |
| Cyclic shift index for DMRS | 0 |
| Test metric | SNR @ UCI block BLER=10-2 SNR at ProbPUCCH DTX→Ack bits ≤ 10-2 Prob(Ack missed detection) ≤ 10-2 |
| Bit pattern | [1 1 1 1] including HARQ-ACK information only |

Simulation results

Table 5 Simulation results for interlaced PUCCH format 0

| Format | Propagation conditions | SCS | Test metric | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- |
| Format 0 | TDLA30-10 | 15 kHz | 1% ACK miss | -4.8 | -2.3 |
|  |  | 30 kHz | 1% ACK miss | -3.7 | -1.2 |

Table 6 Simulation results for interlaced PUCCH format 1

| Format | Propagation conditions | SCS | Test metric | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- |
| Format 1 | TDLA30-10 | 15 kHz | 1% ACK miss | -16.4 | -13.9 |
|  |  |  | 0.1% NACK to ACK | -15.4 | -12.9 |
|  |  | 30 kHz | 1% ACK miss | -15.6 | -13.1 |
|  |  |  | 0.1% NACK to ACK | -14.5 | -12.0 |

Table 7 Simulation results for interlaced PUCCH format 2

| Format | Propagation conditions | SCS | UCI bits | OCC length | Test metric | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 2 | TDLA30-10 | 15 kHz | 22 | 1 | 1% UCI BLER | 1.2 | 3.7 |
|  |  | 30 kHz | 22 | 1 | 1% UCI BLER | 2.2 | 4.7 |

Table 8 Simulation results for interlaced PUCCH format 3

| Format | Propagation conditions | SCS | UCI bits | OCC length | Test metric | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 3 | TDLA30-10 | 15 kHz | 4 | 1 | 1% ACK miss | -5.1 | -2.6 |
|  |  |  |  |  | 1% UCI BLER | -9.9 | -7.4 |
|  |  | 30 kHz | 4 | 1 | 1% ACK miss | -4.2 | -1.7 |
|  |  |  |  |  | 1% UCI BLER | -8.9 | -6.4 |

Conclusion

In this contribution we provided updated interlaced PUCCH simulation results for NR-U considering the agreements from the last RAN4 meeting. Additionally, we brought up additional discussion on the bit patterns to be used for PF3, from which we derived the following observations and proposals:

Observation 1: PF0 and PF3 bit patterns are defined as all +0’s, and the test metric to define the performance requirements is ACK-missed detection.

Proposal 1: Use bit patterns [1] and [1 1 1 1] for PF0 and PF3, respectively.

References

- R4-2103989, Way forward on PUCCH demodulation requirements, 3GPP TSG-RAN WG4 Meeting #98-e, Ericsson

- R4-2106134, Email discussion summary for [98-bis-e][316] NR_unlic_Demod_BS, 3GPP TSG-RAN WG4 Meeting #98-bis-e, Moderator (Huawei, HiSilicon)

- R4-2014942, PUCCH Demodulation performance requirements for operation in unlicensed bands, 3GPP TSG-RAN WG4 Meeting #97-e, Nokia

- R4-2100576, PUCCH Demodulation performance requirements for operation in unlicensed bands, 3GPP TSG-RAN WG4 Meeting #98-e, Nokia

- R4-2104623, PUCCH Demodulation performance requirements for operation in unlicensed bands, 3GPP TSG-RAN WG4 Meeting #98-bis-e, Nokia