3GPP TSG-RAN WG4 Meeting # 99-e                	R4-2109291

Electronic Meeting, 19th - 28th May 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	RRM considerations for extension to 71 GHz

Agenda item: 	9.15.6

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to the operation between 52.6 GHz and 71 GHz. The work item description for the extension up to 71 GHz includes the following objectives related to RAN4 [2]:

- Core specifications for UE, gNB and RRM requirements [RAN4]: - Specify new band(s) for the frequency range from 52.6GHz-71GHz. The band(s) definition should include UL/DL operation and excludes ITS spectrum in this frequency range. - Specify gNB and UE RF core requirements for the band(s) in the above frequency range, including a limited set of example band combinations (see Note 1). - Specify RRM/RLM/BM core requirements. Note 1: The WI can be completed provided requirements for at least one band combination involving a new NR-U band is specified as long as it is in line with country-specific regulatory directives.

which includes planning RRM core requirements. This paper proposed some first aspects to be considered for the scope of the RRM core requirements. Among the aspects covered in this paper are the timing requirements that have already been discussed as part of the study item [7] whose results are reported at [6]. Other aspects include RRM requirements that are influenced by the new sub-carrier spacings and aspects related to the operation in unlicensed bands introduced for the operation above 52.6 GHz.

Discussion

During the study item phase RAN4 aspects were considered regarding timing aspects. The main conclusions related to RRM timing requirements are shown below [1]:

Another outcome of the study is related to the timing requirements. When considering RRM timing requirements for UEs, it is important that the TA advance requirements and initial timing requirements are revised to consider the shorter symbol timing of subcarrier spacings like 480 kHz and 960 kHz. Further investigation on the exact values for the RRM timing requirements may be needed.

It was identified during that study that, due to the shorter symbol length, the timing error requirements should be revised for 480 kHz and 960 kHz. These include the timing error limit, and the TA advance adjustment accuracy. The existing RRM requirements for initial timing acquisition are dependent on the SCS of both SSB and data. This accuracy needs to be scaled according to the UL SCS, resulting in a timing error that is only a fraction of the CP length. The same reasoning holds for the timing advance adjustment accuracy. Considering that, the following proposals are derived:

Define UE timing error limit requirements that consider shorter symbol duration of 480 kHz and 960 kHz sub-carrier spacings.

Define UE timing advance adjustment accuracy limit requirements that consider shorter symbol timing of 480 kHz and 960 kHz sub-carrier spacings.

Investigate exact values for timing requirements for 480 kHz and 960 kHz SCS.

The use of 480 kHz and 960 kHz SCS is agreed for both data and SSB, however the decision on its use for initial access is still pending. Considering that, several RRM requirements are affected by the SCS and would need to be revised in order to support the operation on frequencies up to 71 GHz. The following list of requirements depend on the numerologies and SSB SCS:

| Clause | Title | Comment |
| --- | --- | --- |
| 8 | Signalling characteristics |  |
| 8.2 | Interruption |  |
| 8.2.1 | EN-DC Interruption | Requirement depends on numerology Needs to include 480 and 960 kHz SCS |
| 8.2.2 | SA: Interruptions with Standalone NR Carrier Aggregation | Requirement depends on numerology Needs to include 480 and 960 kHz SCS |
| 8.2.3 | NE-DC Interruptions | Requirement depends on numerology Needs to include 480 and 960 kHz SCS |
| 8.2.4 | NR-DC: Interruptions | Requirement depends on numerology Needs to include 480 and 960 kHz SCS |
| 8.6.2 | DCI and timer based BWP switch delay on a single CC | BWP delay depends on numerology, and have to be revised for 480 kHz and 960 kHz |
| 9 | Measurement Procedure |  |
| 9.1 | General measurement requirement |  |
| 9.1.2 | Measurement gap | Revision of measurement gap parameters for 480 and 960 kHz SCS |
| 10 | Measurement Performance requirements |  |
| 10.1 | NR measurements |  |
| 10.1.3 | Intra-frequency RSRP accuracy requirements for FR2 | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB. Conditions defined at B.2.3 |
| 10.1.5 | Inter-frequency RSRP accuracy requirements for FR2 | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB. Conditions defined at B.2.3 |
| 10.1.8 | Intra-frequency RSRQ accuracy requirements for FR2 | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB. Conditions defined at B.2.2 |
| 10.1.10 | Inter-frequency RSRQ accuracy requirements for FR2 | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB |
| 10.1.13 | Intra-frequency SINR accuracy requirements for FR2 | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB. Conditions defined at B.2.2 |
| 10.1.15 | Inter-frequency SINR accuracy requirements for FR2 | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB. Conditions defined at B.2.2 |
| 10.1.20 | L1-RSRP accuracy requirements for FR2 | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB. Conditions defined at B.2.4 |
| 10.1.22 | CLI measurement accuracy requirements | Defined for 120 and 240 kHz SRS Needs definition for 480 and 960 kHz SRS. Conditions defined at B.2.7 |
| 10.1.28 | L1-SINR accuracy requirements for FR2 | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB |
| B.2 | Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state |  |
| B.2.2 | Conditions for NR intra-frequency measurements | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB |
| B.2.3 | Conditions for NR inter-frequency measurements | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB |
| B.2.4 | Conditions for NR L1-RSRP reporting | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB |
| B.2.5 | Conditions for RRC connection release with redirection to NR | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB |
| B.2.7 | Conditions for SRS-RSRP measurements | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SRS |
| B.2.2 | Conditions for NR intra-frequency measurements | Defined for 120 and 240 kHz SSB Needs definition for 480 and 960 kHz SSB |

Considering the table in the list above, the following proposal is derived:

Define RRM core requirements for interruption; measurement gaps; and Measurement performance requirements considering 480 and 960 kHz SCS.

The extension of up to 71 GHz includes unlicensed bands, the following objectives are described in the WID in relation to CCA/LBT [2]

- Physical layer procedure(s) including [RAN1]: - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz. - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified. - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access - Study, and if needed specify, energy detection threshold enhancement

When the CCA use for unlicensed bands is considered there are still many open issues on RAN1. These include whether operation without LBT is allowed in unlicensed bands, whether to use Cat-2 LBT, and whether to specify directional LBT. Considering the many open issues related to that, we suggest that RAN4 focuses on requirements that do not include LBT before more agreements are reached in RAN1 [3].

Focus initially on requirements without LBT and wait for RAN1 decisions regarding LBT.

Conclusion

This paper has presented Nokia’s views on RRM requirements for the extension to 71 GHz. From this discussion we have derived the following observations and proposals:

Proposal 1: Define UE timing error limit requirements that consider shorter symbol duration of 480 kHz and 960 kHz sub-carrier spacings.

Proposal 2: Define UE timing advance adjustment accuracy limit requirements that consider shorter symbol timing of 480 kHz and 960 kHz sub-carrier spacings.

Proposal 3: Investigate exact values for timing requirements for 480 kHz and 960 kHz SCS.

Proposal 4: Define RRM core requirements for interruption; measurement gaps; and Measurement performance requirements considering 480 and 960 kHz SCS.

Proposal 5: Focus initially on requirements without LBT and wait for RAN1 decisions regarding LBT.

References

- TR 38.808 v17.0.0, Study on supporting NR from 52.6 GHz to 71 GHz

- RP-210862, Revised WID:  Extending current NR operation to 71 GHz

- RP-210462, Status report for WI Extending current NR operation to 71GHz, 3GPP TSG RAN meeting #91e

- TS 38.133, V17.0.0, NR; Requirements for support of radio resource management,

- R1-2104029, Summary #2 of email discussion on initial access aspects of NR extension up to 71 GHz, 3GPP RAN1 #104-bis-e, Moderator (Intel Corporation)

- R4-2102009, TP to TR 38.808: Timing considerations for operation between 52.6 and 71 GHz, 3GPP RAN4 #98-e, Nokia, Nokia Shanghai Bell

- RP-181435, Study on NR beyond 52.6 GHz