3GPP TSG-RAN4 Meeting #99-e	R4-2109310

# Online, 19 – 27 May, 2021

Agenda item:	9.9.2.3

Source:	Apple

Title:	On PUCCH SCell activation and deactivation

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM requirement for PUCCH SCell activation and deactivation has been discussed and the conclusions were captured in the WF[1]. However, there are couple of open issues from last meeting, and in this contribution, we continue discussing the RRM requirement for PUCCH SCell activation.

# Discussion on general part of PUCCH SCell activation

In [1], there were couple of open issues on general part for PUCCH SCell activation, and we discuss those issues in this section.

  - CSI report of PUCCH SCell

In [1] the open issue of CSI report of PUCCH SCell is:

- Issue 1-1-1: The ending point of PUCCH SCell activation? - The ending point for valid TA case: - Option 1: (QC, Ericsson, Huawei, Xiaomi, Apple, NEC, Nokia, ZTE, OPPO, MTK) - For valid TA case, the ending point of PUCCH SCell activation should be the point when UE transmit valid CSI report on target PUCCH SCell. - Option 2: (NTT DOCOMO, CATT) - For valid TA case, the ending point of PUCCH SCell activation is the point when UE transmit valid CSI report on a certain cell (SpCell or PUCCH SCell or others, i.e. not to define which cell is used). - The ending point for invalid TA case: - Option 1: (QC, Ericsson, NTT DOCOMO, Huawei, Xiaomi, Apple, NEC, ZTE, OPPO, MTK) - For invalid TA case, the ending point of PUCCH SCell activation should be the point when UE transmit valid CSI report on target PUCCH SCell. - Option 2: (CATT) - For invalid TA case, the ending point of PUCCH SCell activation should be the point when UE transmit PRACH on target PUCCH SCell. - Option 3: (Nokia) - Depends on if the UE has transmitted the CSI reporting e.g. to inform network the beam information during the activation period.

In the PUCCH SCell activation requirement of LTE (TS36.133 as below), the ending point is when valid CQI is reported via SCell PUCCH as below.

For NR, in RAN1, the cross PUCCH group CSI reporting is not allowed, so the CSI report in this case can only be reported via PUCCH of target SCell. Furthermore, in RAN2 definition, the PUCCH shall be ready to be used when PUCCH SCell is activated, but in invalid TA case, transmitting PRACH on PUCCH of target SCell cannot mean that PUCCH is ready to use since UE needs RAR to acquire the timing for PUCCH transmission. Thus, similar as LTE, the PUCCH SCell activation requirement shall consider the scenario when CSI report of PUCCH SCell is transmitted on PUCCH SCell to be activated for both valid and invalid TA case.

Proposal 1: The ending point of PUCCH SCell activation should be the point when UE transmit valid CSI report on target PUCCH SCell, for both valid TA and invalid TA cases.

  - Beam information during PUCCH SCell activation

In FR2 PUCCH SCell activation, the conclusions on beam information are as below,

- Issue 1-1-3: Whether the beam information (SSB index) of PUCCH Scell is needed to be indicated to NW? - If the target PUCCH Scell is known, no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA, i.e., no additional SSB based beam measurement is needed. - If the target PUCCH Scell is unknown cell in FR2: - If there is at least one active serving cell on that FR2 band (following the same conditions in TS38.133 section 8.3.2 for intra-band FR2 Scell activation), no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA. - If there is no active serving cell on that FR2 band, need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA. - If the target PUCCH Scell is unknown cell in FR1: - Option 1: (QC, Ericsson, NTT DOCOMO, Huawei, vivo, Xiaomi, Apple, NEC, CATT, ZTE, OPPO, MTK) - If it is contiguous to an active serving cell in the same band (following the same conditions in TS38.133 section 8.3.2 for intra-band contiguous FR1 Scell activation), no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA. - If there is no contiguous active serving cell on that FR1 band, need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA. - Option 2: (Nokia) - No need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA, i.e., no additional SSB based beam measurement is needed. - FFS: - “need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA” doesn’t necessarily mean it is always possible for all cases, i.e. there can be cases where DL beam (SSB index) indication can’t be reported to the serving cell due to UE behavior for PUCCH grouping specified by RAN1/2. - FFS: this issue is for invalid TA case only.

For beam information, there are three parts we need to think of:

- The beam information for network to select associated SSB for RACH occasion

- The beam information for network to determine the TCI for PDCCH/PDSCH/CSI for target SCell

- The UL spatial relation for UE to determine the uplink beam for PUCCH

In RAN2 MAC spec TS38.321, it was defined that the RACH procedure on an SCell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000, and the UE behavior is also defined as:

Furthermore, in TS38.212, the DCI 1_0 scrambled by C-RNTI is specified as below,

Thus, it means network needs to know which SSB is the best/suitable one to associate with the RACH occasion. We think it shall consider two different scenarios: known PUCCH SCell case and unknown PUCCH SCell case. The FR1/FR2 known cell condition has been defined in TS38.133 section 8.3.2. Since here the beam information is needed for network to determine the associated SSB for RO and network may use the corresponding Rx beam to receive the CFRA preamble from UE, this beam information shall be considered for both FR1 (network side may also have Rx beamforming) and FR2.

Proposal 2: The beam information for network to determine the associated SSB for PDCCH triggered RACH occasion shall be considered for both FR1 and FR2 cases.

If the being-activated PUCCH SCell is known, network is aware of the SSB based measurement results and can directly determine the associated SSB for the PDCCH triggered RACH occasion and no additional SSB based beam measurement is needed for associated SSB determination. Otherwise, if the being-activated PUCCH SCell is unknown, there are some sub-cases to be considered:

- The unknown PUCCH SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band: following the same conditions in TS38.133 section 8.3.2 for intra-band FR2 SCell activation, no additional beam measurement is needed for associated SSB determination.

- The unknown PUCCH SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band: additional beam measurement is needed for associated SSB determination for RO. In legacy FR2 SCell activation for this case, RAN4 defined L1-RSRP measurement and reporting during the SCell activation delay.

- The unknown PUCCH SCell being activated belongs to FR1 and it is contiguous to an active serving cell in the same band: following the same conditions in TS38.133 section 8.3.2 for intra-band contiguous FR1 SCell activation, no additional beam measurement is needed for associated SSB determination.

- The unknown PUCCH SCell being activated belongs to FR1 and if there is no contiguous active serving cell on that FR1 band: additional beam measurement is needed for associated SSB determination for RO. In legacy FR1 SCell activation for this case, RAN4 did not define L1-RSRP measurement and reporting during the SCell activation delay, however, in order to determine the associated SSB for RO in FR1 PUCCH SCell activation, the L1-RSRP measurement/reporting in PUCCH SCell activation based on SSB on the target PUCCH SCell is needed.

Proposal 3:

If the target PUCCH Scell is known, no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA, i.e., no additional SSB based beam measurement is needed.

If the target PUCCH Scell is unknown cell in FR2:

- If there is at least one active serving cell on that FR2 band (following the same conditions in TS38.133 section 8.3.2 for intra-band FR2 Scell activation), no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

- If there is no active serving cell on that FR2 band, need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

If the target PUCCH Scell is unknown cell in FR1:

- If it is contiguous to an active serving cell in the same band (following the same conditions in TS38.133 section 8.3.2 for intra-band contiguous FR1 Scell activation), no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

- If there is no contiguous active serving cell on that FR1 band, need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

Even though the SSB index is needed only when PDCCH triggered RACH is needed, that is, SSB index is needed for network for invalid TA case. However, even for valid TA case, the beam information shall be also used for TCI for PDCCH/PDSCH/CSI of target SCell in FR1/FR2 and uplink beam for PUCCH in FR2.

This is the spatial information for target PUCCH and it’s not related with timing information, so this beam information shall be indicated to NW for both valid TA and invalid TA cases.

Proposal 4: beam information of PUCCH SCell is needed to be indicated to NW for both valid and invalid TA cases.

However, how to report the beam information to network is another issue 1-1-3b identified in last meeting,

- Issue 1-1-3b: If the conclusion of issue 1-1-3 is the beam information of PUCCH Scell is needed to be indicated to NW (maybe in certain cases), how to resolve the indication issue (e.g. the procedure)? - Option 1: (Huawei) - Allow UE to use CBRA for PUCCH SCell activation. - Option 2: (MTK, QC, Ericsson, CATT) - The valid case: - UE may measure the quality of the PUCCH SCell and report the beam information to network via SpCell. - Network transmits the downlink signals via the beam reported by UE and UE can transmit the uplink signals with valid TA. - The invalid case: - UE may measure the quality of the PUCCH SCell and report the beam information to network via SpCell. - Network will indicate the PDCCH order to UE and then UE will trigger the random access procedure for obtaining the TA command. - After UE obtain the valid TA, UE may transmit the CSI-reporting on its own PUCCH resource. - Note: the procedure needs to be confirmed by RAN1/2 - Option 3: (Apple) - Not define the PUCCH SCell activation requirement for unknown cell case. - Option 4: (Huawei, Apple, Nokia, NTT DOCOMO, ZTE, CATT, MTK) - Send LS to RAN1/RAN2 asking for the feasible solutions for transmitting beam information during activating a PUCCH SCell. - Option 5: (vivo, NEC, OPPO) - Need further check

Based on the PUCCH group definition, the CSI reporting cross PUCCH group is not supported in 3GPP. In TS38.214, the CSI definition is as in the following table, and both SSBRI and L1-RSRP is CSI. Thus, L1-RSRP can neither be reported via PCell PUCCH to network, and that means, neither TCI for PDCCH/PDSCH/CSI for target SCell nor UL spatial relation for UE to determine the uplink beam for PUCCH would be problematic without such beam information report (CSI report).

In order to address this issue, we prefer to not change RAN1 or RAN2 for existing flow of CSI reporting or RACH type since this HO with PUCCH is a R15 feature in RAN1/2, and there might be some alternatives to address this CSI reporting issue, e.g.,

    - Alt 1: using L3 measurement report of PUCCH SCell via SpCell PUSCH

    - Alt 2: Not define the requirement for the unknown PUCCH SCell activation

Using alt 1 (periodic L3 measurement report of PUCCH SCell via PCell PUSCH) which can also provide SSB beam information for network to determine the associated SSB for the PDCCH order based CFRA, or we may directly skip the requirement for this unknown PUCCH SCell activation since we did not see strong motivation/benefit to blindly activate such unknown SCell. However, we understand that companies may have different views on RAN1/2 definitions, and therefore if we cannot conclude on any solution in RAN4 we are open to send LS to RAN1/2 for clarification of this CSI reporting during PUCCH SCell activation.

Proposal 5: discuss if RAN4 could have conclusion on how to indicate beam information of PUCCH SCell to network during PUCCH SCell activation, following alternative solutions are considered:

    - Alt 1: using L3 measurement report of PUCCH SCell via SpCell PUSCH

    - Alt 2: Not define the requirement for the unknown PUCCH SCell activation

RAN4 sends LS to RAN1/2 for clarification on this issue only when the solutions could not be concluded in RAN4.

Regarding the L1-RSRP reporting, in last meeting we had following options:

- Issue 1-1-4: Which cell is the L1-RSRP reporting transmitted for PUCCH SCell activation? - Option 1: (QC, Ericsson, vivo, Xiaomi, CATT, ZTE) - L1-RSRP report is transmitted on the SpCell if L1-RSRP report is needed. - Option 2: (Huawei, Apple, NTT DOCOMO, CATT) - Depends on the conclusion in issue 1-1-3 - Option 3: (NEC, vivo) - Need further study - Option 4: (Nokia, ZTE) - Could be part of the LS in 1-1-3b for more inputs from RAN1/2.

As we analyzed above, L1-RSRP is also one type of CSI, and therefore L1-RSRP of PUCCH SCell can neither be reported via SpCell PUCCH. If the target SCell is known, L1-RSRP report is not needed since the measurement report before activation already provided such information to network. However, if the target SCell is unknown, the L1-RSRP issue could be handled in the same way as beam information reporting issue in issue 1-1-3b.

Proposal 6: same solution of beam reporting in issue 1-1-3b shall be applied to L1-RSRP reporting of target PUCCH SCell during unknown PUCCH SCell activation.

Regarding the uplink spatial relation, in last meeting we had following options:

- Issue 1-1-5-1: Whether the UL spatial relation is needed for PUCCH SCell activation for valid TA case? - Option 1: (QC, Ericsson, NTT DOCOMO, Huawei, vivo, Xiaomi, Apple, NEC, Nokia, ZTE, MTK) - The UL spatial relation of PUCCH on target being-activated SCell should be considered for PUCCH SCell activation in FR2 only. - the time uncertainty of the MAC CE for UL spatial relation activation of PUCCH in target being-activated SCell shall be defined in the baseline FR2 SCell activation delay part (Tactivate_basic). Details are FFS - Option 3: (CATT) - Depending on issue 1-1-1. If the ending point is defined at the point when UE transmit PRACH on target PUCCH SCell, the PUCCH spatial relation is not needed for PUCCH SCell activation. - Issue 1-1-5-2: Whether the UL spatial relation is needed for PUCCH SCell activation for invalid TA case? - Option 1: (QC, Ericsson, NTT DOCOMO, Huawei, vivo, Xiaomi, Apple, NEC, Nokia, ZTE) - The UL spatial relation of PUCCH on target being-activated SCell should be considered for PUCCH SCell activation in FR2 only. - the time uncertainty of the MAC CE for UL spatial relation activation of PUCCH in target being-activated SCell shall be defined in the baseline FR2 SCell activation delay part (Tactivate_basic). Details are FFS - Option 2: (MTK) - No - Option 3: (CATT) - Depending on issue 1-1-1. If the ending point is defined at the point when UE transmit PRACH on target PUCCH SCell, the PUCCH spatial relation is not needed for PUCCH SCell activation.

The UL spatial relation, for UE to determine the uplink beam for PUCCH, shall be considered during the PUCCH SCell activation. As in proposal 1, the SCell PUCCH shall be used for CQI reporting during the activation procedure, so, that means the PUCCH shall be ready to use when UE completes the PUCCH SCell activation. However, it’s necessary to differentiate the FR1 and FR2 cases here. In FR1 the UL spatial relation of PUCCH on target being-activated SCell is not needed, while in FR2 the UL spatial relation should be considered for PUCCH SCell activation.

However, the UL spatial relation activation for PUCCH is carried on a different MAC CE command from the SCell activation MAC, PDCCH/PDSCH activation, or SP-CSI reporting activation, and this new MAC CE for UL spatial relation activation shall be also taken into account in the baseline activation delay. Accordingly, the time uncertainty of the MAC CE for UL spatial relation activation shall be defined in the baseline FR2 SCell activation delay.

Since PUCCH uplink spatial relation is needed for PUCCH transmission on spatial domain, it’s irrelevant with whether TA is valid or not.

Proposal 7: For both valid TA and invalid TA cases in PUCCH SCell activation:

- the UL spatial relation of PUCCH on target being-activated SCell should be considered for PUCCH SCell activation in FR2 only.

  - the time uncertainty of the MAC CE for UL spatial relation activation of PUCCH in target being-activated SCell shall be defined in the baseline FR2 SCell activation delay part (Tactivate_basic). Details are FFS.

Regarding the UE capability for FR2 PUCCH SCell (de)activation requirements, in last meeting we had following options:

- Issue 1-1-7: UE capability for FR2 PUCCH SCell (de)activation requirements? - Option 1: (Qualcomm, NTT DOCOMO) - For UEs not supporting one of the following capabilities, FR2 PUCCH SCell (de)activation requirements are not defined. - beamCorrespondenceWithoutUL-BeamSweeping - beamCorrespondenceSSB-based-r16. - Option 2: (Apple, Huawei, ZTE, OPPO, vivo, Nokia, MTK, CATT) - Need more discussion

In TS38.306, the beamCorrespondenceWithoutUL-BeamSweeping is a R15 UE capability for beam correspondence, and it’s also used as the side condition of applicability for uplink spatial relation switching requirement. However, the beamCorrespondenceSSB-based-r16 is a R16 feature, and in TS38.306 it defined that when UE supports neither beamCorrespondenceSSB-based-r16 nor beamCorrespondenceCSI-RS-based-r16, gNB can expect the UE to fulfil beam correspondence based on Rel-15 beam correspondence requirements. beamCorrespondenceSSB-based-r16 capability can provide better beam correspondence performance based on SSB, but R15 beam correspondence performance shall still be guaranteed as long as UE supports beamCorrespondenceWithoutUL-BeamSweeping.

Even if UE needs beam correspondence for PDCCH triggered RACH based on SSB measurement, the R15 capability can also support this CFRA function. So, in our understanding, that means the PUCCH SCell activation requirement applicability is up to beamCorrespondenceWithoutUL-BeamSweeping.

Proposal 8: For UEs not supporting beamCorrespondenceWithoutUL-BeamSweeping, FR2 PUCCH SCell (de)activation requirements are not defined.

# Activation requirement for PUCCH SCell

In last meeting, the conclusions for valid TA case are as below,

- Issue 1-2 PUCCH Scell activation delay requirement for valid TA case - Option 1: (Xiaomi, OPPO, CMCC, Ericsson, NTT DOCOMO, vivo, Nokia, CATT) - Reuse the Rel-15 SCell activation delay requirement which is (( THARQ + Tactivation_time +TCSI_Reporting)/ NR slot length). - Option 2: (Apple, Huawei, Xiaomi, ZTE, QC) - In FR1, reuse the Rel-15 SCell activation delay requirement which is (( THARQ + Tactivation_time +TCSI_Reporting)/ NR slot length). - In FR2, use normal SCell activation delay (i.e., (( THARQ + Tactivation_time +TCSI_Reporting)/ NR slot length);) in TS38.133 section 8.3.2 as baseline, but the time uncertainty of the MAC CE for UL spatial relation activation of PUCCH in target being-activated SCell shall be considered in the baseline Tactivation_time. - Option 3: (MTK, NEC) - Wait for the conclusions of other open issues.

If the TA is valid for PUCCH SCell at UE, the TA acquisition time is not needed, but UE still needs the UL spatial relation activation for the PUCCH on the target being-activated SCell for FR2 case. Thus, when the TA associated with target PUCCH SCell is valid, the PUCCH SCell activation delay in FR1 could be as same as normal SCell activation delay in TS38.133 section 8.3.2; while the PUCCH SCell activation delay in FR2 could use normal SCell activation delay in TS38.133 section 8.3.2 as baseline, and the time uncertainty of the MAC CE for UL spatial relation activation of PUCCH in target being-activated SCell shall be considered in the baseline FR2 SCell activation delay part.

Proposal 9: Regarding PUCCH Scell activation delay requirement for valid TA case,

- In FR1, reuse the Rel-15 SCell activation delay requirement which is (( THARQ + Tactivation_time +TCSI_Reporting)/ NR slot length).

- In FR2, use normal SCell activation delay (i.e., (( THARQ + Tactivation_time +TCSI_Reporting)/ NR slot length);) in TS38.133 section 8.3.2 as baseline, but the time uncertainty of the MAC CE for UL spatial relation activation of PUCCH in target being-activated SCell shall be considered in the baseline Tactivation_time.

The conclusions of invalid TA case from last meeting were:

- Issue 1-3-1: The PUCCH SCell activation requirements for invalid TA case - Option 1: (Apple, Xiaomi, OPPO, CMCC, NTT DOCOMO, vivo, MTK) - If UE does not have the valid TA on the PUCCH SCell being activated, an additional UL synchronization procedure to obtain the valid TA shall be considered which including the following factors: - the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell (T1); - the delay for obtaining a valid TA command for the sTAG to which the SCell configured with PUCCH belongs (T2); - the delay for applying the received TA for uplink transmission (T3) - Option 2: (CATT) - Only T1 (The delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell) need to be considered - Option 3: (Ericsson, Nokia, NEC, Huawei) - Depending on the conclusions of other open issues. FFS.

When the TA is not valid for PUCCH SCell, UE needs to acquire the TA info during the PUCCH activation. In LTE, additional delay for TA acquisition consists of three parts:

- LTE T1 is the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell. T1 is up to 25 subframes and the actual value of T1 shall depend upon the PRACH configuration used in the PUCCH SCell.

- LTE T2 is the delay for obtaining a valid TA command for the sTAG to which the SCell configured with PUCCH belongs. T2 is up to 13 subframes.

- LTE T3 is the delay for applying the received TA for uplink transmission. T3 is 6 subframes.

In NR PUCCH SCell activation with invalid TA, those three additional delay parts are still needed for TA acquisition. But the value for each part shall be revisited based on NR UE capability/configuration.

Proposal 10: Regarding the PUCCH SCell activation requirements for invalid TA case,

- If UE does not have the valid TA on the PUCCH SCell being activated, an additional UL synchronization procedure to obtain the valid TA shall be considered which including the following factors:

  - the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell (T1);

  - the delay for obtaining a valid TA command for the sTAG to which the SCell configured with PUCCH belongs (T2);

  - the delay for applying the received TA for uplink transmission (T3)

In NR handover requirement, the uncertainty delay of RACH occasion is defined as,

So, the T1 for NR PUCCH SCell activation could be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Proposal 11: In NR PUCCH SCell activation delay requirement with invalid TA, T1 is the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell. T1 is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

T2 in LTE is up to the RAR window position on time domain, and it’s up to 13ms since RAR window starts 3ms after the PRACH msg1 in LTE and the RAR window length is up to 10ms. In NR direct SCell activation requirement at HO, the T2 is also used for the time uncertainty of RAR/TA-command reception. Since in NR the RAR window is configurable and there is no fixed upper bound for RAR window length, so we could reuse the T2 definition from direct SCell activation requirement at HO.

Proposal 12: In NR PUCCH SCell activation delay requirement with invalid TA, T2 is the delay from slot n + (Tactivate_basic +T1)/NR slot length until UE has obtained a valid TA command for the target PUCCH SCell being activated. Tactivate_basic is the normal SCell activation delay in TS38.133 section 8.3.2. slot n is the slot when UE received PUCCH SCell activation MAC CE.

T3 in NR PUCCH SCell activation requirement is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated. Similar as in NR direct SCell activation at HO, T3 is greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213.

Proposal 13: In NR PUCCH SCell activation delay requirement with invalid TA, T3 is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213.

# Interruption requirement for PUCCH SCell activation/deactivation

In last meeting, the conclusions for interruption requirements are as below,

- Issue 1-4-1: Interruption requirements for PUCCH Scell activation in valid TA case - Reuse the existing requirement for Scell activation in Rel-15. - Issue 1-4-2: Interruption requirements for PUCCH Scell activation in invalid TA case - Option 1: (MTK) - The interruption requirement shall include the existing requirement for Scell activation in Rel-15. - FFS whether to introduce interruption by PRACH transmission due to different SCS. - Option 2: (Apple, Xiaomi, ZTE, Ericsson, Nokia, NEC, CATT) - Reuse the interruption requirement of normal Scell activation. - Option 3: (Huawei, QC, OPPO) - FFS after the additional delay are clearly defined

In invalid TA case, the CFRA is used on target PUCCH SCell, and if SCell RACH has different SCS from the PCell data/control channel, the UE capability of diffNumerologyAcrossPUCCH-Group shall be considered. In TS38.306, it defined that,

The additional interruption shall be considered for PUCCH SCell activation with invalid TA, when target PUCCH SCell RACH has different SCS from spCell data/control channel and UE does not support diffNumerologyAcrossPUCCH-Group.

Proposal 14: regarding interruption requirements for PUCCH Scell activation in invalid TA case:

- The interruption requirement shall include the existing requirement for Scell activation in Rel-15.

- Introduce additional interruption by PRACH transmission when target PUCCH SCell RACH has different SCS from spCell data/control channel and UE does not support diffNumerologyAcrossPUCCH-Group.

# Applicability of PUCCH Scell activation requirements

Regarding the applicability of PUCCH SCell activation requirement, the LTE conditions could also be used as baseline, as duplicated as below,

The above delay requirement (Tdelay_PUCCH SCell) shall apply provided that: - The UE has received a PDCCH order to initiate RA procedure on the PUCCH SCell within Tactivate_basic otherwise additional delay to activate the SCell is expected; and - The RA on PUCCH SCell is not interrupted by the RA on PCell otherwise additional delay to activate the SCell is expected; and - No SRS carrier based switching occurs during the SCell activation procedure otherwise the PUCCH SCell activation delay (Tdelay_PUCCH SCell) can be extended.

In NR PUCCH SCell activation requirement, it’s also necessary to assume that UE has received a PDCCH order to initiate RA procedure on the PUCCH SCell within Tactivate_basic otherwise additional delay to activate the SCell is expected.

In LTE applicability requirement, the RA colliding between PCell and target PUCCH SCell would also delay the completion of PUCCH SCell activation, however, dual RACH on PCell and SCell is not an issue to NR anymore with help of RAN1 power allocation design, and it’s also reflected in the NR PSCell addition requirement in TS38.133 section 8.9.2.

In LTE spec, only the possible interruption from SRS carrier based switching is considered to extend the PUCCH SCell activation delay, however, in NR there are many factors that can cause interruption to the PUCCH SCell activation, e.g. SRS carrier switching, BWP switching, CGI reading with autonomous gap, and etc. The target PUCCH SCell could be NR SCell in EN-DC, or in standalone NR carrier aggregation, or in NE-DC, or in NR-DC, and the interruption causes for different NR operation mode is summarized as,

- The causes of interruption in EN-DC is specified in TS38.133 section 8.2.1.1.

- The causes of interruption in NR SA is specified in TS38.133 section 8.2.2.1.

- The causes of interruption in NE-DC is specified in TS38.133 section 8.2.3.1.

- The causes of interruption in NR-DC is specified in TS38.133 section 8.2.4.1.

In addition, for a UE which does not support per-FR measurement gap, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For a UE which supports per-FR gaps, interruptions to PCell and activated SCell may be caused by SCells on the same frequency range as the victim cell. Here, if UE can support per-FR MG, interruptions from the same FR may extend the PUCCH SCell activation delay; otherwise, if UE cannot support per-FR MG, all interruption may extend the PUCCH SCell activation delay. So, we propose that,

Proposal 15: The PUCCH SCell activation delay requirement shall apply provided that,

  - The UE has received a PDCCH order to initiate RA procedure on the PUCCH SCell within Tactivate_basic otherwise additional delay to activate the SCell is expected; and

  - No interruption occurs in same FR as the target PUCCH SCell during the SCell activation procedure if UE supports per-FR MG, otherwise the PUCCH SCell activation delay can be extended, and

  - No interruption occurs during the SCell activation procedure if UE does not support per-FR MG, otherwise the PUCCH SCell activation delay can be extended.

The above interruption is caused by factor defined in TS38.133 section 8.2.1.1 for EN-DC, in TS38.133 section 8.2.2.1 for NR SA, in TS38.133 section 8.2.3.1 for NE-DC and in TS38.133 section 8.2.4.1 for NR-DC.

# Conclusion

In this contribution we discuss the preliminary RRM requirements for PUCCH SCell activation/deactivation, including valid TA and invalid TA cases.

Proposal 1: The ending point of PUCCH SCell activation should be the point when UE transmit valid CSI report on target PUCCH SCell, for both valid TA and invalid TA cases.

Proposal 2: The beam information for network to determine the associated SSB for PDCCH triggered RACH occasion shall be considered for both FR1 and FR2 cases.

Proposal 3:

If the target PUCCH Scell is known, no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA, i.e., no additional SSB based beam measurement is needed.

If the target PUCCH Scell is unknown cell in FR2:

- If there is at least one active serving cell on that FR2 band (following the same conditions in TS38.133 section 8.3.2 for intra-band FR2 Scell activation), no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

- If there is no active serving cell on that FR2 band, need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

If the target PUCCH Scell is unknown cell in FR1:

- If it is contiguous to an active serving cell in the same band (following the same conditions in TS38.133 section 8.3.2 for intra-band contiguous FR1 Scell activation), no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

- If there is no contiguous active serving cell on that FR1 band, need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

Proposal 4: beam information of PUCCH SCell is needed to be indicated to NW for both valid and invalid TA cases.

Proposal 5: discuss if RAN4 could have conclusion on how to indicate beam information of PUCCH SCell to network during PUCCH SCell activation, following alternative solutions are considered:

    - Alt 1: using L3 measurement report of PUCCH SCell via SpCell PUSCH

    - Alt 2: Not define the requirement for the unknown PUCCH SCell activation

RAN4 sends LS to RAN1/2 for clarification on this issue only when the solutions could not be concluded in RAN4.

Proposal 6: same solution of beam reporting in issue 1-1-3b shall be applied to L1-RSRP reporting of target PUCCH SCell during unknown PUCCH SCell activation.

Proposal 7: For both valid TA and invalid TA cases in PUCCH SCell activation:

- the UL spatial relation of PUCCH on target being-activated SCell should be considered for PUCCH SCell activation in FR2 only.

  - the time uncertainty of the MAC CE for UL spatial relation activation of PUCCH in target being-activated SCell shall be defined in the baseline FR2 SCell activation delay part (Tactivate_basic). Details are FFS.

Proposal 8: For UEs not supporting beamCorrespondenceWithoutUL-BeamSweeping, FR2 PUCCH SCell (de)activation requirements are not defined.

Proposal 9: Regarding PUCCH Scell activation delay requirement for valid TA case,

- In FR1, reuse the Rel-15 SCell activation delay requirement which is (( THARQ + Tactivation_time +TCSI_Reporting)/ NR slot length).

- In FR2, use normal SCell activation delay (i.e., (( THARQ + Tactivation_time +TCSI_Reporting)/ NR slot length);) in TS38.133 section 8.3.2 as baseline, but the time uncertainty of the MAC CE for UL spatial relation activation of PUCCH in target being-activated SCell shall be considered in the baseline Tactivation_time.

Proposal 10: Regarding the PUCCH SCell activation requirements for invalid TA case,

- If UE does not have the valid TA on the PUCCH SCell being activated, an additional UL synchronization procedure to obtain the valid TA shall be considered which including the following factors:

  - the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell (T1);

  - the delay for obtaining a valid TA command for the sTAG to which the SCell configured with PUCCH belongs (T2);

  - the delay for applying the received TA for uplink transmission (T3)

Proposal 11: In NR PUCCH SCell activation delay requirement with invalid TA, T1 is the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell. T1 is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Proposal 12: In NR PUCCH SCell activation delay requirement with invalid TA, T2 is the delay from slot n + (Tactivate_basic +T1)/NR slot length until UE has obtained a valid TA command for the target PUCCH SCell being activated. Tactivate_basic is the normal SCell activation delay in TS38.133 section 8.3.2. slot n is the slot when UE received PUCCH SCell activation MAC CE.

Proposal 13: In NR PUCCH SCell activation delay requirement with invalid TA, T3 is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213.

Proposal 14: regarding interruption requirements for PUCCH Scell activation in invalid TA case:

- The interruption requirement shall include the existing requirement for Scell activation in Rel-15.

- Introduce additional interruption by PRACH transmission when target PUCCH SCell RACH has different SCS from spCell data/control channel and UE does not support diffNumerologyAcrossPUCCH-Group.

Proposal 15: The PUCCH SCell activation delay requirement shall apply provided that,

  - The UE has received a PDCCH order to initiate RA procedure on the PUCCH SCell within Tactivate_basic otherwise additional delay to activate the SCell is expected; and

  - No interruption occurs in same FR as the target PUCCH SCell during the SCell activation procedure if UE supports per-FR MG, otherwise the PUCCH SCell activation delay can be extended, and

  - No interruption occurs during the SCell activation procedure if UE does not support per-FR MG, otherwise the PUCCH SCell activation delay can be extended.

The above interruption is caused by factor defined in TS38.133 section 8.2.1.1 for EN-DC, in TS38.133 section 8.2.2.1 for NR SA, in TS38.133 section 8.2.3.1 for NE-DC and in TS38.133 section 8.2.4.1 for NR-DC.

# References

[1] R4-2105788, WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements, CATT, RAN4 #98bis-e