3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109348

# Electronic Meeting, May 19-27, 2021

Agenda item:	5.1.7.4.1

Source:	Apple Inc.

Title:	Discussion on applicability of HST-DPS test cases

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

The single carrier requirements for HST-DPS for single carrier are defined in TS38.101-4. In this contribution we discuss the applicability of requirements with HST-DPS.

# 2. Discussion

In TS 38.101-4, requirements for HST-DPS with 1 and 2 active TCI states. The testcase with 2 active TCI states is applicable to UEs that support more than 1 active TCI state, i.e., indicate capability of maxNumberActiveTCI-PerBWP other than n1. Currently the resources sets for CSI-RS for tracking from adjacent RRH are transmitted on the same slots. In TS 38.306, we have separate UE capability for the following associated with CSI-RS for tracking:

To support the current configuration of simultaneously tracking 2 TRS resource sets in the same slot, the UE should also indicate capability for maxSimultaneousResourceSetsPerCC >1. This would be necessary if UE needs to be tested for 2 TCI states.

Observation #1: To support the current configuration of simultaneously tracking 2 TRS resource sets in the same slot, the UE should also indicate capability for maxSimultaneousResourceSetsPerCC >1.

To make the test applicability complete, we could introduce additional applicability rule for HST-DPS tests with 2 TCI states.

Another solution would be to change the configuration such that the TRS resource sets are not in the same slots. Instead of both configured in slots [1,2] for FDD case, we could configure them in slot [1,2] and [5,6] to maintain the same relative offset between the 2 TRS resource sets.

To complete the applicability of HST-DPS test requirements, one of the following options need to be considered:

- Option 1: HST-DPS test cases with 2 active TCI states are only applicable to UE that support maxSimultaneousResourceSetsPerCC >1.

- Option 2: Update the TRS resource set configurations to have different slot offsets.

Observation #2: To complete test applicability for HST-DPS, one of the updates are required:

Option 1: HST-DPS test cases with 2 active TCI states are only applicable to UE that support maxSimultaneousResourceSetsPerCC >1.

Option 2: TRS resource set configurations are updated to have different slot offsets.

To make the tests applicable to more UEs without additional capability, option 2 would be a better option. Hence, we propose to update the TRS resource set configurations to have different slot offsets. To have same relative delay between 2 sets, we propose to use slot offsets [1,2] for set 1 and [5,6] for set 2 for FDD and  [1,2] for set 1 and [11,12] for set 2 for TDD.

Proposal #1: Update the TRS resource set configurations to have different slot offsets – FDD: [1,2] and [5,6]; TDD: [1,2] and [11,12].

Our proposals are captured in our companion CR [3].

# 3. Conclusion

In this paper, we provide our views on the applicability of HST-DPS tests and provide a solution to complete the applicability definition. Our observations and proposals are captured below:

Observation #1: To support the current configuration of simultaneously tracking 2 TRS resource sets in the same slot, the UE should also indicate capability for maxSimultaneousResourceSetsPerCC >1.

Observation #2: To complete test applicability for HST-DPS, one of the updates are required:

Option 1: HST-DPS test cases with 2 active TCI states are only applicable to UE that support maxSimultaneousResourceSetsPerCC >1.

Option 2: TRS resource set configurations are updated to have different slot offsets.

Proposal #1: Update the TRS resource set configurations to have different slot offsets – [1,2] and [5,6].

# Reference

- TS 38.101-4 v16.4.0.

- TS 38.306 v16.16.4.0

- R4-2109349, “CR to 38.101-4 on Test parameters for HST-DPS - R16”, Apple.