3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109353

# Electronic Meeting, May 19-27, 2021

Agenda item:	6.1.7.2

Source:	Apple Inc.

Title:		Simulation results for PDSCH demod requirements in NR-U

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#98bis-e UE demodulation requirements for NR-U were discussed and way forward [1] was agreed. In this contribution we present simulation results for the agreed simulation assumptions.

# 2. Simulation Results

The simulation parameters agreed in [1] are captured below.

Table 1: Simulation Parameters for PDSCH Demod requirements

| Parameter |  | Value |
| --- | --- | --- |
| Active DL BWP index |  | 1 |
| Slot Pattern |  | 7D1S2U |
| LBT failure probability |  | 0.25 |
| Common serving cell parameters | Physical Cell ID | 0 |
|  | SSB position in burst | The first SSB |
|  | SSB Q factor | 8 |
| CSI-RS for Tracking | Periodicity | 40 slots |
|  | Offset | [20, 21] slots |
|  | BW | Full BW |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | According to DL Transmission Model |
|  | PDSCH aggregation factor | 1 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
|  | Resource allocation type | Type 0 |
|  | RBG size | Config2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DMRS Additional Position | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |
| RB Allocation |  | Full BW |
| MIMO Layers |  | 2 |
| Number of HARQ Processes |  | 8 |
| TX EVM |  | 6% |

Table 2: Parameters for DL transmission model

| DL Transmission Model | COT Duration | ms | 4.5 |
| --- | --- | --- | --- |
|  | Idle Time after COT | ms | 0.5 |
|  | DL Transmission Model Period/ Fixed Frame Period (Note 1) | ms | 5 |
|  | Probability of LBT Failure pLBT |  | 0.25 |
|  | UL scheduling for HARQ feedback within FFP |  | Starts at 4ms (8th slot) and lasts for 0.5 ms (1 slot). |
|  | Possible values of the DL Transmission duration within the COT | Slots | {2,4,6,7} |
|  | Possible values of the overall duration of the last slot in the DL Transmission duration | Symbols | If DL Transmission duration is equal to 2 Slots = {14}; If DL If DL Transmission duration is larger than 2 Slots = {6,9,12,14}; |
| Notes: The Fixed Frame Period denomination applies only for ChannelAccessType-r16 = ‘semistatic’. For ChannelAccessType-r16 = ‘dynamic’ this parameter is identified only as DL Transmission Model Period. |  |  |  |

Table 3: Results with 2RX antenna

| Test num. | CBW (MHz) / SCS (kHz) | MCS | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% Max TP (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1-1 | 20/30 | 13 | TDLA30-10 | 2x2, ULA Low | 12.3 | 14.3 |
| 1-2 | 40/30 | 13 | TDLA30-10 | 2x2, ULA Low | 12.5 | 14.5 |
| 1-3 | 60/30 | 13 | TDLA30-10 | 2x2, ULA Low | 12.5 | 14.5 |
| 1-4 | 80/30 | 13 | TDLA30-10 | 2x2, ULA Low | 12.9 | 14.9 |

Table 4: Results with 4RX antenna

| Test num. | CBW (MHz) / SCS (kHz) | MCS | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% Max TP (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1-1 | 20/30 | 13 | TDLA30-10 | 2x4, ULA Low | 6.8 | 8.8 |
| 1-2 | 40/30 | 13 | TDLA30-10 | 2x4, ULA Low | 7.0 | 9.0 |
| 1-3 | 60/30 | 13 | TDLA30-10 | 2x4, ULA Low | 7.1 | 9.1 |
| 1-4 | 80/30 | 13 | TDLA30-10 | 2x4, ULA Low | 7.3 | 9.3 |

# 3. Conclusion

In this paper we present simulation results for PDSCH demodulation requirements in NR-U.

# Reference

- R4-2106173, “Way Forward on NR-U UE demodulation requirements”, Qualcomm.