3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109357

# Electronic Meeting, May 19-27, 2021

Agenda item:	9.7.3.2

Source:	Apple Inc.

Title:	Simulation results for HST CA scenarios

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#98bis-e UE demodulation for FR1 HST was discussed and way forward [1] was agreed.

In this contribution we present simulation results for PDSCH in CA for HST.

# 2. Simulation Results

## Simulation results for HST-SFN JT

For the simulation parameters agreed in [1], we present for different CBW for FDD and TDD duplex modes.

Table 1: Simulation Results for FDD 2x2 for HST-SFN JT

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1-1 | 5 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 | 11.3 | 13.6 |
| 1-2 | 10 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 | 11.5 | 13.8 |
| 1-3 | 15 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 | 11.9 | 14.1 |
| 1-4 | 20 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 | 12.2 | 14.5 |
| 1-5 | 25 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 | 12.2 | 14.5 |
| 1-6 | 30 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 | 12.5 | 14.8 |
| 1-7 | 40 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 | 12.5 | 14.8 |
| 1-8 | 50 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 | 12.6 | 14.8 |

Table 2: Simulation Results for TDD 2x2 for HST-SFN JT

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | TDD UL-DL pattern | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Alignment | Impairment |
| 1-1 | 5 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x2 | 11.4 | 13.6 |
| 1-2 | 10 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x2 | 11.7 | 13.9 |
| 1-3 | 15 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x2 | 11.9 | 14.2 |
| 1-4 | 20 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x2 | 11.9 | 14.1 |
| 1-5 | 25 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x2 | 12.1 | 14.4 |
| 1-6 | 30 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x2 | 12.2 | 14.5 |
| 1-7 | 40 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x2 | 12.4 | 14.6 |
| 1-8 | 50 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x2 | 12.5 | 14.7 |
| 1-9 | 60 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x2 | 12.6 | 14.8 |
| 1-10 | 80 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x2 | 12.6 | 14.8 |
| 1-11 | 90 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x2 | 12.7 | 14.9 |
| 1-12 | 100 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x2 | 12.5 | 14.8 |

Table 3: Simulation Results for FDD 2x4 for HST-SFN JT

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1-1 | 5 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 | 8.7 | 11.0 |
| 1-2 | 10 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 | 8.8 | 11.1 |
| 1-3 | 15 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 | 9.2 | 11.5 |
| 1-4 | 20 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 | 9.5 | 11.7 |
| 1-5 | 25 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 | 9.6 | 11.9 |
| 1-6 | 30 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 | 9.9 | 12.1 |
| 1-7 | 40 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 | 9.8 | 12.1 |
| 1-8 | 50 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 | 9.9 | 12.1 |

Table 4: Simulation Results for TDD 2x4 for HST-SFN JT

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | TDD UL-DL pattern | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Alignment | Impairment |
| 1-1 | 5 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x4 | 8.9 | 11.2 |
| 1-2 | 10 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x4 | 9.3 | 11.5 |
| 1-3 | 15 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x4 | 9.5 | 11.8 |
| 1-4 | 20 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x4 | 9.4 | 11.7 |
| 1-5 | 25 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x4 | 9.7 | 11.9 |
| 1-6 | 30 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x4 | 9.8 | 12.0 |
| 1-7 | 40 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x4 | 9.9 | 12.2 |
| 1-8 | 50 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x4 | 10.0 | 12.2 |
| 1-9 | 60 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x4 | 10.1 | 12.3 |
| 1-10 | 80 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x4 | 10.1 | 12.3 |
| 1-11 | 90 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x4 | 10.2 | 12.4 |
| 1-12 | 100 / 30 | 16QAM, 0.48 | HST-SFN | FR1.30-1 | 2x4 | 10.0 | 12.3 |

## Simulation results for HST-DPS

For the simulation parameters agreed in [1], we present for different CBW for FDD and TDD duplex modes.

Table 5: Simulation Results for FDD 2x2 for HST-DPS

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1-1 | 5 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-2 | 10 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-3 | 15 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-4 | 20 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-5 | 25 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-6 | 30 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-7 | 40 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-8 | 50 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |

Table 6: Simulation Results for TDD 2x2 for HST-DPS

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | TDD UL-DL pattern | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Alignment | Impairment |
| 1-1 | 5 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x2 | 11.0 | 13.5 |
| 1-2 | 10 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x2 | 11.0 | 13.5 |
| 1-3 | 15 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x2 | 11.0 | 13.5 |
| 1-4 | 20 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x2 | 11.0 | 13.5 |
| 1-5 | 25 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x2 | 11.0 | 13.5 |
| 1-6 | 30 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x2 | 11.0 | 13.5 |
| 1-7 | 40 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x2 | 11.0 | 13.5 |
| 1-8 | 50 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x2 | 11.0 | 13.5 |
| 1-9 | 60 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x2 | 11.0 | 13.5 |
| 1-10 | 80 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x2 | 11.0 | 13.5 |
| 1-11 | 90 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x2 | 11.0 | 13.5 |
| 1-12 | 100 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x2 | 11.0 | 13.5 |

Table 7: Simulation Results for FDD 2x4 for HST-DPS

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1-1 | 5 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 7.8 | 10.3 |
| 1-2 | 10 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.3 | 10.8 |
| 1-3 | 15 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.1 | 10.6 |
| 1-4 | 20 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 7.7 | 10.2 |
| 1-5 | 25 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.1 | 10.6 |
| 1-6 | 30 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.4 | 10.9 |
| 1-7 | 40 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.1 | 10.6 |
| 1-8 | 50 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.4 | 10.9 |

Table 8: Simulation Results for TDD 2x4 for HST-DPS

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | TDD UL-DL pattern | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Alignment | Impairment |
| 1-1 | 5 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x4 | 7.9 | 10.4 |
| 1-2 | 10 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x4 | 8.0 | 10.5 |
| 1-3 | 15 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x4 | 7.8 | 10.3 |
| 1-4 | 20 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x4 | 7.7 | 10.2 |
| 1-5 | 25 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x4 | 7.8 | 10.3 |
| 1-6 | 30 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x4 | 7.8 | 10.3 |
| 1-7 | 40 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x4 | 7.6 | 10.1 |
| 1-8 | 50 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x4 | 8.0 | 10.5 |
| 1-9 | 60 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x4 | 8.1 | 10.6 |
| 1-10 | 80 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x4 | 7.6 | 10.1 |
| 1-11 | 90 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x4 | 7.8 | 10.3 |
| 1-12 | 100 / 30 | 64QAM, 0.43 | HST-DPS | FR1.30-1 | 2x4 | 7.9 | 10.4 |

# 3. Conclusion

In this paper, we provide simulation results for PDSCH requirements in CA for HST scenarios.

# Reference

- R4-2016098, “Way Forward on UE demodulation for FR1 HST”, CMCC.