3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109390

Electronic Meeting, 19 – 27 May 2021

Agenda Item:	11.1.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 37.880: Coexistence studies for High-power UE Vs Public Safety operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71

Document for:	Approval

# 1.	Introduction

The study item on High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71 was approved at TSG RAN#88-e [1]. The purpose of this study item is to study RF requirements that are applicable for high power UE operation in LTE band 12 and band 5, and in NR band n71 for fixed wireless and vehicle-mounted use cases, in ITU Region 2.

One of the objectives of this study item is to carry out coexistence study to evaluate the throughput OOBE impact on a victim band from a high-power aggressor in Band 12, Band 5, and Band n71. In particular, the impact from a high power UE in Band 5 on the public safety downlink in 851-861 MHz and the public safety uplink in 806-816 MHz are listed as tasks. This contribution provides some discussion on the coexistence studies for this scenario and a text proposal for approval to record the discussion into TR 37.880 [2].

# 2.	Discussion

Satisfactory coexistence between two communication systems in the same geographical area normally depends on two main criteria:

1) The interfering transmitter out of band emission (OOBE) received by the affected receiver is sufficiently below this affected receiver noise floor.

2) The total interfering carrier power attenuated by the affected receiver radio frequency (RF), intermediate frequency (IF), and base band (BB) filters is sufficiently below the affected receiver noise floor.

System simulation is required for coexistence studies on the interference cases involving UE. This is because the UE positions are not fixed, and they are not expected to transmit with full power most of the time due to power control. Hence using deterministic analysis could result in findings that are too conservative, which in turn lead to unnecessary tight coexistence requirements. System simulation need to be performed based on agreed methodology and parameters, so that results from different companies can be compared directly to draw the conclusions from the studies.

The public safety narrowband (PSNB) system parameters used for Band 26 and PSNB coexistence studies in TR 37.806 [3] are shown in table 1, and some simulation results can be found in clause 7.2 of TR 37.806. These parameters can be considered together with the assumptions provided in clause 5.1 of this TR for the system simulation between HPUE and public safety operation.

Table 1: PSNB system parameters

|  | Base Station | Portable |
| --- | --- | --- |
| Carrier frequency | 850 MHz |  |
| Channel bandwidth | 6.25 kHz |  |
| Cell radius | 12 km |  |
| Antenna height | 100 m from ground | 1.5m |
| Lognormal fading | 10 dB |  |
| Antenna gain and antenna pattern | 11 dBi omni-directional | Antenna gain + body loss =-6 dBi |
| Noise figure | 5.7 dB | 9.75 dB |
| Transmit power | 45 dBm (after combiner loss) | 36 dBm |
| SNR Threshold | 16.5 dB | 16.5 dB |
| Effective Noise Bandwidth (ENBW) | 6.25 kHz | 6.25 kHz |
| Noise Floor | -130.3dBm / 6.25 kHz | -126.3dBm / 6.25 kHz |
| Sensitivity | -113.8dBm / 6.25 kHz | -109.8dBm / 6.25 kHz |

Note that the target OOBE from a Band 5 HPUE to the public safety downlink and uplink frequency ranges (for criterion 1), as well as the achievable attenuation on the Band 5 HPUE carrier power at the public safety downlink and uplink receiver filters (for criterion 2), need to be agreed on to evaluate the impact from a Band 5 HPUE on the public safety downlink in 851-861 MHz and the public safety uplink in 806-816 MHz. The simulation methodology and parameters for the system simulation should be further discussed and agreed during the work item (WI) phase.

# 3.	Conclusion

This contribution has some discussion on the coexistence studies for the UL HPUE Vs public safety operation scenario. It is discussed why system simulation is required for coexistence studies on the interference cases involving UE. The simulation methodology and parameters for the system simulation should be further discussed and agreed during the work item (WI) phase.

# 4.	Text proposal

<Start of text proposal>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-201261: "New SID on high-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71", U.S. Cellular.

[3]	3GPP TR 36.837: "Public safety broadband high power User Equipment (UE)".

[4]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[5]	3GPP R4-2008924: " LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (below 5 GHz)", Ericsson.

[6]	3GPP TR 36.802: "Evolved Universal Terrestrial Radio Access (E-UTRA); NB-IOT; Technical Report for BS and UE radio transmission and reception".

[7]	3GPP TR 36.942: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios".

[8]	3GPP TS 36.101: " Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[9]	3GPP R4-78AH-0150, “Summary of in-band/guard-band interference modeling from various companies”, Intel, Huawei, Ericsson, ZTE, Nokia Networks.

[10]	3GPP R4-163556, “Simulation results of coexistence studies between NB-IoT and LTE, guard-band case”, Intel Corporation.

[11]	R4-124522, Architecture of Power Class 1 UE in Band 14, RAN4#64, EADS / Cassidian

[12]	R4-1702437, MPR/A-MPR requirement for PC1 HPUE

[13]	3GPP R4-163555, “Simulation results of coexistence studies between NB-IoT and LTE, in-band case”, Intel Corporation.

[14]	3GPP TR 37.806: “Extending 850 MHz study Technical Report”.

<Next change>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3GPP	3rd Generation Partnership Project

BB	Base Band

BS	Base Station

BW	Bandwidth

CL	Coupling Loss

DL	Downlink

HPUE	High Power User Equipment

IF	Intermediate Frequency

ITU	International Telecommunication Union

LTE	Long Term Evolution

MTC	Machine-Type Communications

NB	Narrowband

NR	New Radio

OOBE	Out Of Band Emissions

PA	Power Amplifier

PC	Power Class

PRB	Physical Resource Block

PSNB	Public Safety Narrowband

RF	Radio Frequency

Rx	Receiver

SW	Software

TR	Technical Report

Tx	Transmitter

UE	User Equipment

<Next change>

## 5.3	Coexistence with Public Safety

Satisfactory coexistence between two communication systems in the same geographical area normally depends on two main criteria:

1) The interfering transmitter out of band emission (OOBE) received by the affected receiver is sufficiently below this affected receiver noise floor.

2) The total interfering carrier power attenuated by the affected receiver radio frequency (RF), intermediate frequency (IF), and base band (BB) filters is sufficiently below the affected receiver noise floor.

System simulation is required for coexistence studies on the interference cases involving UE. This is because the UE positions are not fixed, and they are not expected to transmit with full power most of the time due to power control. Hence using deterministic analysis could result in findings that are too conservative, which in turn lead to unnecessary tight coexistence requirements. System simulation need to be performed based on agreed methodology and parameters, so that results from different companies can be compared directly to draw the conclusions from the studies.

The public safety narrowband (PSNB) system parameters used for Band 26 and PSNB coexistence studies in TR 37.806 [14] are shown in table 5.2.4-1, and some simulation results can be found in clause 7.2 of TR 37.806. These parameters can be considered together with the assumptions provided in clause 5.1 of this TR for the system simulation between HPUE and public safety operation.

Table 5.2.4-1: PSNB system parameters

|  | Base Station | Portable |
| --- | --- | --- |
| Carrier frequency | 850 MHz |  |
| Channel bandwidth | 6.25 kHz |  |
| Cell radius | 12 km |  |
| Antenna height | 100 m from ground | 1.5m |
| Lognormal fading | 10 dB |  |
| Antenna gain and antenna pattern | 11 dBi omni-directional | Antenna gain + body loss =-6 dBi |
| Noise figure | 5.7 dB | 9.75 dB |
| Transmit power | 45 dBm (after combiner loss) | 36 dBm |
| SNR Threshold | 16.5 dB | 16.5 dB |
| Effective Noise Bandwidth (ENBW) | 6.25 kHz | 6.25 kHz |
| Noise Floor | -130.3dBm / 6.25 kHz | -126.3dBm / 6.25 kHz |
| Sensitivity | -113.8dBm / 6.25 kHz | -109.8dBm / 6.25 kHz |

Note that the target OOBE from a Band 5 HPUE to the public safety downlink and uplink frequency ranges (for criterion 1), as well as the achievable attenuation on the Band 5 HPUE carrier power at the public safety downlink and uplink receiver filters (for criterion 2), need to be agreed on to evaluate the impact from a Band 5 HPUE on the public safety downlink in 851-861 MHz and the public safety uplink in 806-816 MHz. The simulation methodology and parameters for the system simulation should be further discussed and agreed during the work item (WI) phase.

<End of text proposal>

# References

[1]	RP-201261, “New SID on high-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71”, U.S. Cellular.

[2]	R4-2111007, “TR 37.880 V1.1.1: High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71”, Nokia, Nokia Shanghai Bell.

[3]	3GPP TR 37.806: “Extending 850 MHz study Technical Report”.