3GPP TSG-RAN WG4 Meeting#99             	R4-2109406

E-meeting, 19th – 27th May, 2021

Title:	TP to TR 38.717-03-02 Addition of CA_n2A-n14A-n77A

Source:	AT&T, Nokia

Agenda item:	8.13.2

Document for:	Approval

# 1	Introduction

This contribution is a TP to TR 38.717-03-02 to introduce CA_n2A-n14A-n77A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

### 5.1.x	CA_n2-n14-n77

#### 5.1.x.1		Operating bands for CA

Table 5.1.x.1-1: CA band combination

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n2-n14-n77 | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 5.1.x.2		Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n14A-n77A | CA_n2A-n14A, CA_n2A-n77A, CA_n14A-n77A | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

#### 5.1.x.3	Co-existence studies

For 3DL/2UL NR CA, only the IMD issues due to dual uplink operation of two bands falling into the DL of the third band shall be verified.

Co-existence studies for dual uplink operation of two bands, i.e. CA_n2A-n14A, CA_n2A-n77A and CA_n14A-n77A have been captured in TR 38.716-02-00 and TR 38.717-02-01 where:

-	IMD3 and IMD5 products are produced by Band n2 and n14 that might fall in Rx of band n77.

-	There are no IMD products produced by Band n2 and n77 that might fall in Rx of band n14.

-	IMD3 products are produced by Band n14 and n77 might fall in Rx of band n2.

#### 5.1.x.4	REFSENS requirements

The required MSD values are based on already completed CA_n5A-n25A-n77A for band n2 and n77 MSD.

Table 5.1.x.4-1: MSD for the CA configuration

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n14A-n77A | n2 | 1880 | 5 | 25 | 1960 | 16.5 | FDD | IMD3 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3466 | 10 | 50 | 3466 | 16.0 | TDD | IMD3 |

*********************** End of the TP ***************************************************

# 3	References