3GPP TSG-RAN WG4 Meeting #99e	R4-2109417

Electronic Meeting, 19th - 27th May, 2021

Source:	ZTE Corporation

Title:	On draft reply LS on the intra-band and inter-band (NG)EN-DC or NE-DC Capabilities

Agenda Item:	13.2

Document for:	Discussion

# Introduction

RAN2 sent an LS R2-2104550 on the intra-band and inter-band (NG)EN-DC/NE-DC UE capabilities with the following two questions for RAN4 and RAN1 [1]:

Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable? Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /ul-dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part?

In this contribution, we provide our understanding and answers to the two questions and also the draft reply LS as seen in Annex.

# Discussion

Below figure illustrates all of 5 types of (NG)EN-DC/NE-DC combinations:

Fig. 1, Five types of (NG)EN-DC/NE-DC combinations

And three IEs related to RAN4 in the two questions are shown in the below table:

| R4: 2-16 | PA architectures for intra-band EN-DC | dualPA-Architecture For an intra-band band combination, this field indicates the support of dual PAs. If absent in an intra-band band combination, the UE supports single PA for all the ULs in the intra-band band combination. For other band combinations, this field is not applicable. |
| --- | --- | --- |
| R4:2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | simultaneousRxTxInterBandENDC Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band (NG)EN-DC/NE-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-3 [4]. |
| R4:2-6 | Asynchronous FDD-FDD intra-band EN-DC DC | asyncIntraBandENDC Indicates whether the UE supports asynchronous FDD-FDD intra-band (NG)EN-DC with MRTD and MTTD as specified in clause 7.5 and 7.6 of TS 38.133 [5]. If asynchronous FDD-FDD intra-band (NG)EN-DC is not supported, the UE supports only synchronous FDD-FDD intra-band (NG)EN-DC. |

The dualPA-Architecture capable UE in RAN4 will have some RF requirements different from that without the capability, e.g., MPR/A-MPR for intra-band non-contiguous CA [2] and EN-DC [3] by assuming there are two simultaneous uplinks within the band, therefore, it is applicable to Type 1, Type 2 and Type 5. For Type 3, since there is only one UL transmission at a time in the band, either one NR uplink, or one LTE uplink, dualPA-Architecture is not applicable obviously.

Observation 1: dualPA-Architecture is appliable to Type 1, Type 2 and Type 5.

In RAN4 specs, different DC configurations or some different RF requirements are specified for a simultaneousRxTxInterBandENDC capble UE for some specific DC inter-band combinations, and it is quite clear that the IE is applicable for Type 2, Type 3 and Type 4.

Observation 2: simultaneousRxTxInterBandENDC is appliable to Type 2, Type 3 and Type 4.

In order to differentiate sync and async operation for an FDD-FDD intra-band EN-DC/NE-DC, minimum requirements are defined in RAN4 for the relative transmission or receiving timing difference between subframe timing boundary of involved E-UTRA and NR cells. And this capability is indicated by asyncIntraBandENDC, so the IE is applicable to Type 1, Type 2, Type 3 and Type 5.

Observation 3: asyncIntraBandENDC is appliable to Type 1, Type 2, Type 3 and Type 5.

The table below shows an overview on the applicabilities:

|  | Type 1 | Type 2 | Type 3 | Type 4 | Type 5 |
| --- | --- | --- | --- | --- | --- |
| dualPA-Architecture | √ | √ |  |  | √ |
| simultaneousRxTxInterBandENDC |  | √ | √ | √ |  |
| asyncIntraBandENDC | √ | √ | √ |  | √ |

With the above understanding, we propose to reply RAN2 with the draft appended.

Proposal 1: Send the reply LS shown in Annex.

# Conclusion

In this paper, we have the following observations and proposal for replying RAN2 LS on intra-band and inter-band (NG)EN-DC/NE-DC capabilities:

Observation 1: dualPA-Architecture is appliable to Type 1, Type 2 and Type 5.

Observation 2: simultaneousRxTxInterBandENDC is appliable to Type 2, Type 3 and Type 4.

Observation 3: asyncIntraBandENDC is appliable to Type 1, Type 2, Type 3 and Type 5.

Proposal 1: Send the reply LS shown in Annex.

# Annex

Title:	Draft Reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN4

To:	RAN2

Cc: 	RAN1

Contact Person:

Name:	Aijun CAO

E-mail Address:	cao.aijun@zte.com.cn

# 1. Overall Description

RAN4 kindly thanks RAN2 for the LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC capabilities, and RAN4 reaches a common understanding on the three IEs related to RAN4 involved in the questions raised by RAN2. The detailed answers to the RAN2 questions are shown below:

Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable?

Answer: From RAN4 perspective, dualPA-Architecture is applicable to (NG)EN-DC/NE-DC BC Type 1, Type 2 and Type 5, and simultaneousRxTxInterBandENDC is applicable to (NG)EN-DC/NE-DC BC Type 2, Type 3 and Type 4, and asyncIntraBandENDC is applicable to (NG)EN-DC/NE-DC BC Type 1, Type 2, Type 3 and Type 5.

Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part?

Answer: Yes, dualPA-Architecture/ asyncIntraBandENDC are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part.

# 2. Actions:

ACTION: 	RAN4 kindly asks RAN2 to consider answers of above for RAN2 future work.

3. Date of Next TSG-RAN4 Meetings:

3GPP  RAN4#100-e 	                16 – 27 Aug 2021			Electronic Meeting

3GPP  RAN4#101-e			    1 – 12 Nov 2021             Electronic Meeting

# Reference

R2-2104550, LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities, RAN2/ZTE

3GPP TS 38.101-1 v16.6.0

3GPP TS 38.101-3 v16.6.0