3GPP TSG-RAN WG4 Meeting #99-e		R4-2109610

Electronic meeting, 19th May – 27th May, 2021

Source:	Ericsson

Title:	Simulation results for the PUSCH demodulation performance requirements for 256QAM

Agenda item:	9.11.3.1

Document for:	discussion

# Introduction

In RAN4#98bis-e meeting, a WF for FR1 PUSCH 256QAM performance requirements was agreed [1]. In this contribution, simulation results for the PUSCH demodulation performance requirements for 256QAM will be shown. The detailed simulation assumptions are listed in table 1.

Table 1	Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Bandwidth |  | 15kHz SCS: 5, 10 MHz 30kHz SCS: 10, 40 MHz |
| CP |  | Normal |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | {0, 2, 3, 1} |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | 1-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | PUSCH to DM-RS EPRE Ratio | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| TD-RA | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| FD-RA | RB assignment | Full applicable test bandwidth |
|  | FH | Disabled |
| CBG-based PUSCH transmission |  | Disabled |
| MCS |  | MCS#22, 24 |
| LBRM |  | Disabled |
| SRS |  | Not configured |
| Number of Tx antennas |  | 1 |
| Number of Rx antennas |  | 2, 4, 8 |
| Channel model |  | TDLA30-10 Low |
| Timing offset, Frequency offset |  | 0, 0 |
| Test metric |  | 70% of max. throughput |

# Simulation results

Simulation results are summarized in table 2. Detailed performance curves can be found in the appendix.

Table 2	Simulation results

|  |  | 2Rx (1Tx) |  | 4Rx (1Tx) |  | 8Rx (1Tx) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MCS#22 | MCS#24 | MCS#22 | MCS#24 | MCS#22 | MCS#24 |
| 15kHz | 5MHz | 18.4 | 20.5 | 14.8 | 16.8 | 11.4 | 13.5 |
|  | 10MHz | 18.6 | 20.7 | 14.9 | 16.9 | 11.6 | 13.5 |
| 30kHz | 10MHz | 18.8 | 20.5 | 15 | 16.8 | 11.7 | 13.4 |
|  | 40MHz | 19.1 | 21 | 15.1 | 17 | 11.8 | 13.7 |

From the simulation results, it can be observed that MCS#24 and MCS#22 are both feasible for 256QAM PUSCH demodulation performance definition.

# References

[1]	R4-2106120	Way forward for FR1 PUSCH 256QAM performance requirements	RAN4#98bis-e

# Appendix