3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109635

Electronic Meeting, May. 19-27, 2021

Agenda Item: 9.7.2.1.3

Source: MediaTek Inc.

Title: Discussion on Rel-17 HST in FR1 for inter-frequency measurement

Document for: Discussion

- Introduction

In last meeting, it was agreed that inter frequency measurement would be enhanced with UE capability for R17 HST. Our views on how to enhance the inter frequency measurement are provided in this paper.

- Discussion

For the inter-frequency measurement, the discussions on enhancement for HST in CONNECTED mode and IDLE mode are provided in section 2.1 and 2.2, respectively.

2.1 inter-frequency measurement in CONNECTED mode

In current specification, the inter-frequency measurement in connected mode is divided into with and without measurement gaps (MGs) and our views on these two parts are provided in section 2.1.1 and 2.1.2.

2.1.1 Inter-frequency measurement with MGs

For convenience of reference, the requirement for inter-frequency measurement with measurement gaps in TS 38.133 is provided as follows.

The measurement period for inter-frequency measurements with gaps in clause 9.3.4 of TS 38.133 Table 9.3.4-1: Time period for PSS/SSS detection (Frequency range FR1) Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(8*1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 8 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group.. Table 9.3.4-3: Time period for time index detection (Frequency range FR1) Condition NOTE1,2; TSSB_time_index_inter. No DRX; Max(120ms, 3 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(120ms, Ceil(3 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 3 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group.. Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (Frequency FR1) Condition NOTE1,2; T SSB_measurement_period_inter. No DRX; Max(200ms, 8 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(200ms, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 8 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group..

In our understanding, the inter-RAT measurement (from LTE to NR; defined in clause 8.1.2.4.21 of TS 36.133) is similar to inter-frequency measurement because both two are the measurement on the different frequency point in NR. Thus, we propose to follow the similar logic as the R16 HST inter-RAT measurement (from LTE to NR) to enhance inter-frequency enhancement for R17 HST, i.e., the scaling factor of 1.5 and the sample number 8 can be replaced by the scaling factor of M2 and 4M2, respectively.

Proposal 1: In CONNECTED mode, for the inter-frequency measurements with MGs for Rel-17 HST,  introduce the scaling factor M2, by following the similar logic as the R16 HST inter-RAT measurement (from LTE to NR) specified in clause 8.1.2.4.21 of TS 36.133.

Thus, according to Proposal 1, our views on the TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for R17 HST inter-frequency measurements with MGs are provided as follows:

Proposal 2: In CONNECTED mode, for the inter-frequency measurements with MGs in Rel-17 HST, the TPSS/SSS_sync_inter should be:

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8* M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

Proposal 3: In CONNECTED mode, for the inter-frequency measurements with MGs in Rel-17 HST, the TSSB_time_index_inter should be:

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, 3 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(3 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 3 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

Proposal 4: In CONNECTED mode, for the inter-frequency measurements with MGs in Rel-17 HST, the TSSB_measurement_period_inter should be:

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 4 M2 Note 3 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

2.1.2 Inter-frequency measurement without MGs

For convenience of reference, the inter-frequency measurement without measurement gaps in TS 38.133 is provided as follows. (Note that there are typos in inter-frequency measurement without measurement gaps in clause 9.3.9 of TS 38.133 and the corresponding correction please refer to our CR: R4-2109883)

The measurement period for inter-frequency measurements without gaps in clause 9.3.9 of TS 38.133 Table 9.3.9-1: Time period for PSS/SSS detection, (FR1) DRX cycle; TPSS/SSS_sync_inter. No DRX; max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFinter. DRX cycle≤ 320ms; max( 600ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter. DRX cycle>320ms; ceil(5 x Kp) x DRX cycle x CSSFinter. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 9.3.9-3: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_inter. No DRX; max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFinter. DRX cycle≤ 320ms; max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter. DRX cycle>320ms; Ceil(3 x Kp) x DRX cycle x CSSFinter. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 9.3.9-1: Measurement period for inter-frequency measurements without gaps (FR1) DRX cycle; T SSB_measurement_period_inter. No DRX; max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFinter. DRX cycle≤ 320ms; max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter. DRX cycle>320ms; ceil( 5 x Kp ) x DRX cycle x CSSFinter. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

It is obvious that the requirement for inter-frequency measurement without MGs is similar to intra-frequency measurement. Because the SSB(s) from inter frequency neighbour cell are located in the serving cell activated BWP so that the inter-frequency measurement can be completed with good enough performance within that measurement period. Thus, in our understanding, we can take R16 HST intra-frequency measurement as reference to define the R17 HST inter-frequency measurement without MGs.

Proposal 5: In CONNECTED mode, for the inter-frequency measurements without MGs for Rel-17 HST, introduce the scaling factor M2 and Y, by following the same logic as the R16 HST intra-frequency measurement.

Thus, according to Proposal 5, our views on the TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for R17 HST inter-frequency measurements without MGs are provided as follows:

Proposal 6: In CONNECTED mode, for the inter-frequency measurements without MGs in Rel-17 HST, the TPSS/SSS_sync_inter should be:

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2 x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

Proposal 7: In CONNECTED mode, for the inter-frequency measurements without MGs in Rel-17 HST, the TSSB_time_index_inter should be:

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

Proposal 8: In CONNECTED mode, for the inter-frequency measurements without MGs in Rel-17 HST, the TSSB_measurement_period_inter should be:

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 160ms | max(200ms, ceil(M2 Note 2 x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| 160ms < DRX cycle≤ 320ms | ceil(M2 Note 2 x 4 x Kp) x max(SMTC period,DRX cycle))x CSSFinter |
| DRX cycle>320ms | ceil(Y Note 3 x Kp ) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms |  |

2.2 inter-frequency measurement in IDLE mode

In last meeting, RAN4 agreed that the inter-frequency measurement enhancement for R17 HST is up to UE capability.

Agreement in RAN4 #98b-e - Inter-frequency measurement - Define RRC Connected state inter-frequency measurement enhancements - Support of HST inter-frequency measurement enhancements is up to UE capability. Details are FFS - FFS whether enhancements for RRC IDLE inter-frequency measurements are needed

For IDLE inter-frequency measurements forR17 HST, similar to inter-frequency measurement in CONNECTED mode, we think it also can be up to UE capability because the network can deploy single frequency point along railway so that UE no needs to perform the inter-frequency measurement in IDLE mode.

Proposal 9: In IDLE mode, the support of inter-frequency measurements is up to UE capability.

As inter-frequency measurements with MGs in CONNECTED mode, we can take the RRC_IDLE inter-RAT measurement (from LTE to NR; defined in clause 4.2.2.5.6 of TS 36.133) as reference to enhance RRC_IDLE mode inter-frequency measurement for R17 HST.

Proposal 10: In IDLE mode, the inter-frequency measurements for Rel-17 HST may follow the similar logic as the R16 HST inter-RAT measurement (from LTE to NR) defined in clause 4.2.2.5.6 of TS 36.133, i.e., introduce the scaling factor M2, M3 and M4, to define the requirement.

The requirements of the RRC_IDLE inter-RAT measurement (from LTE to NR; defined in clause 4.2.2.5.6 of TS 36.133) for non-HST and R16 HST are provided as follows for reference.

The measurement period for RRC_IDLE inter-RAT measurements (from LTE to NR; defined in clause 4.2.2.5.7 of TS 36.133) Table 4.2.2.5.6-1: Tdetect,NR, TmeasureNR, and Tevaluate,NR DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR [s] (number of DRX cycles); Tmeasure,NR [s] (number of DRX cycles); Tevaluate,NR [s] (number of DRX cycles). FR1; FR2Note1. 0.32; 1; 8; 11.52 x 1.5 x N1 (36 x 1.5 x N1); 1.28 x 1.5 x N1 (4 x 1.5 x N1); 5.12 x 1.5 x N1 (16 x 1.5 x N1). 0.64; 5; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). NOTE 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length.. Table 4.2.2.5.6-2: Tdetect,NR_HST, TmeasureNR_HST, and Tevaluate,NR_HST for UE configured with highSpeedInterRAT-NR-r16 DRX cycle length [s]; Tdetect,NR_HST [s] (number of DRX cycles); Tmeasure,NR_HST [s] (number of DRX cycles); Tevaluate,NR_HST [s] (number of DRX cycles). 0.32; 4.16 x M2 (13 x M2)Note 2; 0.64 x M3 (2 x M3)Note 2; 0.96 x M4 (3 x M4) Note 2. 0.64; 7.68 (12)); 1.28 (2); 1.92 (3). 1.28; 12.8(10); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). Note 1: FR2 high speed requirements are not specified. Note 2: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=1..

Besides, for convenience of reference, the RRC_IDLE inter-frequency measurement in TS 38.133 is provided as follows.

The measurement period for RRC_IDLE inter-frequency measurements in clause 4.2.2.4 of TS 38.133 Table 4.2.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). FR1; FR2Note1. 0.32; 1; 8; 11.52 x N1 x 1.5 (36 x N1 x 1.5); 1.28 x N1 x 1.5 (4 x N1 x 1.5); 5.12 x N1 x 1.5 (16 x N1 x 1.5). 0.64; 5; 17.92x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length..

According to Proposal 10, our views on the Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for R17 HST RRC_IDLE inter-frequency measurements are provided as follows:

Proposal 11: In IDLE mode, for the inter-frequency measurements in Rel-17 HST, the Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter should be:

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 x M2 (13 x M2)Note 1 | 0.64 x M3 (2 x M3)Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | 7.68 (12)) | 1.28 (2) | 1.92 (3) |
| 1.28 | 12.8(10) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=M2=M3=1. |  |  |  |

- Summary

In this paper, the discussion of R17 HST in FR1 is provided. We have the following proposal:

Proposal 1: In CONNECTED mode, the inter-frequency measurements with MGs for Rel-17 HST may follow the similar logic as the R16 HST inter-RAT measurement (from LTE to NR) defined in clause 8.1.2.4.21 of TS 36.133, i.e., introduce the scaling factor M2, to define the requirement.

Proposal 2: In CONNECTED mode, for the inter-frequency measurements with MGs in Rel-17 HST, the TPSS/SSS_sync_inter should be:

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8* M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

Proposal 3: In CONNECTED mode, for the inter-frequency measurements with MGs in Rel-17 HST, the TSSB_time_index_inter should be:

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, 3 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(3 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 3 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

Proposal 4: In CONNECTED mode, for the inter-frequency measurements with MGs in Rel-17 HST, the TSSB_measurement_period_inter should be:

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 4 M2 Note 3 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

Proposal 5: In CONNECTED mode, the inter-frequency measurements without MGs for Rel-17 HST may follow the same logic as the R16 HST intra-frequency measurement, i.e., introduce the scaling factor M2 and Y, to define the requirement.

Proposal 6: In CONNECTED mode, for the inter-frequency measurements without MGs in Rel-17 HST, the TPSS/SSS_sync_inter should be:

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2 x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

Proposal 7: In CONNECTED mode, for the inter-frequency measurements without MGs in Rel-17 HST, the TSSB_time_index_inter should be:

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

Proposal 8: In CONNECTED mode, for the inter-frequency measurements without MGs in Rel-17 HST, the TSSB_measurement_period_inter should be:

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 160ms | max(200ms, ceil(M2 Note 2 x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| 160ms < DRX cycle≤ 320ms | ceil(M2 Note 2 x 4 x Kp) x max(SMTC period,DRX cycle))x CSSFinter |
| DRX cycle>320ms | ceil(Y Note 3 x Kp ) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms |  |

Proposal 9: In IDLE mode, the inter-frequency measurements should be up to UE capability.

Proposal 10: In IDLE mode, the inter-frequency measurements for Rel-17 HST may follow the similar logic as the R16 HST inter-RAT measurement (from LTE to NR) defined in clause 4.2.2.5.6 of TS 36.133, i.e., introduce the scaling factor M2, M3 and M4, to define the requirement.

Proposal 11: In IDLE mode, for the inter-frequency measurements in Rel-17 HST, the Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter should be:

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 x M2 (13 x M2)Note 1 | 0.64 x M3 (2 x M3)Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | 7.68 (12)) | 1.28 (2) | 1.92 (3) |
| 1.28 | 12.8(10) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=M2=M3=1. |  |  |  |