3GPP TSG-RAN WG4 Meeting #99-e	R4-2109666  
Electronic Meeting, May 19-27, 2021

Agenda item:	10.1.1

Source:	vivo

Title:	TP to TR38.884 v0.3.0 on Environment conditions

Document for:	Approval

1	Introduction

The scope the FR2 test methods SI includes the development of test methods for both NTC and ETC test cases [1-4], the environment condition and applicability should be added in the TR to reflect the agreements.

This contribution provides the text proposals on Environment conditions to TR 38.884.

2	Discussion

The ETC test method has been defined in the TR 38.884, however, the applicability of each RF requirement should follow the decision in TS 38.101-2. This information is added in the Temperature conditions part.

3	References

- RP-210633, Revised SID: Study on enhanced test methods for FR2, Apple Inc., vivo, 3GPP TSG-RAN Meeting #91, March 2021

- R4-2017596, WF on extreme temperature conditions for all applicable FR2 UE RF test cases, vivo, RAN4#97e

- R4-2103920, “WF on ETC and test time reduction”, vivo, RAN4#98e

- R4-2106127, WF on agreements and remaining issues with FR2 test method enhancements, Apple, RAN4#98bis-e, April 2021

4	Text Proposal to TR 38.884

--------------Start of text proposal -------------

Annex A:
Environment conditions

A.1	Operating voltage

All nominal voltage test cases shall be performed with the DUT operated in stand-alone battery powered mode except for DUTs whose power is provided only from AC mains. For extreme voltage test cases, the UE shall fulfil all the requirements in the full voltage range, i.e. the voltage range between the extreme voltages defined in Table A.1-1. When those test cases require an external power source with cables connected to the DUT that could adversely affect the device performance, it shall be demonstrated that there is negligible impact to performance with the device operated in stand-alone battery powered mode compared to the device operated in external power mode first before performing the extreme voltage test cases.

Table A.1-1: Voltage conditions

| Power source | Lower extreme voltage | Higher extreme voltage | Normal conditions voltage |
| --- | --- | --- | --- |
| AC mains | 0,9 * nominal | 1,1 * nominal | nominal |
| Regulated lead acid battery | 0,9 * nominal | 1,3 * nominal | 1,1 * nominal |
| Nonregulated batteries: Leclanché Lithium Mercury/nickel & cadmium | 0,85 * nominal 0,95 * nominal 0,90 * nominal | Nominal 1,1 * Nominal | Nominal 1,1 * Nominal Nominal |

A.2	Temperature

Table A.2-1: Temperature conditions

| + 25 ⁰C ± 10 ⁰C | For normal (room temperature) conditions with relative humidity of 25 % to 75 % |
| --- | --- |
| -10C to +55C | For extreme conditions |

Whether the RF test cases should be performed with normal condition or extreme conditions should follow the RF requirement applicability defined in [TS38.101-2].

--------------End of text proposal -------------