3GPP TSG-RAN WG4 Meeting # 99-e 													R4-2109687

Electronic Meeting, May 19th - 27th, 2021

Title:	[Draft] Reply LS LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities

Response to:	(R2-2104550) LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name: Sanjun Feng

E-mail Address: fengsanjun@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN2 for the LS Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities. RAN4 would like to provide following feedbackfor the two questions.

Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable?

[RAN4 answer]: The applicable BC types for the three respective capabilities are as following:

- dualPA-Architecture: Type 1, Type 2, Type 5

- simultaneousRxTxInterBandENDC: Type 3, Type 4

- asyncIntraBandENDC: Type 1, Type 2, Type 5

- ul-TimingAlignmentEUTRA-NR：Type 1, Type 2, Type 5

- asyncIntraBandENDC：Type 1, Type 2. Only for E-UTRA FDD-NR FDD intra-band EN-DC

Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /ul-dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part?

[RAN4 answer]:

For dualPA-Architecture and asyncIntraBandENDC, for BC types 1,2,5, they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part.

For simultaneousRxTxInterBandENDC, for BC types 3,4, it is used to indicated the restriction to the inter-band (NG)EN-DC/NE-DC BC part.

For ul-TimingAlignmentEUTRA-NR for BC types 1,2,5, they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part.

For asyncIntraBandENDC for BC type 1 and 2, it is used to indicate the restriciton to the E-UTRA FDD-NR FDD intra-band EN-DC part

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above feedback into account.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #100-e	23 - 27 Aug 2021         E-meeting

TSG-RAN WG4 Meeting #101-e	1 - 12 Aug 2021         E-meeting