3GPP TSG-RAN WG4 Meeting #99-e	R4-2109705

# Electronic Meeting, 19th to 27th May, 2021

Agenda Item:	6.7.2

Source: 	MediaTek Inc.

Title: 	DC_n46A-n48A MSD due to receiver harmonic mixing analysis

Document for:	Approval

- 1. Introduction

The same frequency range combination has been specified in TS38.101-1 and TS38.101-3. However it was not analysed for harmonic mixing of 3xn48 DL with 2xn46 UL. The contribution provides the analysis. Corresponding CRs in [3][4][5][6]

- 2. Discussion

REFSENS

The REFSENS calculation assumptions are listed in table 2.1-1 and REFSENS analysis is in table. The uplink power level of n46 is using PC5 which is 20dBm at antenna port.

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n46 filter min rejection at 2f0 | 18 | dB |
| n46 2f0 level relative to f0 | -40 | dB |
| Front-end loss | 4 | dB |
| n48 filter min rejection on 3f0 | 23 | dB |
| Receiver 3f0 harmonic rejection | 9.5 | dB |
| Thermal noise at n48 RX ANT port | -166 | dBm/Hz |
| PA wide band nosie | -130 | dBm/Hz |
| Transceiver effective phase noise due to wider BW | -150 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 2.1-1 Typical receiver performance parameters for MSD analysis

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| n5 TX power at antenna port (Primary path) | 20 | 20 | dBm |
| n46 PA output noise power at RX freq | -130 | -130 | dBm/Hz |
| n46 TX noise power at PRX LNA port | -94 | NA | dBm/25MHz |
| n46 TX noise power at DRX LNA port | NA | -94 | dBm/25MHz |
| n48 receiver reciprocal mixing noise at LNA port | -104 | -104 | dBm/25MHz |
| Thermal noise at RX ant port | -166 |  | dBm/Hz |
| n46 TX 2f0 fall in n48 ANT port | -44 | -44 | dBm |
| n46 TX 2f0 fall in n48 RX LNA port | -67 | -67 | dBm |
| Total noise level refer to n48 receiver LNA input port | -76.4 | -76.4 | dBm |
| Combined |  |  |  |
| n48 REFSENS (5MHz BW) | -76.4 |  | dBm |

Table 2.1-2 Link analysis for n48 SCS=15 KHz, 5 MHz REFSENS calculation

The requirements shall be specified in the TS38.101-1 as below:

Proposal 1: Reference sensitivity exceptions due to harmonic mixing for CA_n46A-n48A in NR FR1 shall be specified as below:

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz(dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz(dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n46 | n481 | 22.6 | 19.5 | 17.8 | 16.6 |  |  | 14 | 13.1 | 12.6 | 12 | 12 | 12 | 12 |
| NOTE 1: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.1-3: Reference sensitivity exceptions due to harmonic mixing for CA

And associated RB allocations should be as defined in 2.1-4.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n46 | 15 | 12 | 25 | 36 | 50 |  |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | FDD |

Table 2.1-4: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

Proposal 2: Reference sensitivity exceptions due to harmonic mixing for DC_48_n46 in NR FR1 in TS38.101-3 shall also be specified accordingly

Table 7.3B.2.3.1-3: Reference sensitivity QPSK PREFSENS (EN-DC with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n46 | 482,x | 22.6 | 19.5 | 17.8 | 16.6 |  |  |  |  |  |  |  |
| NOTE 2: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the downlink transmission bandwidth of a victim (lower) band. NOTE x: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.1-5: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC paring with n46

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n46 | 48 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |

Reference:

- RP-202277, Revised Basket WID on adding channel bandwidth support to existing NR bands, Ericsson

- R4-2103185 Way forward on the introduction of 25 MHz in band n5, RAN4#98-e

- R4-2109721 CR to 38.101-1 for missing MSD due to receiver harmonic mixing for combos with n46-R16, RAN4#99-e

- R4-2109722 CR to 38.101-1 for missing MSD due to receiver harmonic mixing for combos with n46-R17, RAN4#99-e

- R4-2109723 CR to 38.101-3 for missing MSD for combos with n46-R16, RAN4#99-e

- R4-2109724 CR to 38.101-3 for missing MSD for combos with n46-R17, RAN4#99-e