3GPP TSG-RAN WG4 Meeting #99-e	R4-2109733

# Electronic Meeting, May. 19-27, 2021

Source: 	Qualcomm CDMA Technologies

Title: 	Views on the scope and potential RRM impacts of feMIMO WI

Agenda item:	9.18.2	General and work plan for RRM core requirements

Document for:	Discussion

# Introduction

RAN4 99-e will be the first meeting that RAN4 starts working on the RAN1-led WI of feMIMO. As RRM room will focus on the discussion of the work plan, it would be helpful to share some observations on the work scope.

In this paper, we share our views on the major topics of feMIMO and preliminary assessment on the RRM impacts.

# Discussions

Per WID[1] approved in RP86, several objectives are considered for the WI for enhancement such as overhead/latency reduction, enabling multi-panels, further improvement for mTRP, enhancing SRS and typeII CSI-RS etc.

In the following tables, we offer a brief summary RAN1 issues under discussion for each objective and further comments on what we think may be subject to RRM impacts.

- Enhancement on multi-beam operation

| Issues | RAN1 work* | RAN4 RRM impact |
| --- | --- | --- |
| Unified TCI framework | Introducing three new types of common beam TCI states, i.e. joint DL/UL and separate DL/UL TCI states which indicate common beam for multiple DL and/or UL channels. | Update the RRM core requirements on the application time during the TCI state switch, UL spatial relationship and PL-RS etc. Further identify the existing RAN4 requirements that involve the TCI state swith e.g. SCell activation |
| Beam management overhead and latency reduction | DCI based (unified)TCI state switch for beam indication; Letency reduction for TCI state and PL-RS switch |  |
| L1/L2 based inter-cell mobility and inter-cell mTRP | Include additional resources of the non-serving cell to be measured and reported in the L1 BM flow | RRM requirements of the beam measurement delay and reporting procedures for the non-serving cell reference signals shall be introduced. |
| Multi-panel UE | UL beam selection for UEs equipped with multi-panels | Depending on the RAN1 progress in defining the use cases, requirements of the new procedures such as the panel activation/selection may be introduced. |
| MPE mitigation | P-MPR reporting for UL beam/panel selection | Procedure requirements may be possible to define. |
| * Based on RAN1 104-b-e |  |  |

- Enhancement on the support for multi-TRP deployment

| Issues | RAN1 work | RAN4 RRM impact |
| --- | --- | --- |
| PDCCH, PUSCH, and PUCCH reliability and robustness | PDCCH two TCI states; PUCCH TDMed beams/spatial relationships; PUSCH repetition with different TDMed beams; | Minimal RRM impact |
| Inter-cell mTRP | Include TRPs of different cells assuming multi-DCI based reception | Certain overlapped impact in the aspect of supporting the non-serving cell’s beam measurement for the L1/L2 centric inter-cell moblity |
| Beam management for mTRP | Per TRP BFR Beam measurement and report for per TRP resources | RRM requirements to be extended for per TRP based BFR and simultaneous cell BFR and TRP BFR etc. |
| mTRP CSI | CSI reporting for mTRP for NCJT | Minimal RRM impact |
| Enhancement to support HST-SFN deployment | Single DMRS port QCLed with two TRS in non transparent SFN transimission Doppler shift pre-compensation schemes for PDSCH BFR and TCI switch for SFN PDCCH | Potential RRM impacts due to switching the pre-compensation schemes; MAC-CE based activation of two TCI states for coreset. |

- Enhancement on SRS and CSI

| Issues | RAN1 work | RAN4 RRM impact |
| --- | --- | --- |
| SRS enhancement | Aperiodic SRS triggering; SRS switch for up to 8 antennas; SRS coverage enhancement via time bundling or partial frequency sounding | Potential RRM impact due to SRS antenna switch in the R17 context for more antennas. |
| CSI enhancement | FDD reciprocity CSI feedback | Minimal RRM impact |

Based on the observations, the major RRM impacts concentrate on the aspects of multi-beam operations which also affect the topic of mTRP enhancement due to introducing inter-cell TRPs and per TRP BFR. Besides, the new schemes of the SRS antenna switch in the SRS enhancement may have RRM implications.

For the multi-beam operations, support of MPUE and MPE mitigation are still in the on-going discussions in RAN1. So RAN4 may prioritize the discussions on the requirements due to the universal TCI framework, BM overhead/latency reduction and L1/L2 based inter-cell mobility in the beginning phase of the work item.

We thus propose,

Proposal1: RAN4 to prioritize identifying the RRM impacts of the enhancements in the multi-beam operations especially related to the universal TCI state framework, BM overhead/latency reduction and L1/L2 based inter-cell mobility.

Proposal1.1: Discussions on MPUE and MPE mitigation may start later which depends on the RAN1 progress.

Further, as the R17 work item of further enhancement in RRM is defining the requirements for SRS antenna switch by considering the R15 schemes, RAN4 may discuss if R17 related SRS antenna switch schemes shall also be discussed in the same forum.

Proposal2: RAN4 to discuss whether the SRS switch for up to 8 antennas shall be handled in the work item of RRM further enhancement.

Similar situation could happen in HST-SFN related discussion, where RAN1 is still discussing the design to realize the SFN PDSCH and SFN PDCCH. As such, there are potential RRM impacts. For example, there could be scenarios to enable/switch the Dopper shift pre-compensation schemes and perform the MAC-CE activation of two TCI states per coreset. RAN4 may first of all discuss if relevant topics can be handled in the feMIMO WI or HST WI.

Proposal3: Depending on the RAN1 progress, RAN4 to discuss if HST-SFN related RRM topics would be discussed in the feMIMO WI or HST WI.

# Conclusion

In this paper, we shared a brief summary of RAN1 feMIMO work and potential RRM impacts. We further propose,

Proposal1: RAN4 to prioritize identifying the RRM impacts of the enhancements in the multi-beam operations especially universal TCI state framework, BM overhead/latency reduction and L1/L2 based inter-cell mobility.

Proposal1.1: Discussions on MPUE and MPE mitigation may start later which depends on the RAN1 progress.

Proposal2: RAN4 to discuss whether the SRS switch for up to 8 antennas shall be handled in the work item of RRM further enhancement.

Proposal3: Depending on the RAN1 progress, RAN4 to discuss if HST-SFN related RRM topics would be discussed in the feMIMO WI or HST WI.

# References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung