3GPP TSG-RAN WG4 Meeting#99      	R4-2109740

E-meeting, 19th – 27th  May, 2021

Title:	Simulation assumptions for pi/2 BPSK with spectrum shaping

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	10.6.1

Document for:	Approval

# 1	Introduction

This contribution relates to a new study item agreed in RAN#91-e, namely “optimizations of pi/2 BPSK uplink power in NR” [1]. In this paper we propose simulation assumptions for studying the BS receiver performance for pi/2 BPSK with spectrum shaping. These evaluation assumptions have been used for link level evaluations in our companion contributions [2].

# 2	Discussion

For pi/2 BPSK link level studies, we propose to take NR Coverage SI as the starting point. This will ensure a solid baseline for the simulations. The simulation assumptions for FR1 are captured in Annex A.1 of [3]. Table 1 shows the proposed parameters selected based on those.

The main difference between Table 1 and Annex A.1 of [3] is that NR coverage study considers only one data rate for the selected scenario (such as 1Mbps for UL in Urban scenario), and only one PRB allocation (such as 30 PRBs). In here, we propose to study the coding rate and bandwidth aspects in more details to see e.g. how channel estimation works in different scenarios. For that reason, we propose to consider 5 different coding rates and 6 different PRB allocations, respectively.

In order to see the impact of shaping filter, companies should report the shaping filter used in the simulations.

Proposal 1: Take TR 38.830, Annex A.1 as the starting point for defining link level assumptions for link level simulations

Proposal 2: Select parameters shown in Table 1 for pi/2 BPSK -related link simulations.

Proposal 3: Companies should report the shaping filter used in the simulations.

Table 1 Simulation parameters

| Carrier frequency | 4GHz |
| --- | --- |
| Channel BW | 100MHz |
| SCS | 30kHz |
| Channel model | TDL-C 300ns |
| UE speed | 3km/h |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 4 |
| DMRS config | Low PAPR sequence type 2, 2 symbols |
| Waveform | DFT-S-OFDM |
| HARQ config | No retransmissions |
| Number of PRBs | 2, 4, 8, 16, 32, 64 |
| Coding rates | 1/8, 1/4, 1/3, 1/2, 2/3 |
| Channel | PUSCH, 14 OFDM symbols |
| Frequency hopping | No |
| BLER | 10% |

# 3	Conclusion

In this paper we propose simulation assumptions for studying the BS receiver performance for pi/2 BPSK with spectrum shaping. It covers the following proposals:

Proposal 1: Take TR 38.830, Annex A.1 as the starting point for defining link level assumptions for link level simulations

Proposal 2: Select parameters shown in Table 1 for pi/2 BPSK -related link simulations.

Proposal 3: Companies should report the shaping filter used in the simulations.

# 4 	References

- RP-210910, “Optimizations of pi/2 BPSK uplink power in NR”, Moderator (Huawei)

- R4-2109741, “Receiver performance for pi/2 BPSK with spectral shaping”, Nokia

- TR 38.830, “Study on NR coverage enhancements”, 3GPP