3GPP TSG-RAN WG4 Meeting #99-e              	R4-2109797

Electrical Meeting, 19th May-27th May, 2021

Agenda item:	6.1.7.4.4

Source:	Samsung

Title:	Simulation results of PRACH for Rel-16 NR-U

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, it was agreed to define the wideband PRACH performance requirement for Rel-16 NR-U WI.  The test parameters for related requirement were captured in WF [1].

In this contribution, the simulation results of PRACH are provided for performance requirement derived.

# 2	Simulation Results

In this section, based on the agreed test parameters, both ideal and impairment results are provided.

2.1 Simulation Assumption

Table 1:  Simulation assumption for PRACH

| Parameter | Value |
| --- | --- |
| PRACH formats | A2, B4,C2 |
| Antenna configurations | 1T2R |
| Propagation conditions | AWGN |
| Frequency offset | 0 Hz for AWGN 400Hz for TDLA30-10 |
| Ncs | 164 for LRA=1151 (15KHz SCS) 190 for LRA = 571 (30 kHz SCS) |
| Test metric | false alarm probability ≤ 0.1% Probability of detection ≥ 99% |

2.2 Simulation Results

Table 2:  ideal and impairment results of PRACH

| Format | Tx/Rx | SCS | Ncs | L_RA | Channel | Frequency offset | SNR (Ideal results ) | SNR (impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A2 | 1T2R | 15 KHz | 164 | 1151 | AWGN | 0Hz | -23.46 | -21.46 |
| A2 | 1T2R | 30 KHz | 190 | 571 | AWGN | 0Hz | -20.36 | -18.36 |
| A2 | 1T2R | 15 KHz | 164 | 1151 | TDLA30-10 | 400Hz | -17.26 | -15.26 |
| A2 | 1T2R | 30 KHz | 190 | 571 | TDLA30-10 | 400Hz | -14.22 | -12.22 |
| B4 | 1T2R | 15 KHz | 164 | 1151 | AWGN | 0Hz | -27.81 | -25.81 |
| B4 | 1T2R | 30 KHz | 190 | 571 | AWGN | 0Hz | -24.63 | -22.63 |
| B4 | 1T2R | 15 KHz | 164 | 1151 | TDLA30-10 | 400Hz | -20.63 | -18.63 |
| B4 | 1T2R | 30 KHz | 190 | 571 | TDLA30-10 | 400Hz | -18.16 | -16.16 |
| C2 | 1T2R | 15 KHz | 164 | 1151 | AWGN | 0Hz | -23.48 | -21.48 |
| C2 | 1T2R | 30 KHz | 190 | 571 | AWGN | 0Hz | -20.37 | -18.37 |
| C2 | 1T2R | 15 KHz | 164 | 1151 | TDLA30-10 | 400Hz | -17.27 | -15.27 |
| C2 | 1T2R | 30 KHz | 190 | 571 | TDLA30-10 | 400Hz | -14.22 | -12.22 |

# 3	Conclusion

In this contribution, the ideal and impairment results for PRACH are provided for performance requirement derived.

# References

R4-2103808, WF on the NR-U PRACH demodulation requirement, Nokia, Nokia Shanghai Bell.