3GPP TSG-RAN WG4 Meeting #99-e                                                              R4-2109853

# Electronic Meeting, May. 19-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR of test cases on measurement accuracy under CCA for inter-frequency SS-RSRP and L1-RSRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2021-5-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of test cases on measurement accuracy under CCA for the following cases: For inter-frequency absolute and relative accuracies of SS-RSRP (Change #1, #3, #5): - Tests are proivded in A.9.4.1.2, A.10.5.1.2, and A.11.6.1.2. For L1-RSRP (Change #2, #4, #6): - On A.9.4.4, A.10.5.4.1, A.11.6.4.1, it is based on the endorsed CRs from RAN4#98-Bis-e: R4-2105726 Introduction of test cases for L1-RSRP measurement accuracy with CCA serving cell - On A.10.5.4.2, A.11.6.4.2, tests are provided to verify the measurement accuracy on SCells. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of test cases for inter-frequency absolute and relative accuracies for SS-RSRP and for L1-RSRP measurement accuracy with CCA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for inter-frequency SS-RSRP measurement accuracy and L1-RSRP measurement accuracy are not covering cells operating with CCA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.9.4.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell under CCA

A.9.4.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.28.1.1 and 10.1.28.1.2 for inter-frequency measurements with the testing configurations for NR cells in Table A.9.4.1.2.1-1.

Table A.9.4.1.2.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

A.9.4.1.2.2	Test parameters

In this set of test cases there are three cells in the test, PCell (Cell 1), and two cells on different carrier frequency with CCA and transmit SSBs in DBT windows according to DL CCA model: SCell (Cell 2) and a neighbour cell (Cell 3).

Two sub-tests (Test 1 and Test 2) are provided with different Noc. The test parameters for the Cell 1, Cell 2, and Cell 3 are given in Table A.9.4.1.2.2-1 below. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.9.4.1.2.2-1. The inter-frequency measurements are supported by a measurement gap.

Table A.9.4.1.2.2-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit |  | Test 1 |  |  |  |  |  |  |  | Test 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  | Cell 1 |  |  |  |  |  |  | Cell 2 |  |  |  |  |  |  |  |  |  |  |  | Cell 3 |  |  |
| SSB ARFCN |  | 1~3 |  | freq1 |  |  |  | freq2 |  |  |  | freq3 |  | freq1 |  |  |  |  |  |  | freq2 |  |  |  |  |  |  |  |  |  |  |  | freq3 |  |  |
| DL CCA model |  | 1~3 |  | N/A |  |  |  | As specifieed in A.3.20.2.1 |  |  |  |  |  | N/A |  |  |  |  |  |  | As specifieed in A.3.20.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UL CCA model |  | 1~3 |  | N/A |  |  |  | As specified in A.3.20.2.2 |  |  |  |  |  | N/A |  |  |  |  |  |  | As specified in A.3.20.2.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  | 10: NRB,c = 52 |  |  |  |  |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Duplex mode |  | 1 |  | FDD |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  | FDD |  |  |  |  |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | 1 |  | N/A |  |  | TDDConf.1.1 CCA |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | TDDConf.1.1 CCA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD |  |  | SR.1.1 CCA |  |  | - |  |  |  |  |  |  | SR.1.1 FDD |  |  |  |  |  |  | SR.1.1 CCA |  |  |  |  | - |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  | SR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  | SR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD |  |  | CR.1.1 CCA |  |  | - |  |  |  |  |  |  |  |  | CR.1.1 FDD |  |  |  |  |  |  | CR.1.1 CCA |  |  |  |  |  |  |  |  | - |  |  |  |  |  |
|  |  | 2 |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  | CR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD |  |  | CCR.1.1 CCA |  |  | - |  |  |  |  |  |  |  |  | CCR.1.1 FDD |  |  |  |  |  |  | CCR.1.1 CCA |  |  |  |  |  |  |  |  | - |  |  |  |  |  |
|  |  | 2 |  | CCR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CCR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  | CCR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CCR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration for Semi-static channel access |  | 1 |  | SSB.3 FR1 |  |  | SSB.3 CCA |  |  | - |  |  |  |  |  |  |  | SSB.3 FR1 |  |  |  |  |  |  | SSB.3 CCA |  |  |  |  |  |  |  |  | - |  |  |  |  |  |
|  |  | 2 |  | SSB.3 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  | SSB.3 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  | SSB.4 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  | SSB.4 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration for Dynamic channel access |  | 1 |  | SSB.3 FR1 |  | SSB.4 CCA |  | - |  |  | SSB.3 FR1 | SSB.4 CCA | - |
|  |  | 2 |  | SSB.3 FR1 |  |  |  |  |  |  | SSB.3 FR1 |  |  |
|  |  | 3 |  | SSB.4 FR1 |  |  |  |  |  |  | SSB.4 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  |  | OP.1 |  |  |  |  |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD |  |  | TRS.1.2 TDD |  |  |  | - |  |  |  |  |  |  | TRS.1.1 FDD |  |  |  |  |  |  | TRS.1.2 TDD |  |  |  |  |  |  | - |  |  |  |  |  |  |  |
|  |  | 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Initial BWP Configuration |  | 1~3 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dedicated BWP configuration |  | 1~3 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | 1~3 | s | - |  |  | - |  | 3 |  |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  | - |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |
| SMTC configuration |  | 1~3 |  | SMTC.1 |  |  | N/A |  | N/A |  |  |  |  |  |  |  |  |  | SMTC.1 | N/A |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |
| DBT Window Configuration |  | 1~3 |  | N/A |  |  | DBT.1 |  | DBT.1 |  |  |  |  |  |  |  |  |  | N/A |  | DBT.1 |  |  |  |  |  |  |  |  | DBT.1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 |  |  |  |  |  |  |  |  |  | ( for Channel 3 +8dB) |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |  | ( for Channel 3 +8dB) |  |  |  |  |  |  | [-111] |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SSB SCS | -94.65 |  | -91.65 |  |  |  |  |  |  |  | ( for Channel 3 +8dB) |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | ( for Channel 3 +8dB) |  |  |  |  |  |  |  |  |  |  |  | [-108] |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -91.65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ( for Channel 3 +8dB) |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | ( for Channel 3 +8dB) |  |  |  |  |  |  | [-108] |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1~3 | dB | 10 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  | 14 |  | 14 |  |  |  |  |  |  |  |  |  | -3 |  |  |  |  |  |  |  |  |  |
| SS-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1~3 | dBm/SCS | -81.65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (RSRP for Cell 3 +25dB) |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | (RSRP for Cell 3 +25dB)- |  |  |  |  |  |  |  |  |  |  |  | [-111] |  |  |  |  |  |  |  |  |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1~3 | dBm/ 38.16MHz | -50.19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (Io for Channel 3 +19.75dB) |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR_TDD_FR1_I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | (Io for Channel 3 +19.75dB) |  |  |  |  |  |  |  |  |  |  |  | [-75.19] |  |  |  |  |  |  |  |  |
|  |  | 1~3 | dB | 10 |  | 10 |  |  |  |  | 14 | 14 | -3 |
| Propagation condition |  | 1~3 | - |  | AWGN |  |  |  |  |  |  |  | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  | 1~3 |  |  | 1x2 |  |  |  |  |  |  |  | 1x2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.9.4.1.2.3	Test Requirements

In both Test 1 and Test 2, the SS-RSRP measurement accuracy for Cell 2 and Cell 3 shall fulfil the absolute requirement in clause 10.1.28.1.1 and relative requirement in clause 10.1.28.1.2.

<End of Change 1>

<Start of Change 2>

A.9.4.4	L1-RSRP measurement for beam reporting with CCA serving cell

A.9.4.4.1	SSB based L1-RSRP measurement

A.9.4.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.33.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.9.4.4.1.1-1.

Table A.9.4.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

A.9.4.4.1.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a SCell under CCA (Cell2). Cell 2 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model.

Two sub-tests (Test 1 and Test 2) are provided with different Noc  on Cell 2. The test parameters for the Cell 1 and Cell 2 are given in Table A.9.4.4.1.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.9.4.4.1.2-1.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. On Cell 2, UE is configured to perform L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.9.4.4.1.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |  |
| Active PCell/SCell Configuration |  | 1~3 |  | PCell | SCell |  | PCell | SCell |  |
| SSB GSCN |  | 1~3 |  | freq1 | freq2 |  | freq1 | freq2 |  |
| DL CCA model |  | 1~3 |  | N/A | As specifieed in A.3.20.2.1 |  | N/A | As specifieed in A.3.20.2.1 |  |
| UL CCA model |  | 1~3 |  | N/A | As specified in A.3.20.2.2 |  | N/A | As specified in A.3.20.2.2 |  |
| Duplex mode |  | 1 |  | FDD | TDD |  | FDD | TDD |  |
|  |  | 2,3 |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A | TDDConf.1.1 CCA |  | N/A | TDDConf.1.1 CCA |  |
|  |  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 40: NRB,c = 106 |  | 10: NRB,c = 52 | 40: NRB,c = 106 |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 CCA |  | SR.1.1 FDD | SR.1.1 CCA |  |
|  |  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 CCA |  | CR.1.1 FDD | CR.1.1 CCA |  |
|  |  | 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 CCA |  | CCR.1.1 FDD | CCR.1.1 CCA |  |
|  |  | 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| SSB configuration for Semi-static channel access |  | 1 |  | SSB.3 FR1 | SSB.3 CCA |  | SSB.3 FR1 | SSB.3 CCA |  |
|  |  | 2 |  | SSB.3 FR1 |  |  | SSB.3 FR1 |  |  |
|  |  | 3 |  | SSB.4 FR1 |  |  | SSB.4 FR1 |  |  |
| SSB configuration for Dynamic channel access |  | 1 |  | SSB.3 FR1 | SSB.4 CCA |  | SSB.3 FR1 | SSB.4 CCA |  |
|  |  | 2 |  | SSB.3 FR1 |  |  | SSB.3 FR1 |  |  |
|  |  | 3 |  | SSB.4 FR1 |  |  | SSB.4 FR1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.2 TDD |  | TRS.1.1 FDD | TRS.1.2 TDD |  |
|  |  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  | OP.1 |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| SMTC configuration |  | 1~3 |  | SMTC.1 |  | N/A | SMTC.1 |  | N/A |
| DBT Window Configuration |  | 1~3 |  | N/A |  | DBT.1 | N/A |  | DBT.1 |
| reportConfigType |  | 1~3 |  | periodic |  |  | periodic |  |  |
| reportQuantity |  | 1~3 |  | ssb-Index-RSRP |  |  | ssb-Index-RSRP |  |  |
| Number of reported RS |  | 1~3 |  | 2 |  |  | 2 |  |  |
| L1-RSRP reporting period |  | 1~3 |  | slot80 |  |  | slot80 |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 |  |  | -94.65 | - |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_I |  |  |  |  |  | - | -113 |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/SSB SCS | -91.65 |  |  | -91.65 | - |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_I |  |  |  |  |  | - | -110 |  |
|  |  | 1~3 | dB | 10 |  |  | 10 | -3 |  |
| SS-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/SCS | -81.65 |  |  | -81.65 | - |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_I |  |  |  |  |  | - | -113 |  |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/ 38.16MHz | -50.19 |  |  | -50.19 | - |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_I |  |  |  |  |  | - | -77.19 |  |
|  |  | 1~3 | dB | 10 |  |  | 10 | -3 |  |
| Propagation condition |  | 1~3 |  | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

A.9.4.4.1.3	Test Requirements

In both Test 1 and Test 2, the L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 2 shall fulfil the requirements in clauses 10.1.33.1.

<End of Change 2>

<Start of Change 3>

A.10.5.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell under CCA

A.10.5.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Clauses 10.1.28.1.1 and 10.1.28.1.2 for inter-frequency measurements with the testing configurations in Table A.10.5.1.2.1-1.

Table A.10.5.1.2.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations on each supported band |  |

A.10.5.1.2.2	Test parameters

In this set of test cases there are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell under CCA (Cell 2) and a FR1 neighbour cell under CCA (Cell 3) on a different frequency than the PSCell. Cell 2 and Cell3 operate on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model.

Two sub-tests (Test 1 and Test 2) are provided with different Noc. The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 2 and Cell 3 are given in Table A.10.5.1.2.2-1 below. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.10.5.1.2.2-1.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

The inter-frequency measurements are supported by a measurement gap.

Table A.10.5.1.2.2-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 |  | Cell 2 |  |  | Cell 3 |
| SSB ARFCN |  | 1,2 |  | freq1 | freq2 |  | freq1 |  |  | freq2 |
| DL CCA model |  | 1,2 |  | As specifieed in A.3.20.2.1 |  |  | As specifieed in A.3.20.2.1 |  |  |  |
| UL CCA model |  | 1,2 |  | As specified in A.3.20.2.2 |  |  | As specified in A.3.20.2.2 |  |  |  |
| BWchannel |  | 1,2 | MHz | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |  |
| Gap pattern ID |  |  |  | 0 |  |  | 0 |  |  |  |
| Duplex mode |  | 1,2 |  | TDD |  |  | TDD |  |  |  |
| TDD configuration |  | 1,2 |  | TDDConf.1.1 CCA |  |  | TDDConf.1.1 CCA |  |  |  |
| PDSCH Reference measurement channel |  | 1,2 |  | SR.1.1 CCA | - |  | SR.1.1 CCA |  |  | - |
| RMSI CORESET Reference Channel |  | 1,2 |  | CR.1.1 CCA | - |  | CR.1.1 CCA |  |  | - |
| Dedicated CORESET Reference Channel |  | 1,2 |  | CCR.1.1 CCA | - |  | CCR.1.1 CCA |  |  | - |
| SSB configuration for Semi-static channel access |  | 1,2 |  | SSB.3 CCA |  |  | SSB.3 CCA |  |  |  |
| SSB configuration for Dynamic channel access |  | 1,2 |  | SSB.4 CCA |  |  | SSB.4 CCA |  |  |  |
| OCNG Patterns |  | 1,2 |  | OP.1 |  |  | OP.1 |  |  |  |
| TRS configuration |  | 1,2 |  | TRS.1.2 TDD |  | - | TRS.1.2 TDD |  | - |  |
| Initial BWP Configuration |  | 1,2 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP configuration |  | 1,2 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| Time offset with Cell 2 |  | 1,2 | s | - | 3 |  | - | 3 |  |  |
| DBT Window Configuration |  | 1,2 |  | DBT.1 |  |  | DBT.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | 1,2 | dB | 0 | 0 |  | 0 |  |  | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_I | 1,2 | dBm/15kHz | -94.65 |  |  | ( for Cell 3 +8dB) |  |  | [-111] |
| Note2 | NR_TDD_FR1_I | 1,2 | dBm/SCS | -91.65 |  |  | ( for C 3 +8dB) |  |  | [-108] |
|  |  | 1,2 | dB | 10 | 10 |  | 14 |  |  | -3 |
| SS-RSRPNote3 | NR_TDD_FR1_I | 1,2 | dBm/SCS | -81.65 |  |  | (RSRP for Cell 3 +25dB) |  |  | [-111] |
| IoNote3 | NR_TDD_FR1_I | 1,2 | dBm/ 38.16MHz | -50.19 |  |  | (Io for Channel 3 +19.75dB) |  |  | [-75.19] |
|  |  | 1,2 | dB | 10 | 10 |  | 14 |  |  | -3 |
| Propagation condition |  | 1,2 | - | AWGN |  |  | AWGN |  |  |  |
| Antenna configuration |  | 1,2 | - | 1x2 |  |  | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5 The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |

A.10.5.1.2.3	Test Requirements

In both Test 1 and Test 2, the SS-RSRP measurement accuracy for Cell 2 and Cell 3 shall fulfil the Absolute requirement in clause 10.1.28.1.1 and Relative requirement in clause 10.1.28.1.2.

<End of Change 3>

<Start of Change 4>

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell

A.10.5.4.1	SSB based L1-RSRP measurement

A.10.5.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.33.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.10.5.4.1.1-1.

Table A.10.5.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

A.10.5.4.1.2	Test parameters

In this set of test cases there are two cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell under CCA (Cell 2). Cell 2 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model.

Two sub-tests (Test 1 and Test 2) are provided with different Noc  on Cell 2. The test parameters and applicability for Cell 1 are defined in A.3.7A.2. The test parameters for the Cell 2 are given in Table A.10.5.4.1.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.10.5.4.1.2-1.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.10.5.4.1.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1,2 |  | freq1 | freq1 |
| DL CCA model |  | 1,2 |  | As specifieed in A.3.20.2.1 | As specifieed in A.3.20.2.1 |
| UL CCA model |  | 1,2 |  | As specified in A.3.20.2.2 | As specified in A.3.20.2.2 |
| Duplex mode |  | 1,2 |  | TDD | TDD |
| TDD Configuration |  | 1,2 |  | TDDConf.1.1 CCA | TDDConf.1.1 CCA |
| BWchannel |  | 1,2 | MHz | 40: NRB,c = 106 | 40: NRB,c = 106 |
| Duplex mode |  | 1,2 |  | TDD | TDD |
| TDD configuration |  | 1,2 |  | TDDConf.1.1 CCA | TDDConf.1.1 CCA |
| PDSCH Reference measurement channel |  | 1,2 |  | SR.1.1 CCA | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  | 1,2 |  | CR.1.1 CCA | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  | 1,2 |  | CCR.1.1 CCA | CCR.1.1 CCA |
| SSB configuration for Semi-static channel access |  | 1,2 |  | SSB.3 CCA | SSB.3 CCA |
| SSB configuration for Dynamic channel access |  | 1,2 |  | SSB.4 CCA | SSB.4 CCA |
| OCNG Patterns |  | 1,2 |  | OP.1 | OP.1 |
| TRS configuration |  | 1,2 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1,2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1,2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration |  | 1,2 |  | DBT.1 | DBT.1 |
| reportConfigType |  | 1,2 |  | periodic | periodic |
| reportQuantity |  | 1,2 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS |  | 1,2 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1,2 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1,2 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  | 1,2 |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_TDD_FR1_I | 1,2 | dBm/15kHz | -94.65 | [-113] |
| Note2 | NR_TDD_FR1_I | 1,2 | dBm/SCS | -91.65 | [-110] |
|  |  | 1,2 | dB | 10 | -3 |
| SS-RSRPNote3 | NR_TDD_FR1_I | 1,2 | dBm/SCS | -81.65 | [-113] |
| IoNote3 | NR_TDD_FR1_I | 1,2 | dBm/ 38.16MHz | -50.19 | [-77.19] |
|  |  | 1,2 | dB | 10 | -3 |
| Propagation condition |  | 1,2 |  | AWGN | AWGN |
| Antenna configuration |  | 1,2 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |

A.10.5.4.1.3	Test Requirements

In both Test 1 and Test 2, the L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 2 shall fulfil the requirements in clauses 10.1.33.1.

A.10.5.4.2	SSB based L1-RSRP measurement on SCell

A.10.5.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.33.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.10.5.4.2.1-1.

Table A.10.5.4.2.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

A.10.5.4.2.2	Test parameters

In this set of test cases there are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell under CCA (Cell 2) and FR1 SCell under CCA (Cell 3). Cell 2 and Cell 3 operate on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model.

Two sub-tests (Test 1 and Test 2) are provided with different Noc  on Cell 2 and Cell 3. The test parameters and applicability for Cell 1 are defined in A.3.7A.2. The test parameters for the Cell 2 and Cell 3 are given in Table A.10.5.4.2.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.10.5.4.2.2-1.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.10.5.4.2.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  |  | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Active PCell/SCell Configuration |  |  | 1,2 |  | PSCell | SCell | PSCell | SCell |
| SSB GSCN |  |  | 1,2 |  | freq1 | freq2 | freq1 | freq2 |
| DL CCA model |  |  | 1,2 |  | As specifieed in A.3.20.2.1 |  |  |  |
| UL CCA model |  |  | 1,2 |  | As specified in A.3.20.2.2 |  |  |  |
| Duplex mode |  |  | 1,2 |  | TDD |  |  |  |
| TDD Configuration |  |  | 1,2 |  | TDDConf.1.1 CCA |  |  |  |
| BWchannel |  |  | 1,2 | MHz | 40: NRB,c = 106 |  |  |  |
| Duplex mode |  |  | 1,2 |  | TDD |  |  |  |
| TDD configuration |  |  | 1,2 |  | TDDConf.1.1 CCA |  |  |  |
| PDSCH Reference measurement channel |  |  | 1,2 |  | SR.1.1 CCA |  |  |  |
| RMSI CORESET Reference Channel |  |  | 1,2 |  | CR.1.1 CCA |  |  |  |
| Dedicated CORESET Reference Channel |  |  | 1,2 |  | CCR.1.1 CCA |  |  |  |
| SSB configuration for Semi-static channel access |  |  | 1,2 |  | SSB.3 CCA |  |  |  |
| SSB configuration for Dynamic channel access |  |  | 1,2 |  | SSB.4 CCA |  |  |  |
| OCNG Patterns |  |  | 1,2 |  | OP.1 |  |  |  |
| TRS configuration |  |  | 1,2 |  | TRS.1.2 TDD |  |  |  |
| Initial BWP Configuration |  |  | 1,2 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP configuration |  |  | 1,2 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| DBT Window Configuration |  |  | 1,2 |  | DBT.1 | DBT.1 | DBT.1 | DBT.1 |
| reportConfigType |  |  | 1,2 |  | - | periodic | - | periodic |
| reportQuantity |  |  | 1,2 |  | - | ssb-Index-RSRP | - | ssb-Index-RSRP |
| Number of reported RS |  |  | 1,2 |  | - | 2 | - | 2 |
| L1-RSRP reporting period |  |  | 1,2 |  | - | slot80 | - | slot80 |
| EPRE ratio of PSS to SSS |  |  | 1,2 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  | 1,2 |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_I |  | 1,2 | dBm/15kHz | -94.65 | -94.65 | [-113] | [-113] |
| Note2 | NR_TDD_FR1_I |  | 1,2 | dBm/SCS | -91.65 | -91.65 | [-110] | [-110] |
|  |  |  | 1,2 | dB | 10 | 10 | -3 | -3 |
| SS-RSRPNote3 | NR_TDD_FR1_I |  | 1,2 | dBm/SCS | -81.65 | -81.65 | [-113] | [-113] |
| IoNote3 | NR_TDD_FR1_I |  | 1,2 | dBm/ 38.16MHz | -50.19 | -50.19 | [-77.19] | [-77.19] |
|  |  |  | 1,2 | dB | 10 | 10 | 10 | 10 |
| Propagation condition |  |  | 1,2 |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  |  | 1,2 |  | 1x2 | 1x2 | 1x2 | 1x2 |
|  |  | Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |

A.10.5.4.2.3	Test Requirements

In both Test 1 and Test 2, the L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 3 shall fulfil the requirements in clauses 10.1.33.1.

<End of Change 4>

<Start of Change 5>

A.11.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell under CCA and FR1 target cell under CCA

A.11.6.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.28.1.1 and 10.1.28.1.2 for inter-frequency measurements with the testing configurations for NR cells in Table A.11.6.1.2.1-1.

Table A.11.6.1.2.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

A.11.6.1.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell under CCA (Cell 1) and a FR1 neighbour cell under CCA (Cell 2) on a different frequency than the PCell. Cell 1 and Cell 2 operate on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model.

Two sub-tests (Test 1 and Test 2) are provided with different Noc. The test parameters for the Cell 1 and Cell 2 are given in Table A.11.6.1.2.2-1 below. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.11.6.1.2.2-1.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

The inter-frequency measurements are supported by a measurement gap.

Table A.11.6.1.2.2-1: SS-RSRP inter-frequency test parameters

| Parameter |  |  | Config | Unit | Test 1 |  |  |  |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 |  |  |  | Cell 2 |  | Cell 1 |  |  | Cell 2 |
| SSB ARFCN |  |  | 1 |  | freq1 |  |  |  | freq2 |  | freq1 |  |  | freq2 |
| DL CCA model |  |  | 1 |  | As specifieed in A.3.20.2.1 |  |  |  |  |  | As specifieed in A.3.20.2.1 |  |  |  |
| UL CCA model |  |  | 1 |  | As specified in A.3.20.2.2 |  |  |  |  |  | As specified in A.3.20.2.2 |  |  |  |
| BWchannel |  |  | 1 | MHz | 40: NRB,c = 106 |  |  |  |  |  | 40: NRB,c = 106 |  |  |  |
| Duplex mode |  |  | 1 |  | TDD |  |  |  |  |  | TDD |  |  |  |
| TDD configuration |  |  | 1 |  | TDDConf.1.1 CCA |  |  |  |  |  | TDDConf.1.1 CCA |  |  |  |
| PDSCH Reference measurement channel |  |  | 1 |  | SR.1.1 CCA | - | SR.1.1 CCA | - |
| RMSI CORESET Reference Channel |  |  | 1 |  | CR.1.1 CCA | - | CR.1.1 CCA | - |
| Dedicated CORESET Reference Channel |  |  | 1 |  | CCR.1.1 CCA |  |  |  |  |  | CCR.1.1 CCA |  | - |  |
| SSB configuration Semi-static channel access |  |  | 1 |  | SSB.3 CCA |  |  |  |  |  | SSB.3 CCA |  |  |  |
| SSB configuration for Dynamic channel access |  |  | 1 |  | SSB.4 CCA |  |  |  |  |  | SSB.4 CCA |  |  |  |
| OCNG Patterns |  |  | 1 |  | OP.1 |  |  |  |  |  | OP.1 |  |  |  |
| TRS configuration |  |  | 1 |  | TRS.1.2 TDD |  |  | - |  |  | TRS.1.2 TDD |  |  |  |
| Initial BWP Configuration |  |  | 1 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP configuration |  |  | 1 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| Time offset with Cell 1 |  |  | 1 | s | - |  | 3 |  |  |  | - | 3 |  |  |
| DBT Window Configuration |  |  | 1 |  | DBT.1 |  |  |  |  |  | DBT.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | 1 | dB | 0 |  |  |  | 0 |  | 0 |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | NR_TDD_FR1_I | 1 | dBm/15kHz | -94.65 |  |  |  |  |  | ( for Cell 3 +8dB) |  |  | [-111] |
| Note2 |  | NR_TDD_FR1_I | 1 | dBm/SCS | -91.65 |  |  |  |  |  | ( for C 3 +8dB) |  |  | [-108] |
|  |  |  | 1 | dB | 10 |  |  |  | 10 |  | 14 |  |  | -3 |
| SS-RSRPNote3 | NR_TDD_FR1_I |  | 1 | dBm/SCS | -81.65 |  |  |  |  |  | (RSRP for Cell 3 +25dB) |  |  | [-111] |
| IoNote3 | NR_TDD_FR1_I |  | 1 | dBm/ 38.16MHz | -50.19 |  |  |  |  |  | (Io for Channel 3 +19.75dB) |  |  | [-75.19] |
|  |  |  | 1 | dB | 10 |  |  |  | 10 |  | 14 |  |  | -3 |
| Propagation condition |  |  | 1 | - | AWGN |  |  |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  | 1 |  | 1x2 |  |  |  |  |  | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |

A.11.6.1.2.3	Test Requirements

In both Test 1 and Test 2, the SS-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the absolute requirement in clause 10.1.28.1.1 and relative requirement in clause 10.1.28.1.2.

<End of Change 5>

<Start of Change 6>

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell

A.11.6.4.1	SSB based L1-RSRP measurement

A.11.6.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.33.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.11.6.4.1.1-1.

Table A.11.6.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

A.11.6.4.1.2	Test parameters

In this set of test cases there is one cell in the test, PCell under CCA (Cell 1). Cell 1 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model.

Two sub-tests (Test 1 and Test 2) are provided with different Noc  on Cell 1. The test parameters for the Cell 1 are given in Table A.11.6.4.1.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.11.6.4.1.2-1.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.11.6.4.1.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1 |  | freq1 | freq1 |
| DL CCA model |  | 1 |  | As specifieed in A.3.20.2.1 | As specifieed in A.3.20.2.1 |
| UL CCA model |  | 1 |  | As specified in A.3.20.2.2 | As specified in A.3.20.2.2 |
| Duplex mode |  | 1 |  | TDD | TDD |
| TDD configuration |  | 1 |  | TDDConf.1.1 CCA | TDDConf.1.1 CCA |
| BWchannel |  | 1 | MHz | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 CCA | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 CCA | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 CCA | CCR.1.1 CCA |
| SSB configuration Semi-static channel access |  | 1 |  | SSB.3 CCA | SSB.3 CCA |
| SSB configuration for Dynamic channel access |  | 1 |  | SSB.4 CCA | SSB.4 CCA |
| OCNG Patterns |  | 1 |  | OP.1 | OP.1 |
| Initial BWP Configuration |  | 1 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration |  | 1 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Dedicated BWP configuration |  | 1 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration |  | 1 |  | DBT.1 | DBT.1 |
| reportConfigType |  | 1 |  | periodic | periodic |
| reportQuantity |  | 1 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS |  | 1 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_TDD_FR1_I | 1 | dBm/15kHz | -94.65 | [-113] |
| Note2 | NR_TDD_FR1_I | 1 | dBm/SCS | -91.65 | [-110] |
|  |  | 1 | dB | 10 | -3 |
| SS-RSRPNote3 | NR_TDD_FR1_I | 1 | dBm/SCS | -81.65 | [-113] |
| IoNote3 | NR_TDD_FR1_I | 1 | dBm/ 38.16MHz | -50.19 | [-77.19] |
|  |  | 1 | dB | 10 | -3 |
| Propagation condition |  | 1 |  | AWGN | AWGN |
| Antenna configuration |  | 1 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

A.11.6.4.1.3	Test Requirements

In both Test 1 and Test 2, the L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 1 shall fulfil the requirements in clauses 10.1.33.1.

A.11.6.4.2	SSB based L1-RSRP measurement on SCell

A.11.6.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.33.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.11.6.4.2.1-1.

Table A.11.6.4.2.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

A.11.6.4.2.2	Test parameters

In this set of test cases there are two cells in the test, one PCell under CCA (Cell 1) and one SCell under CCA (Cell 2). Cell 1 and Cell 2 operate on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model.

Two sub-tests (Test 1 and Test 2) are provided with different Noc  on Cell 1 and Cell 2. The test parameters for the Cell 1 and Cell 2 are given in Table A.11.6.4.2.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.11.6.4.2.2-1.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.11.6.4.2.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  |  | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Active PCell/SCell Configuration |  |  | 1 |  | PSCell | SCell | PSCell | SCell |
| SSB GSCN |  |  | 1 |  | freq1 | freq2 | freq1 | freq2 |
| DL CCA model |  |  | 1 |  | As specifieed in A.3.20.2.1 |  |  |  |
| UL CCA model |  |  | 1 |  | As specified in A.3.20.2.2 |  |  |  |
| Duplex mode |  |  | 1 |  | TDD |  |  |  |
| TDD Configuration |  |  | 1 |  | TDDConf.1.1 CCA |  |  |  |
| BWchannel |  |  | 1 | MHz | 40: NRB,c = 106 |  |  |  |
| Duplex mode |  |  | 1 |  | TDD |  |  |  |
| TDD configuration |  |  | 1 |  | TDDConf.1.1 CCA |  |  |  |
| PDSCH Reference measurement channel |  |  | 1 |  | SR.1.1 CCA |  |  |  |
| RMSI CORESET Reference Channel |  |  | 1 |  | CR.1.1 CCA |  |  |  |
| Dedicated CORESET Reference Channel |  |  | 1 |  | CCR.1.1 CCA |  |  |  |
| SSB configuration for Semi-static channel access |  |  | 1 |  | SSB.3 CCA |  |  |  |
| SSB configuration for Dynamic channel access |  |  | 1 |  | SSB.4 CCA |  |  |  |
| OCNG Patterns |  |  | 1 |  | OP.1 |  |  |  |
| TRS configuration |  |  | 1 |  | TRS.1.2 TDD |  |  |  |
| Initial BWP Configuration |  |  | 1 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP configuration |  |  | 1 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| DBT Window Configuration |  |  | 1 |  | DBT.1 | DBT.1 | DBT.1 | DBT.1 |
| reportConfigType |  |  | 1 |  | - | periodic | - | periodic |
| reportQuantity |  |  | 1 |  | - | ssb-Index-RSRP | - | ssb-Index-RSRP |
| Number of reported RS |  |  | 1 |  | - | 2 | - | 2 |
| L1-RSRP reporting period |  |  | 1 |  | - | slot80 | - | slot80 |
| EPRE ratio of PSS to SSS |  |  | 1 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_I |  | 1 | dBm/15kHz | -94.65 | -94.65 | [-113] | [-113] |
| Note2 | NR_TDD_FR1_I |  | 1 | dBm/SCS | -91.65 | -91.65 | [-110] | [-110] |
|  |  |  | 1 | dB | 10 | 10 | -3 | -3 |
| SS-RSRPNote3 | NR_TDD_FR1_I |  | 1 | dBm/SCS | -81.65 | -81.65 | [-113] | [-113] |
| IoNote3 | NR_TDD_FR1_I |  | 1 | dBm/ 38.16MHz | -50.19 | -50.19 | [-77.19] | [-77.19] |
|  |  |  | 1 | dB | 10 | 10 | 10 | 10 |
| Propagation condition |  |  | 1 |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  |  | 1 |  | 1x2 | 1x2 | 1x2 | 1x2 |
|  |  | Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |

A.11.6.4.2.3	Test Requirements

In both Test 1 and Test 2, the L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 2 shall fulfil the requirements in clauses 10.1.33.1.

<End of Change 6>