TSG-RAN Working Group 4 (Radio) meeting #99-E	R4-2109870

Electronic Meeting, 19th – 27th May 2021

Source:	Ericsson

Title:	On BS RF transmitter requirements for the frequency range 52 to 71 GHz

Agenda item:	9.15.5.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#98-bis-E) specific details for BS RF core requirements applicable for the frequency range 52.6 to 71 GHz was discussed. The outcome was captured and guidance for this meeting was captured in a way forward contribution in [1].

At last RAN4 meeting it was agreed to use NR BS type 2-O as baseline for further discussion related to transmitter requirements for the frequency range expansion up to 71 GHz.

In this contribution we present an overview of BS transmitter requirements and some proposals necessary to progress the work. To further stimulate the discussion draft specification text related to TS 38.104, clause 9 with updates to extend NR support up to 71 GHz is provided as an attachment at the end of contribution.

# Discussion

Based on NR BS type 2-O requirements a transmitter requirement overview for the frequency range 52.6 to 71GHz is presented in Table 2-1.

Table 2-1: Requirement overview

| Requirement | TS 38.104 subclause | Proposals |
| --- | --- | --- |
| Radiated transmit power | 9.2 | Re-use FR2 requirement. |
| OTA base station power | 9.3 | Re-use FR2 requirement. |
| OTA output power dynamics | 9.4 | Re-use FR2 as base line. Extend table to include new SCS and CBW required for 52.6 to 71 GHz. |
| OTA transmit ON/OFF power | 9.5 | TDD OFF power level can be reused from FR2. While transient period is FFS. |
| OTA transmitted signal quality | 9.6 | Frequency error is re-used from FR2. Re-use FR2 up to 64QAM. The need for TAE is FFS. |
| OTA unwanted emissions | 9.7 | Occupied bandwidth can be re-used from FR2. ACLR needs new table entry, OBUE needs new tables to account for licensed and unlicenced operation. The details of emission masks are FFS due to NR carrier bandwidth support, etc. Spurious emission requires new tables. |
| OTA transmitter intermodulation | 9.8 | Not applicable for FR2 and for frequency range 52.6 to 71 GHz. |

From the requirement overview it is clear that several requirements can be reused directly from FR2, while some requirements need dedicated table entries applicable for the frequency range 52.6 to 71 GHz. Some requirements will require new tables to handle requirements applicable in the frequency range 52.6 to 71 GHz, e.g. unwanted emission requirements.

The way forward from last meeting [1] captured following agreements:

- The radiated transmitter characteristics requirements applying to the BS type 2-O should be considered for NR operation in 52.6 to 71 GHz range as the starting point for discussion. Final requirement values need further consideration.

- The BS output power for NR operation in 52.6 to 71 GHz range is declared by the manufacturer. Additional regional requirements can be added to specification to align with regulatory requirements. Consider further detailed output power accuracy values (EIRP and TRP), taking into account also whether and/or how fractional bandwidth is applied.

- Total power dynamic range requirement based on 10*log10(NRB), similar to current FR2, can be applied to 52.6 to 71 GHz. Details especially on wide channel bandwidths with large SCS and applicability on licensed vs. unlicensed bands need to be further clarified.

- Final evaluation of transient times has to consider not only the general ON/OFF mask at start and end slot, for TDD DL/UL boundaries, used in GP timing, but also other use cases related to UE UL, like SRS time mask and PUSCH-PUCCH and SRS time mask. The cases of SRS time mask and PUSCH-PUCCH and SRS time mask have to be investigated in UE RF session.

- Re-use frequency error requirements from current FR2. New EVM window length is defined for new SCS.

- Further evaluate EVM requirements in 52.6 to 71 GHz range.

- Modulations up to 64 QAM are supported.

- Further evaluate TX OFF power, considering also whether existing FR2 requirement -36 dBm/MHz can be valid.

- Further evaluate TAE (MIMO, CA cases), considering also impact of UE RF architecture for CA related TAE, if applicable.

- How to handle low emission PSD due to wide carrier BW needs to be studied in RAN4.

- For licensed operation, adjust OBUE for 52.6 to 71 GHz. Additional regional requirements can be added to specification to align with regulatory requirements.

- Further consider EN 303 722 for unwanted emissions in unlicensed operation.

- Re-use OTA occupied bandwidth from current FR2. Further discuss the measurement step sizes in conformance phase for wider channel bandwidths.

- Further consider whether to derive ACLR based on 70 GHz co-existence study in TR 38.803 or new coexistence study. (Should be aligned with UE side for the approach).

## 2.1	Output power

In TS 38.104 [2], the output power requirement for NR BS type 2-O base stations are divided into two requirements:

- Radiated output power based on EIRP used to characterize the base station capability to generate power in a specific direction.

- OTA base station output power based on TRP used to characterize the base station output power capability.

The EIRP accuracy requirement for FR2 base stations states that the declared EIRP shall fall within an accuracy window of +/- 3.4 dB. For the frequency range 52.6 to 71 GHz it would be reasonable to use the same accuracy windows as base line.

The NR BS RF core specification have support to allow the manufacturer to declare multiple EIRP values per operating band if the fractional band width is wider than 6 %. The intension is to account for the fact that the antenna gain would vary as function of frequency within the operating band, if the operating band is considered wider than 6 %. When the NR specification was created it was decided to set to limit to allow two EIRP declarations for systems supporting an operating band wider than 6 % fractional bandwidth. Below 6 % it was concluded that the EIRP drop can be managed within the specified EIRP accuracy requirement interval.

The parameter fractional bandwidth is defined in percent as:

FBW=100fh-flfc		(Eq. 2.1-1)

, where fh-fl is the width of the operating band and fc is the centre frequency. The EIRP drop within the operating band due to the relative antenna aperture difference between lowest and highest frequency can be expressed as:

δ=10log⁡fh2fl2			(Eq. 2.1-2)

In Table 2.1-1, the FBW and expected EIRP drop is listed for current FR2 bands and proposed new frequency bands for licensed and unlicensed operation within the frequency range 52.6 to 71 GHz.

Table 2.1-1: Expected EIRP drop due to large operating band support

| Operating band | Frequency range fl to fh (MHz) | Centre frequency fc (MHz) | Width fh-fl (MHz) | FBW (%) | EIRP drop d (dB) |
| --- | --- | --- | --- | --- | --- |
| n257 | 26500 to 29500 | 28000 | 3000 | 10.7 | 1.2 |
| n258 | 24250 to 27500 | 25875 | 3250 | 12.6 | 1.3 |
| n259 | 39500 to 43500 | 41500 | 4000 | 9.6 | 1.2 |
| n260 | 37000 to 40000 | 38500 | 3000 | 7.8 | 1.2 |
| n261 | 27500 to 28350 | 27925 | 850 | 3.0 | 1.1 |
| Proposed licensed band | 66000 to 71000 | 68500 | 5000 | 7.3 | 1.2 |
| Proposed unlicensed band | 57000 to 71000 | 64000 | 14000 | 21.9 | 1.6 |

It can be noticed that multiple EIRP values can be declared for most FR2 bands.

For the proposed licensed band between 66000 to 71000 MHz the EIRP drop is in the same range as for previous FR2 bands, hence the FBW declaration concept can be used as defined for FR2. For the proposed frequency range 57000 to 71000 MHz proposed for unlicensed operation, the EIRP drop of 1.6 dB over frequency within the operating band expected. By utilizing two declared EIRP values it seems reasonable to manage the expected EIRP drop within the EIRP accuracy requirement (+/- 3.4 dB). E.g. the drop is just 0.4 dB larger than for band n257. Since power is a limited resource operating within 52.6 to 71 GHz it would not reasonable to make the accuracy requirement wider for TRP accuracy and EIRP accuracy compared to current FR2 requirements.

Proposal 1: Re-use FR2 radiated transmit power (EIRP) requirement for the frequency range 52.6 to 71 GHz.

Proposal 2: Re-use FR2 OTA base station output power (TRP) requirement for the frequency range 52.6 to 71 GHz.

## 2.2	OTA transmit ON/OFF power

The transmit ON/OFF power requirement specifies the transition period between transmitter ON state and transmitter OFF state for a TDD system. The requirement is divided into two parts: TDD OFF power and transient period.

The TDD OFF level of -36 dBm/MHz derived for base station operating within the frequency range 24.25 to 52.6 GHz. The technical background relates to specifying protection of degradation of the receiver at the victim system in a neighbouring cell. Considering the extension up to 71 GHz, the propagation loss will increase, and the base station antenna gain will increase, which will result in cancelation of the frequency dependency.

Based on the analysis in TR 37.817-02, subclause 9.5 [4] the TRP level at the aggressor can be expressed in linear scale as:

TRPa=kTBFLMGvDa=kTBF4πdλ2M4πAeλ2Da=kTBF4πd2MAeDa		(Eq. 2.2-1)

, where kTBF is the victim receiver noise floor, d is the distance between the aggressor and victim, M is the protection margin, Ae is the effective antenna aperture and Da is the aggressor directivity during TX OFF period. Assuming free space path loss, constant physical antenna area, 2 dB higher noise figure and smaller distances between aggressor and victim, TRPa seems to be frequency independent.

Hence, the TDD OFF power level derived for FR2 can be used up to 71 GHz giving the same protection proven to be sufficient for deployed FR2 base stations.

Proposal 3: Define TDD OFF power level of -36 dBm/MHz up to 71 GHz.

## Unwanted emissions

The unwanted emission requirement levels both for in-band emissions and spurious emission for NR in 52.6 to 71 GHz need further considerations. Power Amplifier (PA) dependencies describing the relation between power, linearity and efficiency has been studied during the Study Item (SI) phase where feasible Adjacent Channel Leakage Ratio (ACLR) range has been investigated and further elaborated in this section. Detailed discussions around Operating Band Unwanted Emission (OBUE) or how spurious emission should be handled has been discussed briefly in previous meeting and is further elaborated in this section.

In addition, as the objective is to specify NR in 52.6 to 71 GHz for both licensed and unlicensed operation, some differentiation might be needed considering requirement definition, emission mask shapes as well as levels due to higher power foreseen for licensed operation.

### 2.3.1	ACLR

Empirical measurements of a 28 nm FD-SOI CMOS PA was used to model the behavior at 70 GHz proxy frequency where the non-linear characteristics is kept with the output power scaled as -20 dB/decade while Power Added Efficiency (PAE) scaled as ~-5dB /decade. CP-OFDM waveform at 400 MHz carrier bandwidth was used. Based on this analysis, the important dependencies between output power, linearity and PAE and feasible ACLR values was presented in TR 38.808, subclause 4.2.5.3 [5].

As shown in Figure 2.3.1-1, the achievable output power decreases with increased ACLR. Figure 2.3.1-1 also depicts the relation between ACLR and PAE.

Figure 2.3.1-1: ACLR versus output power (left) and PAE versus ACLR (right)

Figure 2.3.1-1 indicates that a reasonable PAE of 5 to 10% considering the thermal challenges, a feasible ACLR level would be in the range of 20 to 25 dB for frequency bands within 52 to 71 GHz. The feasible ACLR values should be weighted in when possible co-existence studied are conducted to settle the ACLR requirements.

In addition, as for frequency ranges between 52 to 71 GHz larger bandwidths than for FR2 is anticipated (~2 GHz), it is also essential to consider the impact of ultra-wide bandwidths to account when defining the ACLR, OBUE and spurious emission requirements.

Initial analysis of PA dependencies indicates a feasible ACLR range of 20 to 25 dB for 52 to 71 GHz considering reasonable power efficiency needed to handle the thermal aspects.

In TR 38.803 [3], extensive co-existence studies was conducted and the corresponding conclusion for 30, 45 and 70 GHz proxy frequency relevant for NR in 52.6 to 71 GHz concluded the necessary UE ACS and BS ACLR as listed in Table 2.3.1-1 and Table 2.3.1-2.

Table 2.3.1-1: UE ACS

| UE ACS (dB) |  |  |
| --- | --- | --- |
| 30GHz | 45GHz | 70GHz |
| 22.5 | 21.5 | 20.5 |

Table 2.3.1-2: BS ACLR

| BS ACLR (dB) |  |  |
| --- | --- | --- |
| 30GHz | 45GHz | 70GHz |
| 27.5 | 25.5 | 23.5 |

The needed BS ACLR for base station operating at 70 GHz based on extensive co-existence analysis in TR 38.803 is 23.5 dB which very well match the feasibility study considering power, linearity and PAE.

In addition, considering some existing emission requirements, ACLR level of ~20 dB (described in sub-section 2.3.2), it sems that ACLR level of 21 dB could be a sufficient level for NR in 52.6 to 71 GHz.

Observation 1:

The analysis in TR38.803 considering needed ACLR for 70 GHz proxy frequency very well match the outcome of feasibility analysis during SI.

Proposal 4:

Taking to account both co-existence studies in TR 38.803, existing emission masks and feasibility analysis of power amplifiers, the BS ACLR shall be set to 21 dB.

### 2.3.2 	Unwanted emission PSD levels and existing emission masks

In previous meeting, the impact of large bandwidths such as 2000 MHz and 2160 MHz as maximum bandwidths for NR in 52.6 to 71 GHz on Power Spectral Density (PSD) of the wanted signal/carrier was discussed.

The array antenna sizes has been agreed in TR 38.808, subclause 4.2.5.1 covering the array sizes of 8x16, 16x16 and 32x32. Assuming a moderate conducted power of -3.0 dBm per PA, element gain of 5.3 dBi, element loss of 2.2 dB and 2000 MHz of carrier bandwidth, the corresponding achievable TRP, EIRP and power spectral density over 1 MHz of the carrier can be calculated.

Table 2.3.2-1: Wanted signal PSD levels

| Array type | Array size | TRP (dBm) | EIRP (dBm) | TRP PSD (dBm/MHz) |
| --- | --- | --- | --- | --- |
| 1 | 8x16 | 18.9 | 47.4 | -14.1 |
| 2 | 16x16 | 21.9 | 53.5 | -11.1 |
| 3 | 32x32 | 27.9 | 65.5 | -5.1 |

It can be observed that the power spectral density of wanted signal for large carrier bandwidth is quite low and in case of array type 1 and 2 which is a reasonable implementation assumption, the power spectral density of wanted signal is more or less similar to in-band unwanted emission levels of -13 dBm/MHz given that the emissions from the PA outside the carrier would be even lower compared to carrier.

As all OTA unwanted emission requirements are expressed as TRP and assuming an ACLR level of 21 dB applied to wanted signal TRP, interesting observations can be made as shown in table 2.3.2-2.

Table 2.3.2-2: Expected emission PSD levels for licensed operation

| Array type | Array size | TRP (dBm) | TRP PSD (dBm/MHz) | Adjacent channel TRP PSD (dBm/MHz) |
| --- | --- | --- | --- | --- |
| 1 | 8x16 | 18.9 | -14.1 | -35.1 |
| 2 | 16x16 | 21.9 | -11.1 | -32.1 |
| 3 | 32x32 | 27.9 | -5.1 | -26.1 |

Thus, with an ACLR of 21 dB, the in-band emissions for in particular for array type 1 and possibly array type 2 will be lower than the Category-B spurious emission limit of -30 dBm/MHz which is the strictest regulatory limit.

Taking to account possible measurement uncertainty, array type 2 and array type 3 would possibly require additional measures to reach the Category- B limits.

To further analyse to possible OBUE, a brief summary of some of the existing masks is given below.

For 60 GHz IEEE systems, ITU-R M.2003-2 [6] recommendation consists of several masks based on 2.16 GHz carrier bandwidth as visualised in Figure 2.3.2-1, 2.3.2-2 and 2.3.2-3.

Figure 2.3.2-1: ITU-R recommended mask for single carrier case

Figure 2.3.2-2: ITU-R recommended mask for channel bonding (aggregated carriers) cases for up to 4 carriers

Figure 2.3.2-3: ITU-R recommended alternative mask for channel bonding (aggregated carriers) cases for up to 4 carriers

In Europe, the CEPT emission mask requirements for c1 and c3 [8] are fairly similar to ITU-R recommendation and are defined as following:

The transmitter unwanted emissions in the out-of-band domain shall be less than or equal to the relative limits provided in Figure 2.3.2-4, where the x-axis is the ratio of frequency (F) to declared nominal channel bandwidth (BW), or an absolute level of -30 dBm within a 1 MHz bandwidth, whichever is greater. Non-adjacent channels shall be tested separately. Within the 57 to 71 GHz band and outside -2.5 to +2.5 ratio of frequency to declared nominal bandwidth range the -30 dBr or -30 dBm in 1 MHz whichever is the greater shall apply.

Figure 2.3.2-4: Transmit emission mask

The limits in the relative mask above apply to the declared nominal bandwidth range (can include carrier aggregation) and possibly different carrier bandwidths as discussed for NR in 52.6 to 71 GHz.

It can be observed that for unlicensed operation, all version of the emission masks possibly applicable in different regions, would require either 30 dBc attenuation at almost second adjacent channel or comply to absolute requirement of -30 dBm/MHz which can co-inside as in-band requirements. Thus, the in-band emission requirements and strictest emission requirements seem to be the same.

For unlicensed spectrum with 40 dBm limits on peak EIRP, the array type 1 would imply a PA power of ~-10.4 dBm and the resulting power spectral densities as in Table 2.3.2-3 where the achievable levels are by far below the -30 dBm/MHz limit.

Table 2.3.2-3: Expected emission PSD levels for unlicensed operation

| Array type | Array size | TRP (dBm) | EIRP (dBm) | TRP PSD (dBm/MHz) | Adjacent channel TRP PSD (dBm/MHz) |
| --- | --- | --- | --- | --- | --- |
| 1 | 8x16 | 11.5 | 40 | -22.5 | -42.5 |

Given the diverse options of emission mask applicable in different regions when unlicensed operation is concerned, as well as diverse SCS and carrier bandwidths for NR in 52.6 to 71 GHz, further discussions are necessary in RAN4 to decide on emission mask/OBUE for unlicensed operation.

Proposal 5: Due to diverse emission mask requirements applicable in different regions and the flexibility in terms of both supported SCS and carrier bandwidths for NR in 52.6 to 71 GHz, further discussions and analysis is needed in RAN4 before defining the OBUE/transmitter emission masks.

However, for licensed operation based on array type 2 or array type 3, with higher EIRP, the absolute level of -30 dBm/MHz or 30 dBc as in-band requirement is unnecessarily stringent as ACLR of 21 dB seems sufficient due to co-existence. In addition unnecessarily stringent in-band emission requirements based on 30 dBc or -30 dBm/MHz imply the need for more sophisticated linearization where the linearization bandwidth should be ~5 times the RFBW resulting in linearization bandwidth of many GHz and sampling-rates of many Gbps stressing the signal processing as well as interfaces. Applying power back-off to comply to unnecessarily stringent in-band emission requirements will result in very low power efficiency as depicted in Figure 2.3.1-1.

For licensed operation supporting higher EIRP levels, there is a need to consider proper OBUE requirements to handle the co-existence and ensure power efficient equipment. As the OBUE requirements for FR2 has been designed after long discussion, it is natural to re-use the FR2 approach and adapt the OBUE/emission mask for NR in 52.6 to 71 GHz

Proposal 6: For licensed operation supporting higher EIRP levels, RAN4 should re-use the FR2 approach and adapt the FR2 OBUE/emission mask for NR in 52.6 to 71 GHz and make adaptations taking to account larger carrier bandwidths.

### 2.3.3	Spurious emission

For NR operation in 52.6 to 71 GHz when both licensed and unlicensed operation is concerned, RAN4 requirements should cater for both Category-A and Category-B type of spurious emission requirements valid as regional requirements.

In addition, considering the large bandwidths supported for NR in 52.6 to 71 GHz, the step frequencies Fstep,X should be adapted based on guidelines in ITU-R SM.1539 [7] which are also reflected Table 2.3.3-1.

Table 2.3.3-1: Guideline values for frequency separation between the centre frequency and the boundary of the spurious domain

Proposal 7: For licensed operation and unlicensed operation, RAN4 should re-use the FR2 approach and use FR2 spurious emission requirements for NR in 52.6 to 71 GHz and make adaptations with respect Fstep,X taking to account larger carrier bandwidths.

As discussed in previous meeting, for low PSD scenarios some restrictions in testing can be done and this can be further discussed when conformance work starts.

## 2.4	TAE

In the text proposal to TR 38.808: Timing considerations for operation between 52.6 and 71 GHz [5], the BS Time Alignment Error (TAE) is discussed. The relevant section is shown below.

#### 4.2.2.5 BS Time alignment error (TAE) Another aspect of phase synchronization is MIMO operation and related TAE requirements. The MIMO TAE requirement should be specified with the consideration of the implementation/deployment scenarios. Current BS OTA TAE requirements in clause 9.6.3 of TS 38.104 specify 65 ns for MIMO transmission [77]. It has been discussed in [78] that the current requirement has been in place since UMTS and is the same as quarter of the UMTS chip rate time, i.e. 65 ns matches to 1/(4x3.84) Mcps rate. Improvement in performance has taken place in the past 20 years, and therefore it would be reasonable to consider improvements to TAE requirements.

In the WF from RAN4 #98bis-e [1] it is stated:

… Further evaluate TAE (MIMO, CA cases), considering also impact of UE RF architecture for CA related TAE, if applicable. …

In TS 38.104 the current requirement for BS type 2-O (FR2), looks like this:

#### 9.6.3.3 Minimum requirement for BS type 2-O For MIMO transmission, at each carrier frequency, OTA TAE shall not exceed 65 ns. For intra-band contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 130 ns. For intra-band non-contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 260 ns. For inter-band carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3 µs.

### 2.4.1 	MIMO TAE

Current MIMO requirement, up to and including SCS = 120 kHz for data for FR2 is 65 ns.

The NR MIMO requirement originates, as pointed out in TR 38.808, from WCDMA ¼ * 3.84 Mcip/s = 65 ns. However, this has just been copied over into LTE and NR FR1 and NR FR2 without any technical analysis.

Observation 2: Existing NR MIMO TAE = 65 ns requirement has just been copied over into LTE and NR FR1 and NR FR2 without any technical analysis.

In AAS Work Items, there has been a lot of development regarding how to handle TAE. In AAS for radiated requirements, TR 37.843 [11], we have, for TAE:

### 5.5.3 Time Alignment Error # 5.5.3.1 Background information on the conducted requirement Timing Alignment Error (TAE) is a signal quality requirement specified for transmit diversity, MIMO transmission and CA. TAE is defined as the largest timing difference (i.e. error) between two reference signals, and is only applicable for base stations transmitting from multiple antennas via transmit diversity, MIMO, CA, or some combinations of these three. In case of NR, there is no TX diversity transmission mode defined. As a result, no TAE tests need to be developed for TX diversity for NR operation. In the conducted test setup, the differences between two antenna ports is measured, depending of the functionality of the signal analyser, TAE between the two or more signals can be measured simultaneously. To achieve precise measurements, the RF cables being used during the conducted test should be equal in electrical length. For AAS BS the number of TAB connectors could be large. Therefore, a concept for reducing number of test combinations was introduced in TS 37.105, sub-clause 6.5.3.4 [3]. In TAE requirement for AAS BS is designed as: The TAE between any two TAB connectors from different transmitter groups shall not exceed the specified minimum requirements below. , where transmitter groups are associated with the TAB connectors in the transceiver unit array corresponding to TX diversity, MIMO transmission, CA, etc.

We have, in particular, “The TAE between any two TAB connectors from different transmitter groups shall not exceed the specified minimum requirements below.” This means that AAS BS with radiated requirements do not have TAE requirements inside a transceiver group. Instead the BS conformance EIRP and EVM verify actual performance. The statements for FR1 conducted requirements in TR 37.842 are similar.

Observation 3: AAS BS for radiated requirements do not have TAE requirements inside a transceiver group. Instead the BS conformance EIRP and EVM verify actual performance. This is a concept that could be used for the extension of NR to 52.6 - 71 GHz WI as well. Applying similar calibration schemes all over the array implies good coherency between transceiver groups and thus the TAE requirement become obsolete, in particular for higher frequencies.

In fact, to meet EIRP and EVM requirements, in BS conformance any significant time misalignment would affect the beam and introduce beam widening, loss of gain in main lobes and increased sidelobes. This effect is generic and not strongly connected to any particular feature.

Observation 4: To meet EIRP and EVM requirements, in BS conformance any significant time misalignment would affect the beam and introduce beam widening, loss of gain in main lobes and increased sidelobes. This effect is generic and not strongly connected to any particular feature, like MIMO.

Proposal 8: Remove TAE requirements for MIMO for extension to 71 GHz WI.

### 2.4.2	CA TAE

The same observation like for MIMO in Observation 4 can be made here, for CA contiguous intra band and non-contiguous intra band, if we assume colocation. Applying similar calibration schemes all over the array implies good coherency between transceiver groups and thus the TAE requirement become obsolete in particular for higher frequencies.

Proposal 9: Assume colocation for contiguous intra band CA and non-contiguous intra band CA for extension to 71 GHz WI.

Proposal 10: Remove TAE requirements for contiguous intra band CA and non-contiguous intra band CA for extension to 71 GHz WI.

The TAE requirement for inter band of 3 µs is mainly given by limits of synchronizing non-colocated gNB, so we essentially fall back to the Cell Phase sync requirement for TDD.

Proposal 11: Keep TAE = 3 µs for inter band CA for extension to 71 GHz WI.

# Conclusion

In this contribution technical challenges related to the extension of NR to support up to 71 GHz in TS 38.104 [2] have been studied.

From the technical analysis it is clear that more careful work is required to find a concept on how to handle the support for licensed and unlicensed operation with respect to unwanted emission requirements. For other requirements it is straight forward to add support in current specification.

Based on the technical analysis provided in this contribution following proposals are presented for approval:

Proposal 1: Re-use FR2 radiated transmit power (EIRP) requirement for the frequency range 52 to 71 GHz.

Proposal 2: Re-use FR2 OTA base station output power (TRP) requirement for the frequency range 52 to 71 GHz.

Proposal 3: Define TDD OFF power level of -36 dBm/MHz up to 71 GHz.

Proposal 4: Taking to account both co-existence studies in TR 38.803, existing emission masks and feasibility analysis of power amplifiers, the BS ACLR shall be set to 21 dB.

Proposal 5: Due to diverse emission mask requirements applicable in different regions and the flexibility in terms of both supported SCS and carrier bandwidths for NR in 52.6 to 71 GHz, further discussions and analysis is needed in RAN4 before defining the OBUE/transmitter emission masks.

Proposal 6: For licensed operation supporting higher EIRP levels, RAN4 should re-use the FR2 approach and adapt the FR2 OBUE/emission mask for NR in 52.6 to 71 GHz and make adaptations taking to account larger carrier bandwidths.

Proposal 7: For licensed operation and unlicensed operation, RAN4 should re-use the FR2 approach and use FR2 spurious emission requirements for NR in 52.6 to 71 GHz and make adaptations with respect Fstep,X taking to account larger carrier bandwidths.

Proposal 8: Remove TAE requirements for MIMO for extension to 71 GHz WI.

Proposal 9: Assume colocation for contiguous intra band CA and non-contiguous intra band CA for extension to 71 GHz WI.

Proposal 10: Remove TAE requirements for contiguous intra band CA and non-contiguous intra band CA for extension to 71 GHz WI.

Proposal 11: Keep TAE = 3 µs for inter band CA for extension to 71 GHz WI.

At the end of this contribution draft specification text relevant for TS 38.104, subclause 9 is attached to stimulate further discussion. The draft was created using the base line from last meeting to use BS type 2-O as starting point together with some initial adaptation to support up to 71 GHz added using change marks. The draft has added new table entries and text to account for the frequency extension up to 71 GHz. For non-resolved aspects editorial notes are inserted and FFS for values which require more analysis.

# References

[1]	R4-2106113, “WF on BS RF TX requirements for 52.6 – 71 GHz”, Nokia, Nokia Shanghai Bell

[2]	TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

[3] 	TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”, 3GPP

[4]	TR 38.817-02, “General aspects for Base Station (BS) Radio Frequency (RF) for NR”, 3GPP

[5]	TR 38.808, “Study on supporting NR from 52.6 GHz to 71 GHz”, 3GPP

[6]	Recommendation ITU-R M.2003-2, “Multiple Gigabit Wireless Systems in frequencies around 60 GHz”, ITU-R

[7]	Recommendation ITU-R SM.1539-1, “Variation of the boundary between the out-of-band and spurious domains required for the application of Recommendations ITU-R SM.1541andITU-RSM.329”, ITU-R

[8]	ERC Recommendation 70-03, “Relating to the use of Short Range Devices (SRD)”, CEPT

DRAFT:

# 9	Radiated transmitter characteristics

## 9.1	General

Radiated transmitter characteristics requirements apply on the BS type 1-H, BS type 1-O, or BS type 2-O including all its functional components active and for all foreseen modes of operation of the BS unless otherwise stated.

## 9.2	Radiated transmit power

### 9.2.1	General

BS type 1-H, BS type 1-O and BS type 2-O are declared to support one or more beams, as per manufacturer's declarations specified in TS 38.141-2 [6]. Radiated transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction.

For each beam, the requirement is based on declaration of a beam identity, reference beam direction pair, beamwidth, rated beam EIRP, OTA peak directions set, the beam direction pairs at the maximum steering directions and their associated rated beam EIRP and beamwidth(s).

For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the base station is declared to radiate at the associated beam peak direction during the transmitter ON period.

For each beam peak direction associated with a beam direction pair within the OTA peak directions set, a specific rated beam EIRP level may be claimed. Any claimed value shall be met within the accuracy requirement as described below. Rated beam EIRP is only required to be declared for the beam direction pairs subject to conformance testing as detailed in TS 38.141-2 [6].

NOTE 1:	OTA peak directions set is set of beam peak directions for which the EIRP accuracy requirement is intended to be met. The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

NOTE 2:	A beam direction pair is data set consisting of the beam centre direction and the related beam peak direction.

NOTE 3:	A declared EIRP value is a value provided by the manufacturer for verification according to the conformance specification declaration requirements, whereas a claimed EIRP value is provided by the manufacturer to the equipment user for normal operation of the equipment and is not subject to formal conformance testing.

For operating bands where the supported fractional bandwidth (FBW) is larger than 6%, two rated carrier EIRP may be declared by manufacturer:

-	Prated,c,FBWlow for lower supported frequency range, and

-	Prated,c,FBWhigh for higher supported frequency range.

For frequencies in between FFBWlow and FFBWhigh the rated carrier EIRP is:

-	Prated,c,FBWlow, for the carrier whose carrier frequency is within frequency range FFBWlow ≤ f < (FFBWlow +FFBWhigh) / 2,

-	Prated,c,FBWhigh, for the carrier whose carrier frequency is within frequency range (FFBWlow +FFBWhigh) / 2 ≤ f ≤FFBWhigh.

### 9.2.2	Minimum requirement for BS type 1-H and BS type 1-O

For each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ±2.2 dB of the claimed value.

For BS type 1-O only, for each declared beam, in extreme conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ±2.7 dB of the claimed value.

Normal and extreme conditions are defined in TS 38.141-2, annex B [6].

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

### 9.2.3	Minimum requirement for BS type 2-O

For each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ± 3.4 dB of the claimed value.

For each declared beam, in extreme conditions, for any specific beam peak direction associated with a beam direction pair within the OTA peak directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ±4.5 dB of the claimed value.

Normal and extreme conditions are defined in TS 38.141-2, annex B [6].

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

## 9.3	OTA base station output power

### 9.3.1	General

OTA BS output power is declared as the TRP radiated requirement, with the output power accuracy requirement defined at the RIB during the transmitter ON period. TRP does not change with beamforming settings as long as the beam peak direction is within the OTA peak directions set. Thus the TRP accuracy requirement must be met for any beamforming setting for which the beam peak direction is within the OTA peak directions set.

The BS rated carrier TRP output power for BS type 1-O shall be within limits as specified in table 9.3.1-1.

Table 9.3.1-1: BS rated carrier TRP output power limits for BS type 1-O

| BS class | Prated,c,TRP |
| --- | --- |
| Wide Area BS | (note) |
| Medium Range BS | ≤ + 47 dBm |
| Local Area BS | ≤ + 33 dBm |
| NOTE: There is no upper limit for the Prated,c,TRP of the Wide Area Base Station. |  |

There is no upper limit for the rated carrier TRP output power of BS type 2-O.

For Band n41 and n90 operation in Japan, the rated output power, Prated,c,TRP, declared by the manufacturer shall be equal to or less than 20 W per 10 MHz bandwidth.

Despite the general requirements for the BS output power described in clauses 9.3.2 – 9.3.3, additional regional requirements might be applicable.

NOTE:	In certain regions, power limits corresponding to BS classes may apply for BS type 2-O.

### 9.3.2	Minimum requirement for BS type 1-O

In normal conditions, the BS type 1-O maximum carrier TRP output power, Pmax,c,TRP measured at the RIB shall remain within ±2 dB of the rated carrier TRP output power Prated,c,TRP, as declared by the manufacturer.

Normal conditions are defined in TS 38.141-1, annex B [6].

### 9.3.3	Minimum requirement for BS type 2-O

In normal conditions, the BS type 2-O maximum carrier TRP output power, Pmax,c,TRP measured at the RIB shall remain within ±3 dB of the rated carrier TRP output power Prated,c,TRP, as declared by the manufacturer.

Normal conditions are defined in TS 38.141-2, annex B [6].

### 9.3.4	Additional requirements (regional)

In certain regions, additional regional requirements may apply.

## 9.4	OTA output power dynamics

### 9.4.1	General

The requirements in clause 9.4 apply during the transmitter ON period. Transmit signal quality (as specified in clause 9.6) shall be maintained for the output power dynamics requirements.

The OTA output power requirements are directional requirements and apply to the beam peak directions over the OTA peak directions set.

### 9.4.2	OTA RE power control dynamic range

#### 9.4.2.1	General

The OTA RE power control dynamic range is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,EIRP) for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

#### 9.4.2.2	Minimum requirement for BS type 1-O

The OTA RE power control dynamic range is specified the same as the conducted RE power control dynamic range requirement for BS type 1-C and BS type 1-H in table 6.3.2.2-1.

### 9.4.3	OTA total power dynamic range

#### 9.4.3.1	General

The OTA total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE 1:	The upper limit of the OTA total power dynamic range is the BS maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all RBs. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbol carries PDSCH and not contain RS or SSB.

#### 9.4.3.2	Minimum requirement for BS type 1-O

OTA total power dynamic range minimum requirement for BS type 1-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified for the conducted requirement for BS type 1-C and BS type 1-H in table 6.3.3.2-1.

#### 9.4.3.3	Minimum requirement for BS type 2-O

OTA total power dynamic range minimum requirement for BS type 2-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified in table 9.4.3.3-1.

Table 9.4.3.3-1: Minimum requirement for BS type 2-O total power dynamic range

| SCS | OTA total power dynamic range (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | TBD MHz |
| 60 (NOTE 1) | 18.1 | 21.2 | 24.2 | N/A | N/A | N/A | N/A |
| 120 | 15.0 (NOTE 1) | 18.1 | 21.2 | 24.2 | N/A | N/A | N/A |
| 480 (NOTE 2) | N/A | N/A | N/A | TBD | TBD | TBD | N/A |
| 960 (NOTE 2) | N/A | N/A | N/A | TBD | TBD | TBD | TBD |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25 to 52.60 GHz NOTE 2: Applicable to bands defined within the frequency spectrum range of 52.60 to 71.00 GHz |  |  |  |  |  |  |  |

## 9.5	OTA transmit ON/OFF power

### 9.5.1	General

OTA transmit ON/OFF power requirements apply only to TDD operation of NR BS.

### 9.5.2	OTA transmitter OFF power

#### 9.5.2.1	General

OTA transmitter OFF power is defined as the mean power measured over 70/N µs filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the BS (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

For BS supporting intra-band contiguous CA, the OTA transmitter OFF power is defined as the mean power measured over 70/N us filtered with a square filter of bandwidth equal to the Aggregated BS Channel Bandwidth BWChannel_CA centred on (Fedge,high+Fedge,low)/2 during the transmitter OFF period. N = SCS/15, where SCS is the smallest supported Sub Carrier Spacing in kHz in the Aggregated BS Channel Bandwidth.

For BS type 1-O, the transmitter OFF power is defined as the output power at the co-location reference antenna conducted output(s). For BS type 2-O the transmitter OFF power is defined as TRP.

For multi-band RIBs and single band RIBs supporting transmission in multiple bands, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

#### 9.5.2.2	Minimum requirement for BS type 1-O

The total power from all co-location reference antenna conducted output(s) shall be less than -106 dBm/MHz.

#### 9.5.2.3	Minimum requirement for BS type 2-O

The OTA transmitter OFF TRP spectral density for BS type 2-O shall be less than -36 dBm/MHz.

### 9.5.3	OTA transient period

#### 9.5.3.1	General

The OTA transmitter transient period is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

This requirement shall be applied at each RIB supporting transmission in the operating band.

#### 9.5.3.2	Minimum requirement for BS type 1-O

For BS type 1-O, the OTA transmitter transient period shall be shorter than the values listed in the minimum requirement table 9.5.3.2-1.

Table 9.5.3.2-1: Minimum requirement for the OTA transmitter transient period for BS type 1-O

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

#### 9.5.3.3	Minimum requirement for BS type 2-O

For BS type 2-O, the OTA transmitter transient period shall be shorter than the values listed in the minimum requirement table 9.5.3.3-1.

Table 9.5.3.3-1: Minimum requirement for the OTA transmitter transient period for BS type 2-O

| Frequency range | Transition | Transient period length (µs) |
| --- | --- | --- |
| 24.25 to 52.60 GHz | OFF to ON | 3 |
|  | ON to OFF | 3 |
| 52.6 to 71 GHz | OFF to ON | FFS |
|  | ON to OFF | FFS |

## 9.6	OTA transmitted signal quality

### 9.6.1	OTA frequency error

#### 9.6.1.1	General

The requirements in clause 9.6.1 apply to the transmitter ON period.

OTA frequency error is the measure of the difference between the actual BS transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

OTA frequency error requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

#### 9.6.1.2	Minimum requirement for BS type 1-O

For BS type 1-O, the modulated carrier frequency of each NR carrier configured by the BS shall be accurate to within the accuracy range given in table 6.5.1.2-1 observed over 1 ms.

#### 9.6.1.3	Minimum requirement for BS type 2-O

For BS type 2-O, the modulated carrier frequency of each NR carrier configured by the BS shall be accurate to within the accuracy range given in table 9.6.1.3-1 observed over 1 ms.

Table 9.6.1.3-1: OTA frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±0.05 ppm |
| Medium Range BS | ±0.1 ppm |
| Local Area BS | ±0.1 ppm |

### 9.6.2	OTA modulation quality

#### 9.6.2.1	General

Modulation quality is defined by the difference between the measured carrier signal and an ideal signal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). Details about how the EVM is determined are specified in Annex B for FR1 and Annex C for FR2.

OTA modulation quality requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

#### 9.6.2.2	Minimum Requirement for BS type 1-O

For BS type 1-O, the EVM levels of each NR carrier for different modulation schemes on PDSCH outlined in table 6.5.2.2-1 shall be met. Requirements shall be the same as clause 6.5.2.2 and follow EVM frame structure from clause 6.5.2.3.

#### 9.6.2.3	Minimum Requirement for BS type 2-O

For BS type 2-O, the EVM levels of each NR carrier for different modulation schemes on PDSCH outlined in table 9.6.2.3-1 shall be met, following the EVM frame structure described in clause 9.6.2.3.1.

Table 9.6.2.3-1: EVM requirements for BS type 2-O carrier

| Modulation scheme for PDSCH | Required EVM (%) |
| --- | --- |
| QPSK | 17.5 |
| 16QAM | 12.5 |
| 64QAM | 8 |
| 256QAM (NOTE) | 3.5 |
| NOTE: Applicable to bands defined within the frequency spectrum range of 24.25 to 52.60 GHz |  |

##### 9.6.2.3.1	EVM frame structure for measurement

EVM requirements shall apply for each NR carrier over all allocated resource blocks. Different modulation schemes listed in table 9.6.2.3-1 shall be considered for rank 1.

For NR, for all bandwidths, the EVM measurement shall be performed for each NR carrier over all allocated resource blocks and downlink subframes within 10 ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries.

### 9.6.3	OTA time alignment error

#### 9.6.3.1	General

This requirement shall apply to frame timing in MIMO transmission, carrier aggregation and their combinations.

Frames of the NR signals present in the radiated domain are not perfectly aligned in time. In relation to each other, the RF signals present in the radiated domain may experience certain timing differences.

The TAE is specified for a specific set of signals/transmitter configuration/transmission mode.

For a specific set of signals/transmitter configuration/transmission mode, the OTA Time Alignment Error (OTA TAE) is defined as the largest timing difference between any two different NR signals. The OTA time alignment error requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

#### 9.6.3.2	Minimum requirement for BS type 1-O

For MIMO transmission, at each carrier frequency, OTA TAE shall not exceed 65 ns.

For intra-band contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 260 ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3 µs.

For inter-band carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3 µs.

Table 9.6.3.2-1: Void

Table 9.6.3.2-2: Void

Table 9.6.3.2-3: Void

#### 9.6.3.3	Minimum requirement for BS type 2-O

For MIMO transmission within the frequency spectrum range of 24.25 to 52.60 GHz, at each carrier frequency, OTA TAE shall not exceed 65 ns.

For intra-band contiguous carrier aggregation within the frequency spectrum range of 24.25 to 52.60 GHz, with or without MIMO, OTA TAE shall not exceed 130 ns.

For intra-band non-contiguous carrier aggregation within the frequency spectrum range of 24.25 to 52.60 GHz, with or without MIMO, OTA TAE shall not exceed 260 ns.

For inter-band carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3 µs.

Table 9.6.3.3-1: Void

Table 9.6.3.3-2: Void

Table 9.6.3.3-3: Void

## 9.7	OTA unwanted emissions

### 9.7.1	General

Unwanted emissions consist of so-called out-of-band emissions and spurious emissions according to ITU definitions ITU-R SM.329 [2]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the BS channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The OTA out-of-band emissions requirement for the BS type 1-O and BS type 2-O transmitter is specified both in terms of Adjacent Channel Leakage power Ratio (ACLR) and operating band unwanted emissions (OBUE). The OTA Operating band unwanted emissions define all unwanted emissions in each supported downlink operating band plus the frequency ranges ΔfOBUE above and ΔfOBUE below each band. OTA Unwanted emissions outside of this frequency range are limited by an OTA spurious emissions requirement.

The maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The value of ΔfOBUE is defined in table 9.7.1-1 for BS type 1-O and BS type 2-O for the NR operating bands.

Table 9.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL,high – FDL,low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 MHz | 40 |
| BS type 2-O | FDL,high – FDL,low ≤ 4000 MHz | 1500 |
|  | 4000 MHz < FDL,high – FDL,low ≤ 14000 MHz | FFS |

The unwanted emission requirements are applied per cell for all the configurations.  Requirements for OTA unwanted emissions are captured using TRP, directional requirements or co-location requirements as described per requirement.

There is in addition a requirement for occupied bandwidth.

### 9.7.2	OTA occupied bandwidth

#### 9.7.2.1	General

The OTA occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. See also recommendation ITU-R SM.328 [3].

The value of /2 shall be taken as 0.5%.

The OTA occupied bandwidth requirement shall apply during the transmitter ON period for a single transmitted carrier. The minimum requirement below may be applied regionally. There may also be regional requirements to declare the OTA occupied bandwidth according to the definition in the present clause.

The OTA occupied bandwidth is defined as a directional requirement and shall be met in the manufacturer's declared OTA coverage range at the RIB.

#### 9.7.2.2	Minimum requirement for BS type 1-O and BS type 2-O

The OTA occupied bandwidth for each NR carrier shall be less than the BS channel bandwidth. For intra-band contiguous CA, the OTA occupied bandwidth shall be less than or equal to the Aggregated BS Channel Bandwidth.

### 9.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)

#### 9.7.3.1	General

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

The requirement shall be applied per RIB during the transmitter ON period.

#### 9.7.3.2	Minimum requirement for BS type 1-O

The ACLR (CACLR) absolute basic limits in table 6.6.3.2-2 + X, 6.6.3.2-2a + X (where X = 9 dB) or the ACLR (CACLR) basic limit in table 6.6.3.2-1, 6.6.3.2-2a or 6.6.3.2-3, whichever is less stringent, shall apply.

For a RIB operating in multi-carrier or contiguous CA, the ACLR requirements in clause 6.6.3.2 shall apply to BS channel bandwidths of the outermost carrier for the frequency ranges defined in table 6.6.3.2-1.For a RIB operating in non-contiguous spectrum, the ACLR requirement in clause 6.6.3.2 shall apply in sub-block gaps for the frequency ranges defined in table 6.6.3.2-2a, while the CACLR requirement in clause 6.6.3.2 shall apply in sub-block gaps for the frequency ranges defined in table 6.6.3.2-3.

For a multi-band RIB, the ACLR requirement in clause 6.6.3.2 shall apply in Inter RF Bandwidth gaps for the frequency ranges defined in table 6.6.3.2-2a, while the CACLR requirement in clause 6.6.3.2 shall apply in Inter RF Bandwidth gaps for the frequency ranges defined in table 6.6.3.2-3.

#### 9.7.3.3	Minimum requirement for BS type 2-O

The OTA ACLR limit is specified in table 9.7.3.3-1.

The OTA ACLR absolute limit is specified in table 9.7.3.3-2.

The OTA ACLR (CACLR) absolute limit in table 9.7.3.3-2 or 9.7.3.3-4a or the ACLR (CACLR) limit in table 9.7.3.3-1, 9.7.3.3-3 or 9.7.3.3-4, whichever is less stringent, shall apply.

For a RIB operating in multi-carrier or contiguous CA, the OTA ACLR requirements in table 9.7.3.3-1 shall apply to BS channel bandwidths of the outermost carrier for the frequency ranges defined in the table.For a RIB operating in non-contiguous spectrum, the OTA ACLR requirement in table 9.7.3.3-3 shall apply in sub-block gaps for the frequency ranges defined in the table, while the OTA CACLR requirement in table 9.7.3.3-4 shall apply in sub-block gaps for the frequency ranges defined in the table.

The CACLR in a sub-block gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges.

The assumed filter for the adjacent channel frequency is defined in table 9.7.3.3-4 and the filters on the assigned channels are defined in table 9.7.3.3-5.

For operation in non-contiguous spectrum, the CACLR for NR carriers located on either side of the sub-block gap shall be higher than the value specified in table 9.7.3.3-4.

Table 9.7.3.3-1: BS type 2-O ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400, 800, 1600, TBD | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) 21 (Note 5) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz NOTE 5: Applicable to bands defined within the frequency spectrum range of 52.6 – 71.0 GHz |  |  |  |  |

{Editor’s note: Additions required to account for unlicensed operation within the frequency range 52 to 71 GHz}

Table 9.7.3.3-2: BS type 2-O ACLR absolute limit

| BS class | ACLR absolute limit |
| --- | --- |
| Wide area BS | -13 dBm/MHz |
| Medium range BS | -20 dBm/MHz |
| Local area BS | -20 dBm/MHz |

Table 9.7.3.3-3: BS type 2-O ACLR limit in non-contiguous spectrum

| BS channel bandwidth of lowest/highest carrier transmitted (MHz) | Sub-block gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 50, 100 | Wgap≥ 100 (Note 5) Wgap≥ 250 (Note 6) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| 200, 400 | Wgap≥ 400 (Note 6) Wgap≥ 250 (Note 5) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 5: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 50 or 100 MHz. NOTE 6: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 200 or 400 MHz. |  |  |  |  |  |

Table 9.7.3.3-4: BS type 2-O CACLR limit in non-contiguous spectrum

| BS channel bandwidth of lowest/highest carrier transmitted (MHz) |  | Sub-block gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- | --- | --- |
| 50, 100 |  | 50 ≤Wgap< 100 (Note 5) 50 ≤Wgap< 250 (Note 6) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| 200, 400 |  | 200 ≤Wgap< 400 (Note 6) 200 ≤Wgap< 250 (Note 5) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 5: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 50 or 100 MHz. NOTE 6: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 200 or 400 MHz. |  |  |  |  |  |  |

{Editor’s note: Additions required to account for unlicensed operation within the frequency range 52 to 71 GHz}

Table 9.7.3.3-4a: BS type 2-O CACLR absolute limit

| BS class | CACLR absolute limit |
| --- | --- |
| Wide area BS | -13 dBm/MHz |
| Medium range BS | -20 dBm/MHz |
| Local area BS | -20 dBm/MHz |

Table 9.7.3.3-5: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| NR | NR of same BW with SCS that provides largest transmission bandwidth configuration |

### 9.7.4	OTA operating band unwanted emissions

#### 9.7.4.1	General

The OTA limits for operating band unwanted emissions are specified as TRP per RIB unless otherwise stated.

#### 9.7.4.2	Minimum requirement for BS type 1-O

Out-of-band emissions in FR1 are limited by OTA operating band unwanted emission limits. Unless otherwise stated, the operating band unwanted emission limits in FR1 are defined from ΔfOBUE below the lowest frequency of each supported downlink operating band up to ΔfOBUE above the highest frequency of each supported downlink operating band. The values of ΔfOBUE are defined in table 9.7.1-1 for the NR operating bands.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer's specification. For a RIB operating in multi-carrier or contiguous CA, the requirements apply to BS channel bandwidths of the outermost carrier for the frequency ranges defined in clause 6.6.4.1.

For a RIB operating in non-contiguous spectrum, the requirements shall apply inside any sub-block gap for the frequency ranges defined in clause 6.6.4.1.

For a multi-band RIB, the requirements shall apply inside any Inter RF Bandwidth gap for the frequency ranges defined in clause 6.6.4.1.

The OTA operating band unwanted emission requirement for BS type 1-O is that for each applicable basic limit in clause 6.6.4.2, the power of any unwanted emission shall not exceed an OTA limit specified as the basic limit + X, where X = 9 dB.

##### 9.7.4.2.1	Additional requirements

9.7.4.2.1.1	Protection of DTT

In certain regions the following requirement may apply for protection of DTT. For BS type 1-O operating in Band n20, the level of emissions in the band 470-790 MHz, measured in an 8 MHz filter bandwidth on centre frequencies Ffilter according to table 9.7.4.2.1.1-1, shall not exceed the maximum emission TRP level shown in the table. This requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain.

Table 9.7.4.2.1.1-1: Declared emissions levels for protection of DTT

| Case | Measurement filter centre frequency | Condition on BS maximum aggregate TRP / 10 MHz, PTRP_10MHz (NOTE) | Maximum level PTRP,N,MAX | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| A: for DTT frequencies where | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PTRP_10MHz 59 dBm | 0 dBm | 8 MHz |
| broadcasting is protected | N*8 + 306 MHz, 21 ≤ N ≤ 60 | 36 PTRP_10MHz < 59 dBm | PTRP_10MHz – 59 dBm | 8 MHz |
|  | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PTRP_10MHz < 36 dBm | -23 dBm | 8 MHz |
| B: for DTT frequencies where | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PTRP_10MHz 59 dBm | 10 dBm | 8 MHz |
| broadcasting is subject to an | N*8 + 306 MHz, 21 ≤ N ≤ 60 | 36 PTRP_10MHz < 59 dBm | PTRP_10MHz – 49 dBm | 8 MHz |
| intermediate level of protection | N*8 + 306 MHz, 21 ≤ N ≤ 60 | PTRP_10MHz < 36 dBm | -13 dBm | 8 MHz |
| C: for DTT frequencies where broadcasting is not protected | N*8 + 306 MHz, 21 ≤ N ≤ 60 | N/A | 22 dBm | 8 MHz |
| NOTE: PTRP_10MHz (dBm) is defined by PTRP_10MHz = P10MHz + Gant + 9dB, where Gant is 17 dBi. |  |  |  |  |

9.7.4.2.1.2	Limits in FCC Title 47

The BS may have to comply with the applicable emission limits established by FCC Title 47 [8], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer.

#### 9.7.4.3	Minimum requirement for BS type 2-O

##### 9.7.4.3.1	General

The requirements of either clause 9.7.4.3.2 (Category A limits) or clause 9.7.4.3.3 (Category B limits) shall apply. The application of either Category A or Category B limits shall be the same as for General OTA transmitter spurious emissions requirements (BS type 2-O) in clause 9.7.5.3.2. In addition, the limits in clause 9.7.4.3.4 may also apply.

Out-of-band emissions in FR2 are limited by OTA operating band unwanted emission limits. Unless otherwise stated, the OTA operating band unwanted emission limits in FR2 are defined from ΔfOBUE below the lowest frequency of each supported downlink operating band up to ΔfOBUE above the highest frequency of each supported downlink operating band. The values of ΔfOBUE are defined in table 9.7.1-1 for the NR operating bands.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer's specification. For a RIB operating in multi-carrier or contiguous CA, the requirements apply to the frequencies (ΔfOBUE) starting from the edge of the contiguous transmission bandwidth. In addition, for a RIB operating in non-contiguous spectrum, the requirements apply inside any sub-block gap.

Emissions shall not exceed the maximum levels specified in the tables below, where:

-	f is the separation between the contiguous transmission bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the contiguous transmission bandwidth edge.

-	f_offset is the separation between the contiguous transmission bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency ΔfOBUE outside the downlink operating band, where ΔfOBUE is defined in table 9.7.1-1.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

In addition, inside any sub-block gap for a RIB operating in non-contiguous spectrum, emissions shall not exceed the cumulative sum of the limits specified for the adjacent sub-blocks on each side of the sub-block gap. The limit for each sub-block is specified in clauses 9.7.4.3.2 and 9.7.4.3.3 below, where in this case:

-	f is the separation between the sub-block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub-block edge.

-	f_offset is the separation between the sub-block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub-block gap bandwidth minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

##### 9.7.4.3.2	OTA operating band unwanted emission limits (Category A)

BS unwanted emissions shall not exceed the maximum levels specified in table 9.7.4.3.2-1 and 9.7.4.3.2-2.

Table 9.7.4.3.2-1: OBUE limits applicable in the frequency range 24.25 – 33.4 GHz

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 35 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. |  |  |  |

Table 9.7.4.3.2-2: OBUE limits applicable in the frequency range 37 – 52.6 GHz

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 33 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 41 dB, -20 dBm)) | 1 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. |  |  |  |

Table 9.7.4.3.2-3: Void

##### 9.7.4.3.3	OTA operating band unwanted emission limits (Category B)

BS unwanted emissions shall not exceed the maximum levels specified in table 9.7.4.3.3-1 or 9.7.4.3.3-2.

Table 9.7.4.3.3-1: OBUE limits applicable in the frequency range 24.25 – 33.4 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 35 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 33 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

Table 9.7.4.3.3-2: OBUE limits applicable in the frequency range 37 – 52.6 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 33 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 41 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

{Editor’s note: Additions required to account for unlicensed operation within the frequency range 52 to 71 GHz. A new subclause 9.7.4.3.3A is required to account for masks relevant for unlicensed operating band.}

##### 9.7.4.3.4	Additional OTA operating band unwanted emission requirements

9.7.4.3.4.1	Protection of Earth Exploration Satellite Service

For BS operating in the frequency range 24.25 – 27.5 GHz, the power of unwanted emission shall not exceed the limits in table 9.7.4.3.4.1-1.

Table 9.7.4.3.4.1-1: OBUE limits for protection of Earth Exploration Satellite Service

| Frequency range | Limit | Measurement Bandwidth |
| --- | --- | --- |
| 23.6 – 24 GHz | -3 dBm (Note 1) | 200 MHz |
| 23.6 – 24 GHz | -9 dBm (Note 2) | 200 MHz |
| NOTE 1: This limit applies to BS brought into use on or before 1 September 2027. NOTE 2: This limit applies to BS brought into use after 1 September 2027. |  |  |

### 9.7.5	OTA transmitter spurious emissions

#### 9.7.5.1	General

Unless otherwise stated, all requirements are measured as mean power.

The OTA spurious emissions limits are specified as TRP per RIB unless otherwise stated.

#### 9.7.5.2	Minimum requirement for BS type 1-O

##### 9.7.5.2.1	General

The OTA transmitter spurious emission limits for FR1 shall apply from 30 MHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in table 9.7.1-1. For some FR1 operating bands, the upper limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329 [2].

For multi-band RIB each supported operating band and ΔfOBUE MHz around each band are excluded from the OTA transmitter spurious emissions requirements.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification.

BS type 1-O requirements consists of OTA transmitter spurious emission requirements based on TRP and co-location requirements not based on TRP.

##### 9.7.5.2.2	General OTA transmitter spurious emissions requirements

The Tx spurious emissions requirements for BS type 1-O are that for each applicable basic limit above 30 MHz in clause 6.6.5.2.1, the TRP of any spurious emission shall not exceed an OTA limit specified as the basic limit + X, where X = 9 dB, unless stated differently in regional regulation.

##### 9.7.5.2.3	Protection of the BS receiver of own or different BS

This requirement shall be applied for NR FDD operation in order to prevent the receivers of own or a different BS of the same band being desensitised by emissions from a type 1-O BS.

This requirement is a co-location requirement as defined in clause 4.9, the power levels are specified at the co-location reference antenna output.

The total power of any spurious emission from both polarizations of the co-location reference antenna connector output shall not exceed the basic limits in clause 6.6.5.2.2 + X dB, where X = -21 dB.

##### 9.7.5.2.4	Additional spurious emissions requirements

These requirements may be applied for the protection of systems operating in frequency ranges other than the BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the BS, or they may be set by local or regional regulation as a mandatory requirement for an NR operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in clause 4.5.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM, CDMA, UTRA, E-UTRA, NR, etc.). The Tx additional spurious emissions requirements for BS type 1-O are that for each applicable basic limit in clause 6.6.5.2.3, the TRP of any spurious emission shall not exceed an OTA limit specified as the basic limit + X, where X = 9 dB.

##### 9.7.5.2.5	Co-location with other base stations

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a BS.

The requirements assume co-location with base stations of the same class.

NOTE:	For co-location with UTRA, the requirements are based on co-location with UTRA FDD or TDD base stations.

This requirement is a co-location requirement as defined in clause 4.9, the power levels are specified at the co-location reference antenna output(s).

The power sum of any spurious emission is specified over all supported polarizations at the output(s) of the co-location reference antenna and shall not exceed the basic limits in clause 6.6.5.2.4 + X dB, where X = -21 dB.

For a multi-band RIB, the exclusions and conditions in the notes column of table 6.6.5.2.4-1 apply for each supported operating band.

#### 9.7.5.3	Minimum requirement for BS type 2-O

##### 9.7.5.3.1	General

In FR2, the OTA transmitter spurious emission limits apply from 30 MHz to 2nd harmonic of the upper frequency edge of the downlink operating band, excluding the frequency range from ΔfOBUE below the lowest frequency of the downlink operating band, up to ΔfOBUE above the highest frequency of the downlink operating band, where the ΔfOBUE is defined in table 9.7.1-1.

##### 9.7.5.3.2	General OTA transmitter spurious emissions requirements

9.7.5.3.2.1	General

The requirements of either clause 9.7.5.3.2.2 (Category A limits) or clause 9.7.5.3.2.3 (Category B limits) shall apply. The application of either Category A or Category B limits shall be the same as for Operating band unwanted emissions in clause 9.7.4.3.

Table 9.7.5.3.2-1: Void

NOTE:	Table 9.7.5.3.2-1 is moved to clause 9.7.5.3.2.2 as Table 9.7.5.3.2.2-1.

9.7.5.3.2.2	OTA transmitter spurious emissions (Category A)

The power of any spurious emission shall not exceed the limits in table 9.7.5.3.2-1

Table 9.7.5.3.2.2-1: BS radiated Tx spurious emission limits in FR2

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band | -13 dBm | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [2], s4.1 NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. |  |  |  |

9.7.5.3.2.3	OTA transmitter spurious emissions (Category B)

The power of any spurious emission shall not exceed the limits in table 9.7.5.3.2.3-1.

Table 9.7.5.3.2.3-1: BS radiated Tx spurious emission limits in FR2 (Category B)

| Frequency range (Note 4) | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz 18 GHz | -30 dBm | 1 MHz | Note 1 |
| 18 GHz Fstep,1 | -20 dBm | 10 MHz | Note 2 |
| Fstep,1 Fstep,2 | -15 dBm | 10 MHz | Note 2 |
| Fstep,2 Fstep,3 | -10 dBm | 10 MHz | Note 2 |
| Fstep,4 Fstep,5 | -10 dBm | 10 MHz | Note 2 |
| Fstep,5 Fstep,6 | -15 dBm | 10 MHz | Note 2 |
| Fstep,6 2nd harmonic of the upper frequency edge of the DL operating band | -20 dBm | 10 MHz | Note 2, Note 3 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [2], s4.1 NOTE 2: Limit and bandwidth as in ERC Recommendation 74-01 [19], Annex 2. NOTE 3: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 4: The step frequencies Fstep,X are defined in Table 9.7.5.3.2.3-2. |  |  |  |

Table 9.7.5.3.2.3-2: Step frequencies for defining the BS radiated Tx spurious emission limits in FR2 (Category B)

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| nxxx | TBD | TBD | TBD | TBD | TBD | TBD |
| nyyy | TBD | TBD | TBD | TBD | TBD | TBD |
| NOTE 1: Fstep,X are based on ERC Recommendation 74-01 [19], Annex 2. NOTE 2: Fstep,3 and Fstep,4 are aligned with the values for ΔfOBUE in Table 9.7.1-1. |  |  |  |  |  |  |

##### 9.7.5.3.3	Additional OTA transmitter spurious emissions requirements

These requirements may be applied for the protection of systems operating in frequency ranges other than the BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the BS, or they may be set by local or regional regulation as a mandatory requirement for an NR operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in clause 4.5.

9.7.5.3.3.1	Limits for protection of Earth Exploration Satellite Service

For BS operating in the frequency range 24.25 – 27.5 GHz, the power of any spurious emissions shall not exceed the limits in Table 9.7.5.3.3.1-1.

Table 9.7.5.3.3.1-1: Limits for protection of Earth Exploration Satellite Service

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 23.6 – 24 GHz | -3 dBm | 200 MHz | Note 1 |
| 23.6 – 24 GHz | -9 dBm | 200 MHz | Note 2 |
| NOTE 1: This limit applies to BS brought into use on or before 1 September 2027. NOTE 2: This limit applies to BS brought into use after 1 September 2027. |  |  |  |

## 9.8	OTA transmitter intermodulation

### 9.8.1	General

The OTA transmitter intermodulation requirement is a measure of the capability of the transmitter unit to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station. The requirement shall apply during the transmitter ON period and the transmitter transient period.

The requirement shall apply at each RIB supporting transmission in the operating band.

The transmitter intermodulation level is the total radiated power of the intermodulation products when an interfering signal is injected into the co-location reference antenna.

The OTA transmitter intermodulation requirement is not applicable for BS type 2-O.

### 9.8.2	Minimum requirement for BS type 1-O

For BS type 1-O the transmitter intermodulation level shall not exceed the TRP unwanted emission limits specified for OTA transmitter spurious emission in clause 9.7.5.2 (except clause 9.7.5.2.3 and clause 9.7.5.2.5), OTA operating band unwanted emissions in clause 9.7.4.2 and OTA ACLR in clause 9.7.3.2 in the presence of a wanted signal and an interfering signal, defined in table 9.8.2-1.

The requirement is applicable outside the Base Station RF Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum, the requirement is also applicable inside a sub-block gap for interfering signal offsets where the interfering signal falls completely within the sub-block gap. The interfering signal offset is defined relative to the sub-block edges.

For RIBs supporting operation in multiple operating bands, the requirement shall apply relative to the Base Station RF Bandwidth edges of each operating band. In case the inter RF Bandwidth gap is less than 3*BWChannel (where BWChannel is the minimal BS channel bandwidth of the band), the requirement in the gap shall apply only for interfering signal offsets where the interfering signal falls completely within the inter RF Bandwidth gap.

Table 9.8.2-1: Interfering and wanted signals for
the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR signal or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 |
| Interfering signal level | The interfering signal level is the same power level as the BS (Prated,t,TRP) fed into a co-location reference antenna. |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the RIB are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-2 [6] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. NOTE 3: The Prated,t,TRP is split between polarizations at the co-location reference antenna. |  |