3GPP TSG-RAN WG4 Meeting#99      	R4-2110141

E-meeting, 19th – 27th  May, 2021

Title:	1024QAM simulation assumptions and preliminary results

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	9.6.2.1

Document for:	Approval

# 1	Introduction

This contribution relates to a work item “Introduction of DL 1024QAM for NR FR1” [1].

In this contribution we propose simulation assumptions for further work and show preliminary simulation results.

# 2	Discussion

Two way forwards were agreed in RAN4 #98-bis-e [2] and [3]. Both of them suggested to use parameters in [4] as a starting point for simulations assumptions for the work item.

Table 1 contains simulation parameters based on [4] with some changes proposed in red color. It has been agreed in [3] to use wider bandwidth than 20MHz and hence we propose 40MHz. We also trying to keep the number of combinations in parameters at reasonable level. In order to do so we propose to include only random precoding and leave non-HARQ case also out. We propose to agree the simulation assumption in Table 1.

Proposal: Agree simulation assumptions in Table 1 for further work in 1024QAM work item in RAN4.

Table 1 Proposed simulation parameters (based on R4-2104726)

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| CBW | 2040 MHz |
| SCS | 15kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, Maximum Doppler frequency: 5Hz TDL-D 10ns delay spread, Maximum Doppler frequency: 5Hz |
| MCS | 256QAM: MCS 24 in TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH, and other MCSs are not precluded 1024QAM: MCS 24 in the following Table accroding to the agreement in RAN1 #104, and other MCSs are not precluded MCS Index; Modulation Order; Target code Rate R x [1024]; Spectral. IMCS; Qm; efficiency. 23; 10; 805.5; 7.8662. 24; 10; 853; 8.3301. 25; 10; 900.5; 8.7939. 26; 10; 948; 9.2578. |
| Precoding | Random precoding Precoding configuration defined in 38.101-4 Section 7.2 for fading channels; follow PMI |
| Symbol type | CP-OFDM |
| HARQ | 8, None |
| RANK | 1 and 2 |
| BS antenna configuration | 1 and 2 |
| UE antenna configuration | 4 |
| Antenna correlation (Tx and Rx) | Low correlation |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS (it should be notified in contribution which one is used) |
| txEVM | 2%, 2.5%, 3% |
| rxEVM | 0.5%, 1%, 2%, 3%, 4% |

Preliminary simulations results are shown in Figure 1. These simulations are following the parameters shown in Table 1. Results in Figure 1 and Figure 2 have 1 DMRS symbol and 2.5% and 3% Tx EVM, respectively. Results in Figure 3 and Figure 4 are similar with 2 DMRS symbols. Simulation results show that 1024QAM achieves better throughput than 256QAM at reasonable SNR. Also the results use the EVMs proposed in simulations assumptions which seem to provide adequate performance.

Observation: Initial simulation results for 1024QAM show higher throughput compared to 256QAM for higher, but still reasonable SNR conditions and reasonable EVM requirements

Figure 1. Simulation results for TDL-A 10ns and Tx EVM 2.5%.

Figure 2. Simulation results for TDL-A 10ns and Tx EVM 3%.

Figure 3. Simulation results for TDL-A 10ns, 2DMRS and Tx EVM 2.5%.

Figure 4. Simulation results for TDL-A 10ns, 2DMRS and Tx EVM 3%.

# 3	Conclusion

In this paper we have proposed simulation assumptions for further work and shown preliminary results for link level simulations. We have also made the proposal and observation below.

Proposal: Agree simulation assumptions in Table 1 for further work in 1024QAM work item in RAN4.

Observation: Initial simulation results for 1024QAM show higher throughput compared to 256QAM for higher, but still reasonable SNR conditions and reasonable EVM requirements

# 4 	References

- RP-202886, “Revised WID on Introduction of DL 1024QAM for NR FR1”, Nokia

- R4-2105416, “WF on UE RF requirements for DL 1024QAM”, Ericsson

- R4-2106121, “WF on 1024QAM BS RF requirements”, Ericsson

- R4-2104726, “Link level simulation results for 1024QAM for NR FR1”, CATT