3GPP TSG-RAN WG4 Meeting #99-e 	R4-2110187

Online meeting, 19-27 May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0814 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-1 to correct some errors in Delta TIB and Delta RIB table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Correct some errors in Delta TIB and Delta RIB table In LTE, considering cross band isolation issues for CA_3-41 and CA_25-41, the Delta TIB and Delta RIB of band 41 had been defined different values based on different frequency ranges and used 2545MHz as the frequency division point NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. In NR, resued the LTE method for CA between FDD bands (band n3, n25, n66) and band n41, but CA_n3-n41 used different frequency division point according to the frequency Operators occupied, therefore - CA_n3-n41 used 2515MHz as the frequency division point: NOTE 4: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 5: Applicable for the frequency range of 2496 – 2515 MHz. - CA_n25/n66-n41 still used 2545 as the frequency division point: NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. But currently, the Delta TIB and Delta RIB values for some higher order CA combinations including CA_n3-n41, CA_n25/n66-n41 are mistake, that should also follow above rules for n41, i.e., - The higher order DC including CA_n3-n41 should use 2515MHz as the frequency division point. - The higher order DC including CA_n25/n66_n41 should use 2545MHz as the frequency division point. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct below notations - Correct the notation of CA n8-n75, CA n8-n78 and CA n77-n79 to CA_n8-n75, CA_n8-n78 and CA_n77-n79 - Correct some superscript for band n41. - Adjust the order for some CA cases. - Delete the row of 0 ΔRIB,c in Table 7.3A.3.2.3-1: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The nodation and some superscript for band n41 have some mistakes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

6.2A.4.2.3	ΔTIB,c for Inter-band CA (two bands)

Table 6.2A.4.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3 | n1 | 0.3 |
|  | n3 | 0.3 |
| CA_n1-n7 | n1 | 0.5 |
|  | n7 | 0.6 |
| CA_n1-n8 | n1 | 0.3 |
|  | n8 | 0.3 |
| CA_n1-n28 | n1 | 0.3 |
|  | n28 | 0.6 |
| CA_n1-n40 | n1 | 0.5 |
|  | n40 | 0.5 |
| CA_n1-n41 | n1 | 0.5 |
|  | n41 | 0.5 |
| CA_n1-n77 | n1 | 0.6 |
|  | n77 | 0.8 |
| CA_n1-n78 | n1 | 0.3 |
|  | n78 | 0.8 |
| CA_n2-n5 | n2 | 0.3 |
|  | n5 | 0.3 |
| CA_n2-n48 | n2 | 0.6 |
|  | n48 | 0.8 |
| CA_n2-n66 | n2 | 0.5 |
|  | n66 | 0.5 |
| CA_n2-n77 | n2 | 0.6 |
|  | n77 | 0.8 |
| CA_n2-n78 | n2 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n7 | n3 | 0.5 |
|  | n7 | 0.5 |
| CA_n3-n8 | n3 | 0.3 |
|  | n8 | 0.3 |
| CA_n3-n18 | n3 | 0.3 |
|  | n18 | 0.3 |
| CA_n3-n28 | n3 | 0.3 |
|  | n28 | 0.3 |
| CA_n3-n38 | n3 | 0.5 |
|  | n38 | 0.5 |
| CA_n3-n40 | n3 | 0.5 |
|  | n40 | 0.5 |
| CA_n3-n41 | n3 | 0.5 |
|  | n41 | 0.34 |
|  |  | 0.85 |
| CA_n3-n77 | n3 | 0.6 |
|  | n77 | 0.8 |
| CA_n3-n78 | n3 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n79 | n3 | 0.3 |
|  | n79 | 0.8 |
| CA_n5-n7 | n5 | 0.3 |
|  | n7 | 0.3 |
| CA_n5-n25 | n5 | 0.3 |
|  | n25 | 0.3 |
| CA_n5-n48 | n5 | 0.3 |
|  | n48 | 0.3 |
| CA_n5-n66 | n5 | 0.3 |
|  | n66 | 0.3 |
| CA_n5-n77 | n5 | 0.6 |
|  | n77 | 0.8 |
| CA_n5-n78 | n5 | 0.6 |
|  | n78 | 0.8 |
| CA_n7-n25 | n7 | 0.5 |
|  | n25 | 0.5 |
| CA_n7-n28 | n7 | 0.3 |
|  | n28 | 0.3 |
| CA_n7-n66 | n7 | 0.5 |
|  | n66 | 0.5 |
| CA_n7-n77 | n7 | 0.5 |
|  | n77 | 0.8 |
| CA_n7-n78 | n7 | 0.5 |
|  | n78 | 0.8 |
| CA_n8-n20 | n8 | 0.4 |
|  | n20 | 0.4 |
| CA_n8-n28 | n8 | 0.6 |
|  | n28 | 0.5 |
| CA_n8-n39 | n8 | 0.3 |
|  | n39 | 0.3 |
| CA_n8-n40 | n8 | 0.3 |
|  | n40 | 0.3 |
| CA_n8-n41 | n8 | 0.6 |
|  | n41 | 0.3 |
| CA_n8-n75 | n8 | 0.3 |
| CA_n8-n78 | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n8-n79 | n8 | 0.3 |
|  | n79 | 0.8 |
| CA_n13-n25 | n13 | 0.3 |
|  | n25 | 0.3 |
| CA_n13-n66 | n13 | 0.3 |
|  | n66 | 0.3 |
| CA_n18-n41 | n18 | 0.3 |
|  | n41 | 0.3 |
| CA_n20-n28 | n20 | 0.5 |
|  | n28 | 0.5 |
| CA_n20-n75 | n20 | 0.3 |
| CA_n20-n78 | n20 | 0.6 |
|  | n78 | 0.8 |
| CA_n25-n29 | n25 | 0.3 |
| CA_n25-n38 | n25 | 0.5 |
|  | n38 | 0.5 |
| CA_n25-n41 | n25 | 0.5 |
|  | n41 | 0.46 |
|  |  | 0.97 |
| CA_n25-n48 | n25 | 0.6 |
|  | n48 | 0.8 |
| CA_n25-n66 | n25 | 0.5 |
|  | n66 | 0.5 |
| CA_n25-n71 | n25 | 0.3 |
|  | n71 | 0.6 |
| CA_n25-n77 | n25 | 0.6 |
|  | n77 | 0.8 |
| CA_n28-n40 | n28 | 0.3 |
|  | n40 | 0.3 |
| CA_n28-n41 | n28 | 0.3 |
|  | n41 | 0.3 |
| CA_n28-n50 | n28 | 0.3 |
|  | n50 | 0.4 |
| CA_n28-n75 | n28 | 0.3 |
| CA_n28-n77 | n28 | 0.5 |
|  | n77 | 0.8 |
| CA_n28-n78 | n28 | 0.5 |
|  | n78 | 0.8 |
| CA_n28-n79 | n28 | 0.5 |
|  | n79 | 0.8 |
| CA_n29-n66 | n66 | 0.3 |
| CA_n29-n70 | n70 | 0.3 |
| CA_n34-n79 | n34 | 0.3 |
|  | n79 | 0.8 |
| CA_n38-n66 | n38 | 0.5 |
|  | n66 | 0.5 |
| CA_n38-n78 | n38 | 0.3 |
|  | n78 | 0.8 |
| CA_n39-n41 | n39 | 02 |
|  | n41 | 02 |
|  | n39 | 0.53 |
|  | n41 | 0.53 |
| CA_n39-n79 | n39 | 0.3 |
|  | n79 | 0.8 |
| CA_n40-n41 | n40 | 0.53 |
|  | n41 | 0.53 |
| CA_n40-n78 | n40 | 0 |
|  | n78 | 0.5 |
| CA_n40-n79 | n40 | 0.3 |
|  | n79 | 0.8 |
| CA_n41-n50 | n41 | 0.3 |
|  | n50 | 0.4 |
| CA_n41-n66 | n41 | 0.86 |
|  |  | 1.37 |
|  | n66 | 0.5 |
| CA_n41-n71 | n41 | 0.3 |
|  | n71 | 0.6 |
| CA_n41-n771 | n41 | 0.3 |
|  | n77 | 0.8 |
| CA_n41-n781 | n41 | 0.3 |
|  | n78 | 0.8 |
| CA_n41-n79 | n41 | 0.3 |
|  | n79 | 0.8 |
| CA_n48-n66 | n48 | 0.8 |
|  | n66 | 0.6 |
| CA_n50-n78 | n50 | 02 |
|  | n78 | 02 |
|  | n50 | 0.53 |
|  | n78 | 0.53 |
| CA_n66-n70 | n66 | 0.5 |
|  | n70 | 0.5 |
| CA_n66-n71 | n66 | 0.3 |
|  | n71 | 0.3 |
| CA_n66-n77 | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n66-n78 | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n70-n71 | n70 | 0.3 |
|  | n71 | 0.6 |
| CA_n71-n77 | n71 | 0.5 |
|  | n77 | 0.8 |
| CA_n71-n78 | n71 | 0.5 |
|  | n78 | 0.8 |
| CA_n75-n78 | n78 | 0.8 |
| CA_n76-n78 | n78 | 0.8 |
| CA_n77-n79 | n77 | 0.5 |
|  | n79 | 0.5 |
| CA_n78-n79 | n78 | 0.5 |
|  |  | 1.58 |
|  | n79 | 0.5 |
|  |  | 1.58 |
| CA_n78-n92 | n78 | 0.8 |
|  | n92 | 0.6 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

Table 6.2A.4.2.3-2: Void

Table 6.2A.4.2.3-3: Void

6.2A.4.2.4	ΔTIB,c for Inter-band CA (three bands)

Table 6.2A.4.2.4-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n7 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n7 | 0.6 |
| CA_n1-n3-n8 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n8 | 0.3 |
| CA_n1-n3-n28 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n28 | 0.6 |
| CA_n1-n3-n41 | n1 | 0.5 |
|  | n3 | 0.5 |
|  | n41 | 0.31 |
|  |  | 0.82 |
| CA_n1-n3-n78 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n7-n28 | n1 | 0.5 |
|  | n7 | 0.6 |
|  | n28 | 0.6 |
| CA_n1-n7-n78 | n1 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n8-n78 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n8-n79 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n79 | 0.8 |
| CA_n1-n28-n78 | n1 | 0.3 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n40-n78 | n1 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |
| CA_n1-n77-n79 | n1 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |
| CA_n1-n78-n79 | n1 | 0.3 |
|  | n78 | 0.8 |
|  |  | 1.57 |
|  | n79 | 0.5 |
|  |  | 1.57 |
| CA_n2-n66-n77 | n2 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n3-n5-n7 | n3 | 0.5 |
|  | n5 | 0.3 |
|  | n7 | 0.5 |
| CA_n3-n7-n28 | n3 | 0.5 |
|  | n7 | 0.5 |
|  | n28 | 0.3 |
| CA_n3-n7-n78 | n3 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n8-n78 | n3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n18-n41 | n3 | 0.5 |
|  | n18 | 0.3 |
|  | n41 | 0.31/0.82 |
| CA_n3-n28-n77 | n3 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| CA_n3-n28-n78 | n3 | 0.5 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| CA_n3-n40-n41 | n3 | 0.5 |
|  | n40 | 0.5 |
|  | n41 | 0.51,3 |
|  |  | 0.82,3 |
| CA_n3-n41-n77 | n3 | 0.6 |
|  | n41 | 0.31/0.82 |
|  | n77 | 0.8 |
| CA_n3-n41-n78 | n3 | 0.6 |
|  | n41 | 0.31/0.82 |
|  | n78 | 0.8 |
| CA_n3-n41-n79 | n3 | 0.3 |
|  | n41 | 0.31 |
|  |  | 0.82 |
|  | n79 | 0.8 |
| CA_n5-n7-n78 | n5 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| CA_n5-n25-n66 | n5 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |
| CA_n5-n25-n77 | n5 | 0.6 |
|  | n25 | 0.6 |
|  | n77 | 0.8 |
| CA_n5-n25-n78 | n5 | 0.6 |
|  | n25 | 0.6 |
|  | n78 | 0.8 |
| CA_n5-n66-n77 | n5 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n5_n66-n78 | n5 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n7_n25-n66 | n7 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |
| CA_n7_n28-n78 | n7 | 0.3 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| CA_n7_n66-n78 | n7 | 0.5 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n8-n39-n41 | n8 | 0.6 |
|  | n39 | 0.54 |
|  | n41 | 0.54 |
| CA_n8-n40-n41 | n8 | 0.3 |
|  | n40 | 0.33 |
|  | n41 | 0.33 |
| CA_n8-n41-n79 | n8 | 0.6 |
|  | n41 | 0.3 |
|  | n79 | 0.8 |
| CA_n8-n78-n79 | n8 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.8 |
| CA_n13-n25-n66 | n13 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |
| CA_n20-n28-n78 | n20 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
| CA_n25-n29-n66 | n25 | 0.5 |
|  | n29 | 0 |
|  | n66 | 0.5 |
| CA_n25-n38-n78 | n25 | 0.5 |
|  | n38 | 0.4 |
|  | n78 | 0.8 |
| CA_n25-n41-n66 | n25 | 0.5 |
|  | n41 | 0.85 |
|  |  | 1.36 |
|  | n66 | 0.5 |
| CA_n25-n41-n71 | n25 | 0.5 |
|  | n41 | 0.5 |
|  | n71 | 0.6 |
| CA_n25-n41-n77 | n25 | 0.5 |
|  | n41 | 0.5 |
|  | n77 | 0.6 |
| CA_n25-n48-n66 | n25 | 0.6 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |
| CA_n25-n66-n71 | n25 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.6 |
| CA_n25-n66-n77 | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n25-n66-n78 | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n25-n71-n77 | n25 | 0.6 |
|  | n71 | 0.6 |
|  | n77 | 0.8 |
| CA_n28-n40-n78 | n28 | 0.5 |
|  | n40 | 0.3 |
|  | n78 | 0.8 |
| CA_n28-n41-n77 | n28 | 0.5 |
|  | n41 | 0.3 |
|  | n77 | 0.8 |
| CA_n28-n41-n78 | n28 | 0.5 |
|  | n41 | 0.3 |
|  | n78 | 0.8 |
| CA_n28-n77-n79 | n28 | 0.5 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |
| CA_n28-n78-n79 | n28 | 0.5 |
|  | n78 | 0.8 |
|  |  | 1.57 |
|  | n79 | 0.5 |
|  |  | 1.57 |
| CA_n29-n66-n70 | n29 | 0 |
|  | n66 | 0.5 |
|  | n70 | 0.5 |
| CA_n38-n66-n78 | n38 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.8 |
| CA_n39-n40-n41 | n39 | 0.3 |
|  | n40 | 0.3 |
|  | n41 | 0.3 |
| CA_n39-n40-n79 | n39 | 0.3 |
|  | n40 | 0 |
|  | n79 | 0.8 |
| CA_n39-n41-n79 | n39 | 0.3 |
|  | n41 | 0.34 |
|  | n79 | 0.84 |
| CA_n40-n41-n79 | n40 | 0.53 |
|  | n41 | 0.53 |
|  | n79 | 0.8 |
| CA_n41-n66-n71 | n41 | 0.85 |
|  |  | 1.36 |
|  | n66 | 0.5 |
|  | n71 | 0.3 |
| CA_n41-n66-n77 | n41 | 0.5 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n41-n71-n77 | n41 | 0.3 |
|  | n71 | 0.5 |
|  | n77 | 0.8 |
| CA_n66-n70-n71 | n66 | 0.5 |
|  | n70 | 0.5 |
|  | n71 | 0.6 |
| CA_n66-n71-n77 | n66 | 0.6 |
|  | n71 | 0.6 |
|  | n77 | 0.8 |
| CA_n66-n71-n78 | n66 | 0.6 |
|  | n71 | 0.5 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and 41. NOTE 4: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx between n39 and n41. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 7: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

6.2A.4.2.5	ΔTIB,c for Inter-band CA (four bands)

Table 6.2A.4.2.5-1: ΔTIB,c due to NR CA (four bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n7-n28 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n7 | 0.6 |
|  | n28 | 0.6 |
| CA_n1-n3-n7-n78 | n1 | 0.7 |
|  | n3 | 0.7 |
|  | n7 | 0.7 |
|  | n78 | 0.8 |
| CA_n1-n3-n8-n78 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n1-n3-n28-n78 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n5-n7-n78 | n3 | 0.6 |
|  | n5 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n7-n28-n78 | n3 | 0.6 |
|  | n7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.6 |
| CA_n3-n28-n41-n77 | n3 | 1 |
|  | n28 | 0.5 |
|  | n41 | 0.31/0.82 |
|  | n77 | 0.8 |
| CA_n3-n28-n41-n78 | n3 | 1 |
|  | n28 | 0.5 |
|  | n41 | 0.31/0.82 |
|  | n78 | 0.8 |
| CA_n7-n25-n66-n78 | n7 | 0.5 |
|  | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n25-n41-n66-n71 | n25 | 0.5 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.3 |
| CA_n41-n66-n71-n77 | n41 | 0.33/0.84 |
|  | n66 | 1 |
|  | n71 | 0.5 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: Applicable for the frequency range of 2545-2690 MHz. NOTE 4: Applicable for the frequency range of 2496-2545 MHz. |  |  |

< end of changes >

< start of changes >

7.3A.3.2.1	ΔRIB,c for two bands

Table 7.3A.3.2.1-1: ΔRIB,c due to CA (two bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n28 | n28 | 0.2 |
| CA_n1-n77 | n1 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n78 | n78 | 0.5 |
| CA_n2-n48 | n2 | 0.2 |
|  | n48 | 0.5 |
| CA_n2-n66 | n2 | 0.3 |
|  | n66 | 0.3 |
| CA_n2-n77 | n2 | 0.2 |
|  | n77 | 0.5 |
| CA_n2-n78 | n2 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n41 | n41 | 04 |
|  |  | 0.55 |
| CA_n3-n77 | n3 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n78 | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n79 | n79 | 0.5 |
| CA_n5-n77 | n5 | 0.2 |
|  | n77 | 0.5 |
| CA_n5-n78 | n5 | 0.2 |
|  | n78 | 0.5 |
| CA_n7-n66 | n7 | 0.5 |
|  | n66 | 0.5 |
| CA_n7-n77 | n77 | 0.5 |
| CA_n7-n78 | n7 | 0.5 |
|  | n78 | 0.5 |
| CA_n8-n28 | n8 | 0.2 |
|  | n28 | 0.2 |
| CA_n8-n78 | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n8-n79 | n79 | 0.5 |
| CA_n20-n78 | n78 | 0.5 |
| CA_n25-n48 | n25 | 0.2 |
|  | n48 | 0.5 |
| CA_n25-n66 | n25 | 0.3 |
|  | n66 | 0.3 |
| CA_n25-n71 | n71 | 0.3 |
| CA_n25-n77 | n25 | 0.2 |
|  | n77 | 0.5 |
| CA_n25-n78 | n25 | 0.2 |
|  | n78 | 0.5 |
| CA_n28-n75 | n28 | 0.2 |
| CA_n28-n77 | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n28-n79 | n28 | 0.2 |
|  | n79 | 0.5 |
| CA_n34-n79 | n79 | 0.5 |
| CA_n38-n66 | n38 | 0.5 |
|  | n66 | 0.5 |
| CA_n38-n78 | n38 | 0.4 |
|  | n78 | 0.5 |
| CA_n39-n40 | n39 | 0.3 |
|  | n40 | 0.3 |
| CA_n39-n41 | n39 | 0.22 |
|  | n41 | 0.22 |
|  | n39 | 0.23 |
|  | n41 | 0.23 |
| CA_n39-n79 | n79 | 0.5 |
| CA_n40-n78 | n40 | 0.4 |
|  | n78 | 0.5 |
| CA_n40-n79 | n79 | 0.5 |
| CA_n41-n66 | n41 | 0.56 |
|  |  | 17 |
|  | n66 | 0.5 |
| CA_n41-n71 | n71 | 0.2 |
| CA_n41-n771 | n77 | 0.5 |
| CA_n41-n781 | n78 | 0.5 |
| CA_n41-n79 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n48-n66 | n48 | 0.5 |
|  | n66 | 0.2 |
| CA_n50-n78 | n50 | 0.22 |
|  | n78 | 0.22 |
|  | n50 | 0.23 |
|  | n78 | 0.23 |
| CA_n66-n77 | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n66-n78 | n66 | 0.2 |
|  | n78 | 0.5 |
| CA_n71-n77 | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n71-n78 | n71 | 0.2 |
|  | n78 | 0.5 |
| CA_n75-n78 | n78 | 0.5 |
| CA_n76-n78 | n78 | 0.5 |
| CA_n78-n92 | n78 | 0.5 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz |  |  |

Table 7.3A.3.2.1-2: void

7.3A.3.2.2	Void

7.3A.3.2.3	ΔRIB,c for three bands

Table 7.3A.3.2.3-1: ΔRIB,c due to CA (three bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n28 | n28 | 0.2 |
| CA_n1-n3-n41 | n41 | 01 |
|  |  | 0.52 |
| CA_n1-n3-n78 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n7-n28 | n28 | 0.2 |
| CA_n1-n7-n78 | n1 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n8-n78 | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n8-n79 | n8 | 0.2 |
|  | n79 | 0.5 |
| CA_n1-n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n40-n78 | n78 | 0.5 |
| CA_n1-n77-n79 | n1 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n78-n79 | n78 | 0.5 |
| CA_n2-n66-n77 | n2 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n7-n78 | n3 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n8-n78 | n3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n18-n41 | n41 | 01/0.52 |
| CA_n3-n28-n41 | n41 | 01/0.52 |
| CA_n3-n28-n77 | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n40-n41 | n41 | 01,3 |
|  |  | 0.52,3 |
| CA_n3-n41-n77 | n3 | 0.2 |
|  | n41 | 01/0.52 |
|  | n77 | 0.5 |
| CA_n3-n41-n78 | n3 | 0.2 |
|  | n41 | 01/0.52 |
|  | n78 | 0.5 |
| CA_n3-n41-n79 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n5-n7-n78 | n5 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| CA_n5-n25-n77 | n5 | 0.2 |
|  | n25 | 0.2 |
|  | n77 | 0.5 |
| CA_n5-n25-n78 | n5 | 0.2 |
|  | n25 | 0.2 |
|  | n78 | 0.5 |
| CA_n5-n66-n77 | n5 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n5-n66-n78 | n5 | 0.5 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |
| CA_n7-n25-n66 | n7 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |
| CA_n7-n28-n78 | n78 | 0.5 |
| CA_n7-n66-n78 | n7 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |
| CA_n8-n39-n41 | n39 | 0.24 |
|  | n41 | 0.24 |
| CA_n8-n41-n79 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n8-n78-n79 | n8 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0.5 |
| CA_n13-n25-n66 | n25 | 0.3 |
|  | n66 | 0.3 |
| CA_n20-n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n25-n29-n66 | n25 | 0.3 |
|  | n66 | 0.3 |
| CA_n25-n38-n78 | n25 | 0.2 |
|  | n38 | 0.4 |
|  | n78 | 0.5 |
| CA_n25-n41-n66 | n25 | 0.3 |
|  | n41 | 0.55 |
|  |  | 16 |
|  | n66 | 0.3 |
| CA_n25-n41-n71 | n71 | 0.2 |
| CA_n25-n48-n66 | n25 | 0.3 |
|  | n48 | 0.5 |
|  | n66 | 0.3 |
| CA_n25-n66-n71 | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n71 | 0.3 |
| CA_n25-n66-n78 | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n78 | 0.5 |
| CA_n25-n66-n77 | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |
| CA_n25-n71-n77 | n25 | 0.2 |
|  | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n28-n40-n78 | n78 | 0.5 |
| CA_n28-n41-n77 | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n28-n41-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n28-n77-n79 | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n28-n78-n79 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n39-n40-n79 | n39 | 0.3 |
|  | n40 | 0.3 |
|  | n79 | 0.5 |
| CA_n39-n41-n79 | n39 | 0.34 |
|  | n41 | 0.34 |
|  | n79 | 0.8 |
| CA_n40-n41-n79 | n40 | 03 |
|  | n41 | 0.53 |
|  | n79 | 0.5 |
| CA_n41-n66-n71 | n41 | 0.51 |
|  |  | 12 |
|  | n66 | 0.5 |
| CA_n41-n66-n77 | n41 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n41-n71-n77 | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n66-n71-n77 | n66 | 0.2 |
|  | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n66-n71-n78 | n66 | 0.2 |
|  | n71 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and 41. NOTE 4: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx between n39 and n41. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 7: Void. NOTE 8: Void. |  |  |

7.3A.3.2.4	ΔRIB,c for four bands

Table 7.3A.3.2.4-1: ΔRIB,c due to CA (four bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n7-n28 | n28 | 0.2 |
| CA_n1-n3-n7-n78 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.5 |
| CA_n1-n3-n8-n78 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n3-n28-n78 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n5-n7-n78 | n3 | 0.2 |
|  | n5 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n7-n28-n78 | n3 | 0.2 |
|  | n7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n28-n41-n77 | n3 | 0.5 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
|  | n77 | 0.5 |
| CA_n3-n28-n41-n78 | n3 | 0.5 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
|  | n78 | 0.5 |
| CA_n7-n25-n66-n78 | n7 | 0.5 |
|  | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n25-n41-n66-n71 | n25 | 0.3 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |
| CA_n41-n66-n71-n77 | n41 | 03/0.54 |
|  | n66 | 0.5 |
|  | n71 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: Applicable for the frequency range of 2545-2690 MHz. NOTE 4: Applicable for the frequency range of 2496-2545 MHz. |  |  |

< end of changes >