3GPP TSG-RAN WG4 Meeting #99-e		                    R4-2110210

Online, 19th May – 27th May, 2021

Source: 	Keysight Technologies

Title: 	R15 BS demod test item error correction on TS38.141-1 and -2, summary of CRs
Agenda Item:	4.1.9.3

Document for:	Information

# Introduction

We submit CRs related with BS demod tests which defined in R15 work item. By some unknown reason, strange error but critical to make correction, introduced after some time from when these tests were introduced and none of us found by now.

This document is to provide summary of error to correct by these CRs, all of these are critical to make correction to perform testing correctly.

# Description

We submit following CRs;

- R4-2110202

  - For R15 TS38.141-1, by some unknown reason, error exist in R15 version of TS38.141-1 only (no error on R16/R17 document). Table 8.2.1.5-5 for PUSCH test requirement has wrong FRC and SNR value as requirement for 2x8

- R4-2110205

  - For R15 TS38.141-2, PUCCH format 1 AWGN level table, Table 8.3.2.1.4.2-1 (for NACK to ACK detection) and Table 8.3.2.2.4.2-1 (for ACK missed detection), 120kHz SCS description missing also AWGN level description are incorrect for ACK missed detection test. Different set of error exist in R16/R17 version of this text on the same table.

- R4-2110206

  - For R16 TS38.141-1, PUCCH format 3 requirement, Table 8.3.4.5-1 shows incorrect value for number of Tx antenna

  - Same error exists in R17 (but not with R15 spec), cat A for R17 reserved. (R4-2110208)

- R4-2110207

  - For R16 TS38.141-2, PUCCH format 1 AWGN level table, on the same table as R15, Table 8.3.2.1.4.2-1 (for NACK to ACK detection) and Table 8.3.2.2.4.2-1 (for ACK missed detection), 120kHz SCS description missing and AWGN level description has incorrect value, but although this is on the same set of table, different error introduced from R16

  - Same error exists in R17, cat A for R17 reserved. (R4-2110209)