3GPP TSG-RAN WG4 Meeting # 99-e 	    R4-2110229

Electronic Meeting 19th May – 27th May 2021

Agenda Item:	6.5			NR Positioning Support
6.5.2.3.3 	gNB Rx-Tx time difference requirements

Source: 	Ericsson

Title:	gNB Rx-Tx time difference requirements

Document for:	Discussion

# Introduction

In RAN4#98-bis-e it was agreed to further analyze the results of link level simulation results which shall be conducted according to the updated link level simulation assumptions. [1][2]

It was discussed that it should be identified if gNB Rx-Tx accuracy requirements can be defined agnostic to UL-SRS-NumSymbols and UL-SRS-CombSizeN parameters and therefore, more configurations were added in [2] to further analyze the issue.

This paper is in relation with the simulation results contribution [3] and shall further analyze the issue of deriving and defining gNB Rx-Tx measurement accuracy requirements.

# Discussion gNB Rx-Tx time difference analysis

As observed in [3], gNB TOA accuracy is agnostic to UL-SRS-NumSymbols and UL-SRS-CombSizeN for most SRS configurations and only shows dependent behaviour for very low channel bandwidth and SRS bandwidth configurations. While the performance gap between all analyzed permutations of UL-SRS-NumSymbols and UL-SRS-CombSizeN is non-neglible for the corner case of SRS configurations, e.g. 5MHz carrier, 15kHZ subcarrier spacing, 24RB SRS bandwidth and also for low Ês/Iot side condition, it decreases rapidly when SRS configurations with more bandwidth are analyzed or when analyzing the accuracy values for high Ês/Iot side condition.

As an excerpt of the simulation results presented in [3] we can look at the following results for FR1:

Table 1 Excerpt of simulated gNB TOA accuracy in FR1 for low BW

| BW (MHz) | SCS (kHz) | SRS BW (RB) | FFT size | Sampling rate (Tc) | UL-SRS-NumSymbols | UL-SRS-CombSizeN | Accuracy @ 90% (Tc), SINR >= -13dB | Accuracy @ 90% (Tc), SINR >= +3dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 15 | 24 | 512 | 256 | 1 | 2 | 170 | 110 |
|  |  |  |  |  | 2 | 2 | 131 | 110 |
|  |  |  |  |  | 4 | 4 | 118 | 110 |
|  |  |  |  |  | 8 | 8 | 114 | 110 |
|  |  |  |  |  | 12 | 4 | 110 | 110 |
|  |  |  |  |  | 12 | 8 | 110 | 110 |
| 10 | 15 | 52 | 1024 | 128 | 1 | 2 | 65 | 58 |
|  |  |  |  |  | 2 | 2 | 60 | 58 |
|  |  |  |  |  | 4 | 4 | 58 | 58 |
|  |  |  |  |  | 8 | 8 | 58 | 58 |
|  |  |  |  |  | 12 | 4 | 58 | 58 |
|  |  |  |  |  | 12 | 8 | 58 | 58 |

As previously stated, the dependent behavior of gNB TOA accuracy on UL-SRS-NumSymbols and UL-SRS-CombSizeN only shows for the configurations with low bandwidth, therefore in Table 1, the performance gap for certain configurations for the bandwidths of 5MHz/24RB and 10MHz/52RB can be observed.

For the Ês/Iot ≥ 3dB side condition, the performance gap across all UL-SRS-NumSymbols and UL-SRS-CombSizeN is neglible.

Define gNB Rx-Tx measurement accuracy requirements agnostic to UL-SRS-NumSymbols and UL-SRS-CombSizeN for high Ês/Iot side condition

For the Ês/Iot ≥ -13dB side condition, the performance gap across most UL-SRS-NumSymbols and UL-SRS-CombSizeN is neglible. Therefore, one can discuss how to cover this issue in the performance requirement. There are several options:

- Option 1: Define gNB Rx-Tx measurement accuracy requirements agnostic to UL-SRS-NumSymbols and UL-SRS-CombSizeN for both Ês/Iot side conditions and take the worst case as the value for requirement definition

- Option 2: Define gNB Rx-Tx measurement accuracy requirements for low Ês/Iot side condition only for UL-SRS-NumSymbols and UL-SRS-CombSizeN combinations above certain threshold value where the performance gap is neglible

- Option 3: Define gNB Rx-Tx measurement accuracy requirements for low Ês/Iot side condition for all UL-SRS-NumSymbols and UL-SRS-CombSizeN combinations, but define separate accuracy requirements for configurations when performance gap is non-neglible

To simplify gNB Rx-Tx measurement accuracy requirements definition under the prerequisite, that the defined accuracy is meaningful, we would prefer option 1 of above listed options and define gNB Rx-Tx measurement accuracy requirements agnostic to UL-SRS-NumSymbols and UL-SRS-CombSizeN and define the accuracy requirement for all configurations of UL-SRS-NumSymbols and UL-SRS-CombSizeN from the worst case of the simulation results.

Define gNB Rx-Tx measurement accuracy requirements agnostic to UL-SRS-NumSymbols and UL-SRS-CombSizeN for low Ês/Iot side condition with a worst-case approach

The observations from [3] and proposals from this contribution are captured in the updated draft CR to 38.133 [4] and include following proposal for table structure, grouping of SRS bandwidth, etc. Note, that the measurement accuracy values are in brackets since the actual accuracy is still subject to agreement.

Table 2 gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-C, 1-H and 1-O

| Accuracy (X) | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| ±[170] | ≥ -13 | 15 | 24 ≤ BW ≤ 40 |
| ±[65] |  |  | 40 ≤ BW ≤ 84 |
| ±[32] |  |  | 88 ≤ BW ≤ 168 |
| ±[15] |  |  | 176 ≤ BW ≤ 264 |
| ±[110] | ≥ +3 |  | 24 ≤ BW ≤ 40 |
| ±[58] |  |  | 40 ≤ BW ≤ 84 |
| ±[30] |  |  | 88 ≤ BW ≤ 168 |
| ±[15] |  |  | 176 ≤ BW ≤ 264 |
| ±[31] | ≥ -13 | 30 | 48 ≤ BW ≤ 84 |
| ±[15] |  |  | 88 ≤ BW ≤ 168 |
| ±[7] |  |  | 176 ≤ BW ≤ 272 |
| ±[30] | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| ±[15] |  |  | 88 ≤ BW ≤ 168 |
| ±[7] |  |  | 176 ≤ BW ≤ 272 |
| ±[18] | ≥ -13 | 60 | 48 ≤ BW ≤ 84 |
| ±[8] |  |  | BW ≥ 84 |
| ±[15] | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| ±[8] |  |  | BW ≥ 84 |

Table 3 gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 2-O

| Accuracy (X) | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| ±[8] | ≥ -13 | 60 | BW ≥ 132 |
| ±[4] |  |  | BW ≥ 264 |
| ±[8] | ≥ +3 |  | BW ≥ 132 |
| ±[4] |  |  | BW ≥ 264 |
| ±[25] | ≥ -13 | 120 | 32 ≤ BW ≤ 40 |
| ±[8] |  |  | 44 ≤ BW ≤ 84 |
| ±[4] |  |  | BW ≥ 88 |
| ±[15] | ≥ +3 |  | 32 ≤ BW ≤ 40 |
| ±[8] |  |  | 44 ≤ BW ≤ 84 |
| ±[4] |  |  | BW ≥ 88 |

- gNB Rx-Tx time difference accuracy is defined as: ± (X+Z) Tc.

- Where:

  - X is numerical value derived from the link simulation results

  - Z accounts for RF margin and is declared by manufacturer.

    - Z depends on gNB type (1-C, 1-H, 1-O, 2-O)

As per earlier agreement in RAN4#98-bis-e and captured in the last meetings WF [1], the error margins that shall apply to the baseline accuracy derived from link level simulation results should be declared by manufacturer:

Therefore, Table 2 and Table 3 incorporate the value derived from link simulation results, X in units of [Tc], while Z is left to declaration by manufacturer. This can be captured in the specification in the following fashion in 38.133 clause 13.2.2.2:

[…] The gNB Rx-Tx accuracy requirements in Table 13.2.2.2-1 and Table 13.2.2.2-2, ±X Tc, are subject to an error margin ±Z Tc, which is declared by manufacturer and can be different for gNB types 1-C, 1-H, 1-O and 2-O respectively, such that the applicable measurement accuracy requirement for gNB Rx-Tx can be identified as ±(X+Z) Tc. […]

Capture the following in 38.133 clause 13.2.2.2: “The gNB Rx-Tx accuracy requirements in Table 13.2.2.2-1 and Table 13.2.2.2-2, ±X Tc, are subject to an error margin ±Z Tc, which is declared by manufacturer and can be different for gNB types 1-C, 1-H, 1-O and 2-O respectively, such that the applicable measurement accuracy requirement for gNB Rx-Tx can be identified as ±(X+Z) Tc.”

# Conclusion

- Define gNB Rx-Tx measurement accuracy requirements agnostic to UL-SRS-NumSymbols and UL-SRS-CombSizeN for high Ês/Iot side condition

- Define gNB Rx-Tx measurement accuracy requirements agnostic to UL-SRS-NumSymbols and UL-SRS-CombSizeN for low Ês/Iot side condition with a worst-case approach

- Capture the following in 38.133 clause 13.2.2.2: “The gNB Rx-Tx accuracy requirements in Table 13.2.2.2-1 and Table 13.2.2.2-2, ±X Tc, are subject to an error margin ±Z Tc, which is declared by manufacturer and can be different for gNB types 1-C, 1-H, 1-O and 2-O respectively, such that the applicable measurement accuracy requirement for gNB Rx-Tx can be identified as ±(X+Z) Tc.”

# References

- R4-2105755, “WF on gNB positioning measurement requirements”, Ericsson

- R4-2105758, “Updated link simulation assumptions for gNB positioning measurement”, Huawei

- R4-2110225, “gNB link level simulation results”, Ericsson

- R4-2110230, “gNB Rx-Tx measurement”, Ericsson