3GPP TSG-RAN WG4 Meeting # 99-e 		R4-2110269

# Electronic Meeting, May 19-27, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Pre-configured MG patterns

Agenda item:	9.10.2.1

Document for:	Discussion

Introduction

At RAN4 #98-bis-e, the technical discussion on pre-configured measurement gap patterns as part of the Rel-17 work item on NR and MR-DC measurement gap enhancements [1] was continued. Agreements and open issues were captured in the WF [2].

Following agreements were reached and following items for further study were identified.

Agreements Scenarios for use of preconfigured MG patterns: - Pre-configured MG will serve at least for SSB measurements. - Focus is on single CC BWP switching scenario. Phasing: - Basic pre-configured MG configuration and (de-)activation mechanism for single CC BWP switching case in 1st phase (until RAN4 #100) - Whether to define pre-configured MG for multiple CC BWP switching scenario for NR-CA case can be considered in 2nd phase. - Whether pre-configured gaps shall be considered as a part of multiple concurrent gap patterns framework can be considered in 2nd phase. Configuration procedure: - Pre-configured MG(s) are configured per-UE or per-FR. Common configuration parameters of pre-configured MG which are the same as for Rel-16 legacy MG, can be configured similarly to legacy Rel-16 MGs. Activation/Deactivation procedures: - Conditions for activating / deactivating pre-configured MG are: - BWP switching, - gapless measurements are not possible / possible after BWP switch, and - other TBD conditions. Gap patterns: - single pre-configured MG for non-concurrent MG scenarios. - Rules for pre-configured MG pattern applicability should be same as for legacy Rel-16 MGPs. Signalling: - RAN4 to send LS to RAN2 on signalling aspects for pre-configured MGPs once the procedures are stabilized in RAN4.

Items for further study Scenarios for use of preconfigured MG patterns: - Whether pre-configured MG can be used for PRS or CSI-RS L3 measurements. Configuration procedure: - Whether pre-configured MG(s), configurable per-UE and per-FR, can be additionally configured per BWP. - Which specific configuration parameters for new aspects of pre-configured MG will be defined (e.g. single bit or flag per BWP to indicate ON/OFF status of pre-configured MG or based on other specific configuration parameters). - Whether preconfigured MG is activated or not after configuration. - Whether MG configuration can be changed after BWP switching. - The relation between pre-configured MG pattern and current RRC configured MG Activation/Deactivation procedures: - Whether network can fully control (de-)activation. - How the (de-)activation is done - whether implicitly/autonomously triggered by condition change or via explicit (de-)activation command, using ON/OFF MG status, or via network indication to enable a MG pattern change after BWP switch - How to evaluate the activation/deactivation mechanism (in terms of robustness, delay, etc.) RRM requirements: - whether to specify a separate delay for (de-)activation, - whether there is a change in measurement period for pre-configured MGs, - whether to limit transitions between gapless and gap-based measurement procedures during measurements, - whether there is any scheduling restriction during pre-configured MGs when MG is not used, - UE behaviour after deactivation of pre-configured MG Gap patterns: - FFS if more than 1 pre-configured MG will be defined in the context of multiple concurrent MGPs. - Whether to reuse Rel-16 MGPs: IDs #0-#23 or #0-#25 Terminology: - Whether the abbreviation should be P-MG or Pre-MG for pre-configured MG

This contribution provides our view on some of the above open issues.

Discussion

Scenarios for use of preconfigured MG pattern

Regarding scenarios for use of pre-configured MG pattern, we prefer a generic approach is taken, that is pre-configured MG pattern can be applied not only for SSB measurements, but also for measuring other reference signals such as PRS and CSI-RS L3, as well can be used for inter-RAT measurements. Thus, pre-configured MG pattern can cover the same use case scenarios as legacy MG pattern.

It is emphasized that the benefit in reduced latency for activation and de-activation of MG patterns through use of pre-configured MG pattern is not only relevant for gap-based SSB measurements after BWP switching, but also for positioning and CSI-RS L3 measurements, where a change / optimization of the MGP can be performed much faster compared to legacy MGPs and hence latency of the positioning / beam management procedure can be efficiently reduced.

a) The use case scenarios for pre-configured MGP include measuring intra-/inter-frequency SSB and additionally at least measuring PRS and CSI-RS L3.

- b) It may even include inter-RAT measurements.

Relation between preconfigured MG and multiple concurrent MG patterns

With regard to the relation of pre-configured MG pattern to multiple concurrent MG patterns, we prefer to reuse the pre-configured MG mechanism for scenarios with multiple concurrent MG patterns. Hence the procedures for configuration and (de-)activation of pre-configured MG patterns should support both single MGP and multiple concurrent MGPs. Although this has been agreed to be investigated within the framework with multiple concurrent MGPs only in 2nd phase, this should already be taken into account in the 1st phase.

The procedures for configuration and (de-)activation of pre-configured MG patterns are designed such to support both single MGP and multiple concurrent MGPs.

Configuration Procedure

Regarding configuration of pre-configured MG patterns, we assume that the basic gap configuration is still performed using RRC signalling as currently done. Hence the legacy approach is followed reusing per-UE and per-FR gap support. For NR Rel-15, only one MG pattern (per-UE or per-FR gap) is configured and applied to UE, i.e. it is setup or released, without consideration on the active BWP. For pre-configured MG patterns in Rel-17, supporting fast (de-) activation of MGPs following BWP switch, it is desirable to support different measurement configurations with a single or multiple MGPs per BWP depending on the active BWP, for instance for different measurement purposes, e.g. one MGP is designated for SSB monitoring while another one is used for positioning measurements. As discussed during RAN4 #98-bis-e, acceptable MG overhead can be a function of scheduled data volume and hence is expected to be different for different BWPs. Thus, the configuration procedure may configure different preconfigured MGPs for different BWPs. The configuration of pre-configured MGPs should be done by RRC signalling with appropriate message extensions, as outlined below.

Existing RRC signalling message (i.e. RRC reconfiguration command) shall be extended to configure a single or multiple pre-configured MGPs per BWP used for per-UE or per-FR gap support.

This can be realized with only little impact to MG configuration in the RRC signalling (e.g. in RRCReconfiguration and RRCResume messages) due to the message extension (pending RAN2 message design). It means, that pre-configured MG(s), configurable per-UE and per-FR, can additionally be configured per BWP.

Pre-configured MG(s), configurable per-UE and per-FR, can additionally be configured per BWP.

When pre-configured MG patterns are configured via RRC signalling, they are not activated at the same time of configuration as in the legacy. It rather means that each pre-configured MGP included in the MG configuration information is inactive at the time of configuration and requires subsequent activation.

When pre-configured MG patterns are configured via RRC signalling, they are not activated at the same time of configuration, hence remain inactive.

Regarding signalling robustness for configuring pre-configured MG, there is no difference to legacy MG configuration, as RRC signalling is reused.

Activation and Deactivation of MG pattern

For (de-)activation of a particular measurement gap pattern, there are basically three options other than RRC signalling:

- No signalling (implicit / autonomous triggering by condition change)

- MAC signalling (e.g. MAC CE)

- DCI (including DCI timer)

The first listed option of no signalling has the drawback that it has limited scope as the trigger condition can only activate or de-activate a single pre-configured MGP and cannot signal a change of MGP. Thus, the above objective to define a generic procedure for preconfigured MG in order to support both single MGP and multiple concurrent MGPs, cannot be met. Therefore, explicit signalling options based on MAC and/or DCI signalling are favoured. Both options ensure fast signalling compared against RRC signalling and may even be used concurrently.

Signalling for activation / deactivation of a pre-configured MG pattern is based on explicit activation/deactivation command.

However, a key aspect when discussing which method to use, is to investigate the robustness of the particular method.

RAN4 to consider the robustness of the mechanisms for activation and deactivation of MG pattern when they do not use RRC signalling.

In legacy, all measurement gap configurations (including gap activations) are done using RRC signalling. Measurement gaps are configured by network based on the need from network side to have the UE to perform e.g. inter-frequency measurements for the purpose of mobility or setting up CA/DC. The need for such measurements and results has been more semi-static and therefore RRC control was good enough. Additional aspect is that the RRC signalling is seen as being very robust. Robustness is an important factor when configuring (or removing) measurements gaps as failures in gap configurations can have severe system impact.

In NR Rel-15, RRC signalling for configuring and activating measurement gaps has been used due to signalling robustness.

If the measurement gap configurations in legacy case were not robust, it could lead to the situation where the network has configured the UE with a MGP and assumes that the UE has activated the MGP. Using a typical example of using GP#0, the UE has a gap every 40 ms with MGL of 6 ms. When a GP has been configured to a UE this would be accounted in the network scheduler and the UE would not be scheduled during the measurement gap (plus accounting the scheduling restrictions before and after the gap). If the gap configuration failed due to e.g. signalling error, the UE would then not apply gaps. In this case the UE will still not be scheduled (as network assumes UE has gaps active) and UE will suffer in terms of inefficient data scheduling and unnecessary power consumption.

Same can of course happen if the network de-configures measurement gaps. In this case the network will observe ‘strange’ behaviour from UE in terms of unreliable link and lot of lost scheduling, since the UE still employs measurement gaps.

Errors in measurement gap configuration can have significant negative UE and system impact.

Hence, it is important that changes in the measurement gaps applied by the UE are signalled in a robust way.

It is important that changes in the measurement gaps are signalled in a robust way.

For DCI signalling, TS 38.212 specifies that active BWP change via DCI is signalled in DCI format 0-1 or 0-2 for UL active BWP change and in DCI format 1-1 or 1-2 for DL active BWP change. Thus, signalling for (de-)activation of pre-configured MG pattern needs to be investigated in terms of robustness for these DCI formats.

DCI formats 0-1, 0-2, 1-1 and 1-2 need to be investigated for inclusion of information bits related to (de-)activation of pre-configured MGs and for associated signalling robustness.

Based on these important system observations, our view is that RAN4 need to account for robustness of the gap changes when evaluating and agreeing on explicit activation/deactivation of MG pattern(s) without using RRC signalling.

RAN4 need to account for robustness of the measurement gap changes when evaluating and agreeing on explicit activation/deactivation of MG pattern(s) without using RRC signalling.

Evaluation criteria for MGP (de-)activation

Regarding the suitability of a signalling mechanism, we propose RAN4 to consider following evaluation criteria:

- MGP change latency (how long time it takes the UE to account a MGP change),

- MGP change robustness (compared to existing baseline),

- Impact on measurement latency (e.g. possible impact on the UE measurement period for ongoing measurements),

- Impact on cell detection latency (e.g. possible impact on the UE cell detection delay for ongoing cell detection).

Other evaluation parameters can be included as needed. Based on analysing each of the above mechanism RAN4 would be able to decide which mechanism.

MGP change delay

When evaluating the MGP change delay this would be done by evaluating and agreeing on the minimum UE minimum delay we can agree on in RAN4. Hence, the evaluation should be based on the on agreed latency values for setting up a new MG, change of MG and removing a MG.

MGP change delay shall be evaluated based on realistic latencies.

We would expect that such delays would be similar to already defined delays defined for applying MAC settings if MAC message is used, or delays already defined for applying DCI based changes (e.g. BWP change)

No additional separate delay is needed for activating a preconfigured MGP.

No separate additional delay is needed for deactivating a preconfigured MGP.

MGP change robustness

Evaluating the robustness of the MGP change method it should be feasible to re-use the existing signalling robustness assumptions. Hence, MAC based MG change mechanism there is the signalling loss probability as well as the HARQ protection. For DCI based change mechanism the robustness would be the existing estimated DCI loss probability.

Robustness shall be evaluated including the final signal loss probability.

Impact on latency

Here RAN4 need to analyse and evaluate, under realistic assumption, the possible impact on cell detection and measurement latencies, when UE is requested to setup a GP, change between GP and teardown a GP. The aspect to analyse and identify here would be if a change in MGP would have a negative impact on the existing ongoing cell detection and/or measurements possibly already ongoing (already performed). E.g. would assigning a new GP to a UE lead to existing detected cells would need to be re-verified, would it lead to a reset in UE measurements or measurements etc.

These aspects are clearly important as any additional delay can impact how the feature can be used and would be used in the field. If for example a change in MGP would lead to that the UE would need to re-detect already detected cells or lead to resetting of the measurement period, this would clearly limit the how often a MGP realistically can be changed.

Analyse and evaluate, under realistic assumption, the possible impact on ongoing cell detection from a change in MGP.

Analyse and evaluate, under realistic assumption, the possible impact on the latency of ongoing measurements from a change in MGP.

Network controlled usage of MG patterns

In NR Rel-15, up to 4 BWPs can be configured for UE by one RRC Reconfiguration message, where the MG configuration is common and applies to all BWPs. As pre-configured MG patterns are suited to address different measurement purposes, such as intra-frequency, inter-frequency and inter-RAT measurements, including positioning measurements, it is preferable from network perspective to configure one or more MG patterns per BWP suited for these measurements and to ensure network controlled operation of the MG patterns through explicit activation/deactivation.

There against autonomous/implicit activation / deactivation of the MG pattern following a BWP switch, e.g. based on the knowledge whether the active BWP covers SSB or not for intra-frequency measurements, is an inferior solution, as it does not offer any flexibility than using the common MG pattern and does not provide full network control of the MG pattern usage. For instance, in case the network determines to change the MG pattern in the active BWP for another measurement purpose, then it has to use RRC signalling in order to provide the new MG pattern configuration to the UE. The lack of flexibility in MG configuration and activation as well as the lack of full network control of the MG pattern usage should be avoided when defining the procedures for pre-configured MG pattern support.

RAN 4 to define procedures for pre-configured MG pattern support, which provide flexibility and full network control related to MG pattern usage.

Based on the above, the concept of flexible configuration of MG patterns is described thereafter.

To address how pre-configured MG patterns can be configured by RRC signalling and subsequently activated or de-activated via DCI or MAC CE signalling in a flexible manner, we consider the case of 4 dedicated BWPs and per-UE gap. For using pre-configured MG patterns, one or more MG patterns are configured per BWP (BWP1 … BWP4) as follows:

- For BWP1, covering SSB, one MG pattern is configured. If BWP1 identifies the active BWP, MG pattern1 can be mapped to legacy MG pattern configured in RRC Reconfiguration message.

- For BWP2, not covering SSB, two MG patterns are configured as pre-configured MG patterns.

- For BWP3, not covering SSB, two MG patterns are configured as pre-configured MG patterns.

- For BWP4, not covering SSB, two MG patterns are configured as pre-configured MG patterns.

In case inactive BWP2 or BWP3 or BWP4 becomes active BWP, ON/OFF signalling of the preconfigured MG pattern is not appropriate, and extra signalling is needed to indicate which MG pattern out of the set of pre-configured MG patterns for this BWP is to be activated. To avoid this drawback, the MG pattern needs to be indicated in the MG activation / deactivation command after DCI or timer-based BWP switching. Based on the assumption some MG patterns can be shared between BWPs, latency and signalling load in the RRC signalling can efficiently be reduced in defining a pool of MG patterns for some configured BWPs, as shown in Figure 1.

Figure 1: Configuration of pre-configured MG patterns using MG pattern pool.

The definition of a MG pattern pool reduces signalling load and yields flexibility for sharing MGPs among BWPs. Each pre-configured MG pattern in the MG pattern pool is uniquely indexed and can be used for activation or deactivation for inactive BWPs when they become active. The MG pattern with all gap parameters (gap offset, MGL, MGRP, MGTA) is indexed. This way, a flexible configuration of pre-configured MG patterns is achieved without the need to send RRCReconfiguration message. We propose to consider this approach for defining the configuration and (de-)activation mechanisms of pre-configured MG patterns.

Consider sharing of MG patterns using a MG pattern pool for specifying the configuration of pre-defined MG patterns.

Use of existing MG patterns for Pre-configured MG

Regarding the reuse of existing MG patterns for use as pre-configured MG pattern, according to above proposed scenarios for use of preconfigured MG patterns, all MGPs defined up to Rel-16 should be considered, i.e. we prefer option 2 in section 1.

RAN4 to consider MG patterns #0-#25 for use as pre-configured MG pattern.

Transitions between gapless and gap-based measurement 	procedures

Regarding transitions between gapless and gap-based measurement procedures during measurement period for ongoing measurements, we cannot agree to the option to limit the number of transitions during ongoing measurements. In our view, how the network is operated, i.e. how often it changes BWP, cannot be restricted, and it is not part of the RAN4 work. However, it can be discussed if UE requirements for certain scenarios need to be defined.

RAN4 not to specify maximum number of transitions between gapless and gap-based measurement procedures for ongoing measurements.

Scheduling restrictions

Regarding whether there is any scheduling restriction during pre-configured MGs when MG is not used, i.e. it is deactivated, and no other MG is required, usual scheduling restrictions as defined in TS 38.133, clause 9.2.5.3, Scheduling availability of UE during intra-frequency measurements, apply.

Scheduling restrictions in TS 38.133, clause 9.2.5.3 apply in case the UE does not require pre-configured or legacy measurement gaps.

Terminology

For the abbreviation of pre-configured measurement gaps, our preference is to name this Pre-MG, as this sounds more meaningful.

RAN4 to use the abbreviation Pre-MG for pre-configured measurement gaps in TS 38.133.

Conclusion

In last RAN4 meeting, the technical discussion on pre-configured measurement gap patterns as part of the Rel-17 work item on NR and MR-DC measurement gap enhancements [1] was continued. Agreements and open issues were captured in the WF [2]. In this paper, we discuss the various open issues, make observations and list a number of proposals.

We observed:

- In NR Rel-15, RRC signalling for configuring and activating measurement gaps has been used due to signalling robustness.

- Errors in measurement gap configuration can have significant negative UE and system impact.

- It is important that changes in the measurement gaps are signalled in a robust way.

- DCI formats 0-1, 0-2, 1-1 and 1-2 need to be investigated for inclusion of information bits related to 	(de-)activation of pre-configured MGs and for associated signalling robustness.

Following proposals are made, which we see necessary to be considered in the discussion to ensure that a practically usable feature will be developed:

- a) The use case scenarios for pre-configured MGP include measuring intra-/inter-frequency SSB and additionally at least measuring PRS and CSI-RS L3.

- b) It may even include inter-RAT measurements.

The procedures for configuration and (de-)activation of pre-configured MG patterns are designed such to support both single MGP and multiple concurrent MGPs.

Existing RRC signalling message (i.e. RRC reconfiguration command) shall be extended to configure a single or multiple pre-configured MGPs per BWP used for per-UE or per-FR gap support.

Pre-configured MG(s), configurable per-UE and per-FR, can additionally be configured per BWP.

When pre-configured MG patterns are configured via RRC signalling, they are not activated at the same time of configuration, hence remain inactive.

Signalling for activation / deactivation of a pre-configured MG pattern is based on explicit activation/deactivation command.

RAN4 to consider the robustness of the mechanisms for activation and deactivation of MG pattern when they do not use RRC signalling.

RAN4 need to account for robustness of the measurement gap changes when evaluating and agreeing on explicit activation/deactivation of MG pattern(s) without using RRC signalling.

MGP change delay shall be evaluated based on realistic latencies.

No additional separate delay is needed for activating a preconfigured MGP.

No separate additional delay is needed for deactivating a preconfigured MGP.

Robustness shall be evaluated including the final signal loss probability.

Analyse and evaluate, under realistic assumption, the possible impact on ongoing cell detection from a change in MGP.

Analyse and evaluate, under realistic assumption, the possible impact on the latency of ongoing measurements from a change in MGP.

RAN 4 to define procedures for pre-configured MG pattern support, which provide flexibility and full network control related to MG pattern usage.

Consider sharing of MG patterns using a MG pattern pool for specifying the configuration of pre-defined MG patterns.

RAN4 to consider MG patterns #0-#25 for use as pre-configured MG pattern.

RAN4 not to specify maximum number of transitions between gapless and gap-based measurement procedures for ongoing measurements.

Scheduling restrictions in TS 38.133, clause 9.2.5.3 apply in case the UE does not require pre-configured or legacy measurement gaps.

RAN4 to use the abbreviation Pre-MG for pre-configured measurement gaps in TS 38.133.

References

- RP-202658, Revised WID on NR and MR-DC measurement gap enhancements, Intel Corporation, MediaTek Inc.

- R4-2105791, WF on R17 NR MG enhancements – Pre-configured MG, Intel Corporation