3GPP TSG-RAN WG4 Meeting # 99-e                               R4-2110400

Electronic Meeting, 19– 27 May, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-16 NR V2X AMPR value for both NS_33 and NS_52

Agenda Item:	6.2.1

Document for:	Approval

Introduction

Based on the WF [1], the following AMPR requirements for NR V2X NS_52 were introduced into the specification. However, after further analysis and measurements, it seems that AMPR value for region 1 is too stringent to meet the ASEM requirement. In addition, the AMPR values of some RB allocation cases are missing for NS_33 PSSCH/PSCCH. Thus, we’d like to further discuss it in this paper.

Discussion for NS_52

Referring to clause 6.5E.2.3.2 from TS 38.101-1, the additional spectrum mask requirements for NS_52 are copied as below.

Table 2-1: Additional spectrum mask requirements for 40MHz channel bandwidth (fc = 5885MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0-2 | -32 | 100kHz |
| 2-10 | -36 | 100kHz |
| 10-20 | -38 | 100kHz |
| 20-40 | -43 | 100kHz |
| 40-100 | -50 | 100kHz |

The required AMPR is 13.5dB in region 1, considering the worst case (RBstart = 0, LCRB=10). This case is restricted by the first ΔfOOB (0-2) -32dBm/100kHz.

Comparing to the NS_33 for fc=5860MHz, the required AMPR for the worst case (RBstart = 0, LCRB=10) is 24dB. This case is restricted by the ASE -30dBm/MHz.

Thus, the difference between these two cases can be summarized as below.

Table 2-2: the AMPR difference for worst case between NS_52 and NS_33

| NS | NS_52 (fc=5885) | NS_33 (fc=5860) |
| --- | --- | --- |
| Requirements for ΔfOOB (0-2 MHz) | -32dBm/100kHz | -30dBm/MHz (=-40dBm/100kHz) |
| AMPR value for worst case (RBstart = 0, LCRB=10) | 13.5dB | 24dB |

The difference of Requirements for ΔfOOB (0-2 MHz) is 8dB. (-32dBm/100kHz minus -40dBm/100kHz is 8dB)

Considering the difference of Requirements for ΔfOOB (0-2 MHz), current AMPR requirements for NS_52 is not enough. At least, 16 dB AMPR requirements for NS_52 region 1 is needed.

Proposal 1: It’s proposed to correct the AMPR requirements as 16dB for NS_52 region 1 as below.

Table 2-3: A-MPR for PSSCH/PSCCH by NS_52

| Carrier frequency(MHz) | Modulation | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Region 1 | Region 2 | Region 3 |
| 5885 | QPSK | ≤ 16 | ≤ 8.0 | ≤ 5.5 |
|  | 16QAM |  | ≤ 8.0 | ≤ 5.5 |
|  | 64QAM |  | ≤ 8.5 | ≤ 5.5 |
|  | 256QAM |  | ≤ 8.5 | ≤ 6.0 |
| Note1: Void. |  |  |  |  |

In addition, based on the FCC regulation [2], the ITS operation using C-V2X technology is in 5895~5925. The specified emissions limits in FCC regulation are not what RAN4 specified in clause 6.5E.2.3.2 from TS 38.101-1. Furthermore, it’s unclear what the maximum channel bandwidth is, in which the emissions limits apply.

Observation 1: The specified emissions limits in FCC regulation are not what RAN4 specified in clause 6.5E.2.3.2 from TS 38.101-1.

Observation 2: Currently, there is no 40MHz ITS spectrum allocation based on FCC regulatory.

Proposal 2: Companies are encouraged to further check the FCC regulation. It’s up to RAN4 how to address this mismatching issue.

Option 1: RAN4 can delete the NS_52 additional requirements and further check and review the FCC regulation.

Option 2: RAN4 can send FCC a LS and ask what the accurate FCC regulation for NR V2X is.

Option 3: Other solutions are not precluded.

Discussion for NS_33

The AMPR values of some RB allocation cases are missing for NS_33 PSSCH/PSCCH. As RAN1 update the RB allocation rules, the allowed RB allocation can be found as below.

{10, 12, 15, 20, 24, 25, 30, 36, 40, 45, 48, 50, 60,70,72, 75,80,84, 90, 96, 100, 105, 108, 110, 120, 130, 132, 135, 140, 144, 150, 156, 160, 165,168,170,175,180,190,192,195,200,204,210, 216}

Probably, we need to review our AMPR requirements. After checking, the missing allocations are highlighted as below.

Table 2-4: A-MPR for PSSCH/PSCCH by NS_33 (at Fc =5860MHz)

| Carrier frequency [MHz] | Resources Blocks (LCRB) | Start Resource Block | A-MPRBase (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 10 and ≤ 15 | 0 | ≤ 24 |  |  |
|  |  | ≥ 1 and ≤ 3 | ≤19 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 26 and ≤ 38 | ≤6 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 38 | 7 |  |  |
|  | ≥ 10 and ≤ 20 | ≥ 12 and ≤ 14 | ≤11 |  |  |
|  |  | ≥ 15 and ≤ 19 | ≤9.5 |  |  |
|  |  | ≥ 20 and ≤ 25 | ≤8.0 |  |  |
|  | > 15 and < 25 | ≥ 25 | 8 |  |  |
|  | ≥ 10 and < 40 | ≥ 4 and ≤7 | ≤ 16 |  |  |
|  |  | ≥ 8 and ≤ 11 | ≤ 13.5 |  |  |
|  | ≥ 20 and <40 | ≥ 0 and ≤ 3 | ≤ 22 |  |  |
|  | ≥ 25 and <40 | ≥ 16 and ≤ 21 | ≤ 9.5 |  |  |
|  |  | ≥ 22 and ≤ 27 | ≤ 8.0 |  |  |
|  | ≥ 25 and ≤ 40 | ≥ 12 and ≤ 15 | ≤ 12 |  |  |
|  | ≥ 40 and ≤ 45 | 0 and 1 | ≤ 19 |  |  |
|  |  | ≥ 2 and ≤ 5 | ≤ 16 |  |  |
|  |  | ≥ 6 and ≤ 11 | ≤ 13.5 |  |  |
|  | >45 | ≥ 0 | ≤ 16 |  |  |
| NOTE 1: A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

Proposal 3: It’s proposed to further update the AMPR requirements for NS_33 PSSCH/PSCCH (at Fc =5860MHz).

Conclusion

Based on the analysis and discussion, the proposal are shown below.

Proposal 1: It’s proposed to correct the AMPR requirements as 16dB for NS_52 region 1 as below.

Table 6.2E.3.3-1: A-MPR for PSSCH/PSCCH by NS_52

| Carrier frequency(MHz) | Modulation | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Region 1 | Region 2 | Region 3 |
| 5885 | QPSK | ≤ 16 | ≤ 8.0 | ≤ 5.5 |
|  | 16QAM |  | ≤ 8.0 | ≤ 5.5 |
|  | 64QAM |  | ≤ 8.5 | ≤ 5.5 |
|  | 256QAM |  | ≤ 8.5 | ≤ 6.0 |
| Note1: Void. |  |  |  |  |

Observation 1: The specified emissions limits in FCC regulation are not what RAN4 specified in clause 6.5E.2.3.2 from TS 38.101-1.

Observation 2: Currently, there is no 40MHz ITS spectrum allocation based on FCC regulatory.

Proposal 2: Companies are encouraged to further check the FCC regulation. It’s up to RAN4 how to address this mismatching issue.

Proposal 3: It’s proposed to further update the AMPR requirements for NS_33 PSSCH/PSCCH (at Fc =5860MHz).

Table 2-4: A-MPR for PSSCH/PSCCH by NS_33 (at Fc =5860MHz)

| Carrier frequency [MHz] | Resources Blocks (LCRB) | Start Resource Block | A-MPRBase (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 10 and ≤ 15 | 0 | ≤ 24 |  |  |
|  |  | ≥ 1 and ≤ 3 | ≤19 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 26 and ≤ 38 | ≤6 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 38 | 7 |  |  |
|  | ≥ 10 and ≤ 20 | ≥ 12 and ≤ 14 | ≤11 |  |  |
|  |  | ≥ 15 and ≤ 19 | ≤9.5 |  |  |
|  |  | ≥ 20 and ≤ 25 | ≤8.0 |  |  |
|  | > 15 and < 25 | ≥ 25 | 8 |  |  |
|  | ≥ 10 and < 40 | ≥ 4 and ≤7 | ≤ 16 |  |  |
|  |  | ≥ 8 and ≤ 11 | ≤ 13.5 |  |  |
|  | ≥ 20 and <40 | ≥ 0 and ≤ 3 | ≤ 22 |  |  |
|  | ≥ 25 and <40 | ≥ 16 and ≤ 21 | ≤ 9.5 |  |  |
|  |  | ≥ 22 and ≤ 27 | ≤ 8.0 |  |  |
|  | ≥ 25 and ≤ 40 | ≥ 12 and ≤ 15 | ≤ 12 |  |  |
|  | ≥ 40 and ≤ 45 | 0 and 1 | ≤ 19 |  |  |
|  |  | ≥ 2 and ≤ 5 | ≤ 16 |  |  |
|  |  | ≥ 6 and ≤ 11 | ≤ 13.5 |  |  |
|  | >45 | ≥ 0 | ≤ 16 |  |  |
| NOTE 1: A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

- References

[1]	R4-2011704	WF on A-MPR for NS_52, Qualcomm

[2] https://docs.fcc.gov/public/attachments/DOC-367827A1.pdf.