3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2110514

# Electronic Meeting, May. 19-27, 2021

Title: 	Simulation results on NR-U PUCCH performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.1.7.4.3

Document for:	Discussion

Background

As per WF [1], open issues for NR-U PUCCH requirements were finalized. The agreements reached in last meeting are:

- Test metric for PF3 - Prob(DTX->ACK)<=1% and Prob(ACK miss)<=1% - Pattern of information bits - PF0: [0] including HARQ-ACK information only - PF1:[0 1] including HARQ-ACK information only - PF3: [0 0 0 0] including HARQ-ACK information only - Pattern of PF2 information bits - Random information bits selection

Simulation results

Simulation results are shown in Table 2-1.

Table 2-1: Summary of simulation results for NR-U PUCCH.

| Test metric (SNR dB @ 1% of BLER or 1% of ACK miss or 0.1%of NACK) |  | PF0 | PF1 |  | PF2 | PF3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ACK miss | ACK miss | NACK2ACK | UCI BLER | ACK miss |
| TDLA30-10 | 15kHz | -4.6 | -17.2 | -16.7 | 1.3 | -5.6 |
|  | 30kHz | -3.9 | -17.1 | -16.3 | 1.9 | -4.9 |

Conclusion

In this paper, we provide our simulation results for NR-U PUCCH performance requirements.

Reference

[1]   R4-2106010 WF on NR-U BS demod requirements. Huawei, HiSilicon.