3GPP TSG-RAN WG4 Meeting #99-e	R4-2110546

# Electronic Meeting, 19th - 27th May, 2021

Title: 	pCR on IAB-MT radiated conformance testing (General and Demodulation) to TS 38.176-2

Source: 	Huawei, HiSilicon

Agenda item:	6.3.6.3

Document for:	Approval

- Introduction

During RAN4#98-bis-e meeting, way forward [1] for NR IAB demodulation requirements was approved. This contribution provides the updated text proposal about IAB-MT radiated conformance testing (General and Demodulation) to TS 38.176-2 as per work split.

- Reference

- R4-2103994, WF on Rel-16 NR IAB demodulation requirements, RAN4#98-e, Nokia

- R4-2106089, WF on IAB demod editorial, RAN4#98-bis-e, Ericsson

Text Proposal

<START OF THE TEXT PROPOSAL 1>

8.2	IAB-MT performance requirements

8.2.1	General

8.2.1.1	Scope and definitions

Radiated performance requirements specify the ability of the IAB-MT type 1-O and IAB-MT type 2-O to correctly demodulate signals in various conditions and configurations. Radiated performance requirements are specified at the RIB.

Radiated performance requirements for the IAB-MT are specified for the fixed reference channels defined in annex A and the propagation conditions in annex J. The requirements only apply to those FRCs that are supported by the IAB-MT.

The radiated performance requirements for IAB-MT type 1-O and for IAB-MT type 2-O are limited to two OTA demodulations branches as described in clause 8.2.1.2. Conformance requirements can only be tested for 1 or 2 demodulation branches depending on the numbezr of polarizations supported by the IAB-MT, with the required SNR applied separately per polarization.

NOTE 1: IAB-MT can support more than 2 demodulation branches, however OTA conformance testing can only be performed for 1 or 2 demodulation branches.

In tests performed with signal generators, a synchronization signal may be provided between the IAB node and the signal generator, or a common (e.g., GNSS) source may be provided to both IAB node and the signal generator, to enable correct timing of the wanted signal.

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in the slot on a single TAB connector (for IAB-MT type 1-H).

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot on a single TAB connector (for IAB-MT type 1-H).

8.2.1.2	Applicability rule for IAB-MT

8.2.1.2.1	General

Unless otherwise stated, for a IAB-MT declared to support more than 2 demodulation branches (for IAB-MT type 1-O and IAB-MT type 2-O), the performance requirement tests for 2 demodulation branches shall apply, and the mapping between connectors and demodulation branches is up to IAB-MT implementation.

The tests requiring more than [20] dB SNR level are set to N/A in the test requirements.

8.2.1.2.2	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, the tests shall apply only for each subcarrier spacing declared to be supported (see D.xxx in table xxx).

8.2.1.2.3	Applicability of requirements for different channel bandwidths

Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests for a specific channel bandwidth shall apply only if the IAB-MT supports it (see D.xxx in table xxx).

Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.

8.2.1.2.4	Applicability of requirements for TDD with different UL-DL patterns

Unless otherwise stated, for each subcarrier spacing declared to be supported, if IAB-MT supports multiple TDD UL-DL patterns, only one of the supported TDD UL-DL patterns shall be used for all tests.

8.2.2	Demodulation performance requirements

8.2.2.1	Performance requirements for PDSCH

8.2.2.1.1	Definition and applicability

The performance requirement of PDSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to IAB-MT is based on the test applicability rules defined in clause 8.2.1.2.

8.2.2.1.2	Minimum requirements

For IAB type 1-O, the minimum requirement is in TS 38.174 [TBD], clause 11.2.2.1.1.

For IAB type 2-O, the minimum requirement is in TS 38.174 [TBD], clause 11.2.2.2.1.

8.2.2.1.3	Test purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR.

8.2.2.1.4	Method of test

8.2.2.1.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M, see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

8.2.2.1.4.2	Test procedure

1)	Place the IAB-MT with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the IAB-MT with the test system.

3)	Set the IAB-MT in the declared direction to be tested.

4)	Connect the IAB-MT tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.2.1.4.2-1.

Table: 8.2.2.1.4.2-1 Test parameters for testing PDSCH

| Parameter |  | IAB type 1-O | IAB type 2-O |
| --- | --- | --- | --- |
| Cyclic prefix |  | Normal | Normal |
| Default TDD UL-DL pattern (Note 1) |  | 7D1S2U, S=6D:4G:4U | 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 | 4 |
|  | RV sequence | 0, 2, 3, 1 | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 | 1 |
|  | DM-RS duration | single-symbol DM-RS | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 | 2 |
|  | Additional DM-RS position | pos1 | pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests | 1 |
|  | DM-RS port(s) | {1000} for Rank 1 tests {1000-1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests | {1000} for Rank 1 tests {1000-1001} for Rank 2 tests |
|  | DM-RS sequence generation | NID0=0 | NID0=0 |
| Time domain resource assignment | PDSCH mapping type | A | A |
|  | Start symbol | 2 | 1 |
|  | Allocation length | 12 | 13 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth | Full applicable test bandwidth |
| PT-RS configuration | Frequency density (KPT-RS) | Not configured | 2 |
|  | Time density (LPT-RS) |  | 1 |
| PRB bundling size |  | 2 | 2 |
| VRB-to-PRB mapping type |  | Not interleaved | Not interleaved |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. |  |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the IAB-MT receiver is as specified in clause 8.2.2.1.5.1 and 8.2.2.1.5.2 for IAB type 1-O and IAB type 2-O respectively, and that the SNR at the IAB receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.2.2.1.4.2-2.

Table 8.2.2.1.4.2-2: AWGN power level at the IAB-MT input

| IAB type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| IAB type 1-O | 30 | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
| IAB type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  | 120 | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the IAB, measure the throughput.

8.2.2.1.5	Test requirements

8.2.2.1.5.1	Test requirement for IAB type 1-O

The throughput shall be equal to or larger than the fraction of maximum throughput for the FRCs stated in Table 8.2.2.1.5.1-1 and 8.2.2.1.5.1-2 at the given SNR with the test parameters stated in Table 8.2.2.1.4.2-1.

Table 8.2.2.1.5.1-1: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex A) | Modulation format and code rate | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex J) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | M-FR1-A.3.3-1 | 256QAM, 0.82 | 40/30 | TDLA30-10 | 2x2, ULA Low | 70 | TBD |
| 1-2 | M-FR1-A.3.1-1 | 16QAM, 0.48 | 40/30 | TDLA30-10 | 2x2, ULA Low | 30 | TBD |

Table 8.2.2.1.5.1-2: Minimum requirements for PDSCH Type A with Rank 2

| Test number | FRC (Annex A) | Modulation format and code rate | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex J) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | M-FR1-A.3.2-1 | 64QAM, 0.50 | 40/30 | TDLA30-10 | 2x2, ULA Low | 70 | TBD |

8.2.2.1.5.2	Test requirement for IAB type 2-O

The throughput shall be equal to or larger than the fraction of maximum throughput for the FRCs stated in Table 8.2.2.1.5.2-1 and 8.2.2.1.5.2-2 at the given SNR with the test parameters stated in Table 8.2.2.1.4.2-1.

Table 8.2.2.1.5.2-1: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex A) | Modulation format and code rate | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex J) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | M-FR2-A.3.1-1 | 16QAM, 0.48 | 100/120 | TDLA30-75 | 2x2, ULA Low | 30 | TBD |
| 1-2 | M-FR2-A.3.2-1 | 64QAM, 0.46 | 100/120 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |

Table 8.2.2.1.5.2-2: Minimum requirements for PDSCH Type A with Rank 2

| Test number | FRC (Annex A) | Modulation format and code rate | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex J) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | M-FR2-A.3.1-2 | 16QAM, 0.48 | 100/120 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |
| 2-2 | M-FR2-A.3.1-3 | 16QAM, 0.48 | 50/60 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |
| 2-3 | M-FR2-A.3.2-2 | 64QAM, 0.43 | 100/120 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |

8.2.2.2	Demodulation performance requirements for PDCCH

8.2.2.2.1	Definition and applicability

The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

Which specific test(s) are applicable to IAB-MT is based on the test applicability rules defined in clause 8.2.1.2.

8.2.2.2.2	Minimum requirement

For IAB type 1-O, the minimum requirement is in TS 38.174 [TBD], clause 11.2.2.1.2.

For IAB type 2-O, the minimum requirement is in TS 38.174 [TBD], clause 11.2.2.2.2.

8.2.2.2.3	Test purpose

The test shall verify the receiver's ability to detect the Downlink Scheduling Grant (Pm-dsg) under multipath fading propagation conditions for a given SNR.

8.2.2.2.4	Method of test

8.2.2.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M, see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

8.2.2.2.4.2	Test procedure

1)	Place the IAB-MT with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the IAB-MT with the test system.

3)	Set the IAB-MT in the declared direction to be tested.

4)	Connect the IAB-MT tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.2.2.4.2-1.

Table: 8.2.2.1.4.2-1 Test parameters for testing PDSCH

| Parameter | IAB type 1-O | IAB-type 2-O |
| --- | --- | --- |
| Cyclic prefix | Normal | Normal |
| Default TDD UL-DL pattern (Note 1) | 7D1S2U, S=6D:4G:4U | 3D1S1U, S=10D:2G:2U |
| DM-RS sequence generation | NID=0 | NID=0 |
| Frequency domain resource allocation for CORESET | Start from RB = 0 with contiguous RB allocation | Start from RB = 0 with contiguous RB allocation |
| CCE to REG mapping type | Interleaved | Interleaved |
| Interleaver size | 3 | 2 for Test 2 3 for others |
| REG bundle size | 6 for Test 5, 6 2 for others | 6 for Test 2 2 for others |
| Shift Index | 0 | 0 |
| Slots for PDCCH monitoring | Each slot | Each slot |
| Number of PDCCH candidates for the tested aggregation level | 1 | 1 |
| PDCCH Precoding configuration | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with REG bundling granularity for number of Tx larger than 1 | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with REG bundling granularity for number of Tx larger than 1 |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the IAB-MT receiver is as specified in clause 8.2.2.2.5.1 and 8.2.2.2.5.2 for IAB type 1-O and IAB type 2-O respectively, and that the SNR at the IAB receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.2.2.2.4.2-2.

Table 8.2.2.2.4.2-2: AWGN power level at the IAB-MT input

| IAB type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| IAB type 1-O | 30 | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
| IAB type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  | 120 | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the IAB, measure the miss-detection of the Downlink Scheduling Grant (Pm-dsg).

8.2.2.2.5	Test requirements

8.2.2.2.5.1	Test requirement for IAB type 1-O

The Pm-dsg shall be equal to or smaller than 1%, for the cases stated in Table 8.2.2.2.5.1-1 at the given SNR with the test parameters stated in Table 8.2.2.1.4.2-1.

Table 8.2.2.2.5.1-1: Minimum requirements for PDCCH

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | CORESET RB | CORESET duration | Aggregation level | FRC (Annex A) | Propagation conditions (Annex J) | Antenna configuration | Pm-dsg (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40/30 | 102 | 1 | 2 | M-FR1-A.3.4-1 | TDLA30-10 | 1x2, ULA Low | 1 | TBD |
| 2 | 40/30 | 102 | 1 | 4 | M-FR1-A.3.4-1 | TDLA30-10 | 1x2, ULA Low | 1 | TBD |
| 3 | 40/30 | 90 | 1 | 8 | M-FR1-A.3.4-1 | TDLA30-10 | 2x2, ULA Low | 1 | TBD |

8.2.2.2.5.2	Test requirement for IAB type 2-O

The Pm-dsg shall be equal to or smaller than 1%, for the cases stated in Table 8.2.2.2.5.2-1 at the given SNR with the test parameters stated in Table 8.2.2.1.4.2-1.

Table 8.2.2.2.5.2-1: Minimum requirements for PDCCH

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | CORESET RB | CORESET duration | Aggregation level | FRC (Annex A) | Propagation conditions (Annex J) | Antenna configuration | Pm-dsg (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 60 | 1 | 2 | M-FR2-A.3.4-1 | TDLA30-75 | 1x2, ULA Low | 1 | 6.4 |
| 2 | 100/120 | 60 | 1 | 4 | M-FR2-A.3.4-2 | TDLA30-75 | 1x2, ULA Low | 1 | TBD |
| 3 | 100/120 | 60 | 1 | 8 | M-FR2-A.3.4-3 | TDLA30-75 | 2x2, ULA Low | 1 | 0.1 |

<END OF THE TEXT PROPOSAL 1>