3GPP TSG-RAN WG4 Meeting # 99-e	R4-2110662

# Electronic Meeting, May. 19-27, 2021

Source: 	Huawei, HiSilicon

Title: 	Evaluation for overlapping UE channel bandwidths

Agenda Item:	10.2.3

Document for:	Approval

Introduction

In last RAN meeting, a new SI has been approved on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths. One of the objective is to study the use of overlapping UE channel bandwidths.

Study the use of overlapping UE channel bandwidths (from both UE and network perspective) to cover operator’s license spectrum for both UL and DL, and if new gNB channel bandwidths are needed.

NOTE:	For all considered solutions, new (dedicated) channel filters (e.g. non-integer-multiples of 5MHz) are not considered for the UE and not prioritized for the gNB.

The comparison of the two solutions were discussed in [2]. In the contribution, we provide further update on solution 2 overlapping CA.

Solution 1: new channel bandwidth for gNB and legacy channel bandwidth for UE (Overlapping UE CBW in [4])

New channel bandwidth for gNB

UE1: legacy channel bandwidth

UE2: legacy channel bandwidth

Solution 2: Overlapping CA

Overlapping CA for gNB

Overlapping CA for UE3

UE1: legacy channel bandwidth

UE2: legacy channel bandwidth

Discussion

The flexible channel bandwidth has been discussed since Rel-15 NR study item. To trade-off the implementation complexity and efficient utilization of operators’ spectrum, 13 channel bandwidths has been introduce for FR1 in Rel-15/16, i.e. {5MHz, 10MHz, 15 MHz, 20 MHz, 25 MHz, 30 MHz, 40 MHz, 50 MHz, 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz}, and two new channel bandwidths (35 MHz and 45 MHz) will be introduced in Rel-17.

Observation 1: for channel bandwidths less than 50 MHz, integer-multiples of 5MHz channel bandwidths are supported/will be supported in BS/UE specifications.

And from the WID, the new (dedicated) channel filter are not considered for the UE and not prioritized for the gNB.

“NOTE:	For all considered solutions, new (dedicated) channel filters (e.g. non-integer-multiples of 5MHz) are not considered for the UE and not prioritized for the gNB.”

Proposal 1: New dedicated channel bandwidths are not considered for both BS and UE.

Terminology

Per approved WF [3],

Overlapping CA means that the irregular BW is covered by two overlapping component carriers (CCs) with existing NR CBW each. One of them (PCell) or both CCs contain an SSB. The network prevents collisions between the CCs.

Scenarios

Per approved WF [3],

- Overlapping CA from UE perspective is an optional feature and only applicable in the DL

- Overlapping CA from network perspective is a declared feature in the DL and in the UL.

Guardband and spectrum utilization

For overlapping CA solution, the legacy channel bandwidth should be supported, hence the minimum guardband should not be less than the minimum guardband of lower UE channel bandwidth than operator licensed bandwidth.

The second aspect is channel positions. In order to support legacy UEs, channel raster should be applied for each UE channels. And for solution 2 overlapping CA, RB alignment is not needed and only subcarrier alignment is required. As a consequence, the channel spacing is an in integer multiple of 300 KHz. The points for RB alignment is still within the full set of channel spacing.

Channel spacing=2*BWChannel-(BWChannel1+BWChannel2)0.60.3 (MHz)

With the channel spacing decided, the spectrum utilization can be calculated. Table 2-1 show the channel spacing and spectrum utilization for example channel bandwidth combinations.

Table 2-1: channel spacing and spectrum utilization for solution 2

| Band (s) | Channel Raster | Channel Bandwidth(MHz) | SCS(kHz) | CA combos | Nominal channel spacing (MHz) | Transmission bandwidth configuration NRB | SU(%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 100 kHz | 7 | 15 | 5+5 | 1.8 | 35 | 90 |
|  |  | 11 | 15 | 10+10 | 0.9 | 57 | 93.3 |
|  |  | 12.5 | 15 | 10+10 | 2.4 | 65 | 93.6 |
| n12 | 100 kHz | 12 | 15 | 10+5 | 4.5 | 63 | 94.5 |
| n26 | 100 kHz | 7 | 15 | 5+5 | 1.8 | 35 | 90 |
| n28 | 100 kHz | 13 | 15 | 10+10 | 3 | 68 | 94.2 |
| n29 | 100 kHz | 6 | 15 | 5+5 | 0.9 | 30 | 90 |
|  | 100 kHz | 11 | 15 | 10+10 | 0.9 | 57 | 93.3 |

SSB

For the case the overlapping part can contain the SSB, single common SSB is preferred since it avoid the overhead of multiple SSBs. The approach can also apply to overlapping CA solution, where the SSB is only configured for Pcell, especially for the case new UE is the majority. And for the case the size of overlapping part is too narrow to contain the SSB, two SSBs are needed to allow legacy UEs to operate in both RF channels. And separate SSBs can be configured in frequency domain or time domain with the consideration of bandwidth size. If it is allowed that legacy UE only access in one of the RF channels, single SSB is only configure at one of the RF channels. The configuration of SSB can be left to network and no specification impact is needed.

Observation 2: The configuration of SSB can be left to network and no specification impact is needed.

RF requirements

BS RF requirements

The primary assessment on each BS RF requirement is shown in Table 2.5-1

Table 2.5-1: analysis on the impact to BS core specification

| Subject | Clause in 36.104 | Requirement | Assessment |
| --- | --- | --- | --- |
| Transmitter | 6.1 | General | no change |
|  | 6.2 | Base station output power | no change |
|  | 6.3.2 | RE Power control dynamic range | no change |
|  | 6.3.3 | Total power dynamic range | no impact, no limits defined for CA |
|  | 6.4 | Transmit ON/OFF power | no change |
|  | 6.5 | Transmitted signal quality | no change |
|  | 6.6.1 | Occupied bandwidth | No change for the requirement |
|  | 6.6.2 | ACLR | no change, it is defined for the edge carrier |
|  | 6.6.3 | Operating band unwanted emissions | no change, one uniform emission mask is defined and can be reused. |
|  | 6.6.4 | Transmitter spurious emissions | no change |
|  | 6.7 | Transmitter intermodulation | no change, it is defined for the edge carrier |
| Receiver | 7.1 | General | no change |
|  | 7.2 | Reference sensitivity level | no change, reference sensitivity level including FRC can be re-used |
|  | 7.3 | Dynamic range | no change, the requirements is defined per channel BW. |
|  | 7.4 | In-band selectivity and blocking | no change, it is defined for the edge carrier |
|  | 7.5 | Out-of-band blocking | no change |
|  | 7.6 | Receiver spurious emissions | no change |
|  | 7.7 | Receiver intermodulation | no change, it is defined for the edge carrier |
|  | 7.8 | In-channel selectivity | no change, the requirements is defined per channel BW. |

UE RF requirements

The primary assessment on each UE RF requirement is shown in Table 2.5-2

Table 2.5-2: analysis on the impact to UE core specification

| Subject | Clause in 38.101-1 | Requirement | Initial assessment |
| --- | --- | --- | --- |
| Transmitter requirements | 6.2A.1 | UE maximum output power for CA | N/A, overlapping CA from UE perspective is only used in the DL |
|  | 6.2A.2 | UE maximum output power reduction for CA | N/A, overlapping CA from UE perspective is only used in the DL |
|  | 6.2A.3 | UE additional maximum output power reduction for CA | N/A, overlapping CA from UE perspective is only used in the DL |
|  | 6.2A.4 | Configured output power for CA | N/A, overlapping CA from UE perspective is only used in the DL |
|  | 6.3A | Output power dynamics for CA | N/A, overlapping CA from UE perspective is only used in the DL |
|  | 6.3A.3 | Transmit ON/OFF time mask for CA | N/A, overlapping CA from UE perspective is only used in the DL |
|  | 6.4A | Transmit signal quality for CA | N/A, overlapping CA from UE perspective is only used in the DL |
|  | 6.5A | Output RF spectrum emissions for CA | N/A, overlapping CA from UE perspective is only used in the DL |
| Receiver requirements | 7.3A.2 | Reference sensitivity power level for CA | the framework can be reused with that it is measured carrier by carrier. |
|  | 7.4A | Maximum input level for CA | the framework can be reused with that it is measured carrier by carrier. |
|  | 7.5A | Adjacent channel selectivity for CA | the framework can be reused with that it is measured carrier by carrier. |
|  | 7.6A | Blocking characteristics for CA | the framework can be reused with that it is measured carrier by carrier. |
|  | 7.7A | Spurious response for CA | No change |
|  | 7.8A | Intermodulation characteristics for CA | the framework can be reused with that it is measured carrier by carrier. |
|  | 7.9A | Spurious emissions for CA | No change. |

Observation 3: The impact to RF core requirements is very limited to support overlapping CA.

Impact to other WG

NR provides the flexibility to avoid any collisions of SSB, CORESET and reference signals between two overlapping CCs in frequency domain or time domain. The network can avoid the collision of PDSCHs between two CCs.

From procedure wise, the network can indicate UE the location and bandwidth of BWP by using RRC signaling on individual CC. The procedure applies to each CC (serving cell) for intra-band overlapping CA. Thus network can indicate the channel bandwidth for each CC.

Regarding measurement, the normal RRM/RLM measurement and RAN2 procedures can be followed for intra-band overlapping CA from network perspective. For intra-band overlapping CA from UE perspective, the same RRM/RLM measurement and RAN2 procedure including SCell addition/release can apply. As mentioned above the network can make use of the flexibility provided by NR and the proper scheduling to avoid any collision of SSBs and other reference signals between two CCs. Thus there is no impact on RRM/RLM measurement. There is no necessity to restrict the relation between CCs in terms of time or frequency synchronization, QCL.

The only thing needs be done would be to introduce some capability signaling for the new UE.

Observation 4: there is no impact on RAN1 and RAN2 of intra-band overlapping CA to support the irregular channel bandwidth except for some capability signaling for the new UE.

Conclusions

In the contribution, we provide discussion on overlapping CA (from both UE and network perspective):

Observation 1: for channel bandwidths less than 50 MHz, integer-multiples of 5MHz channel bandwidths are supported/will be supported in BS/UE specifications.

Proposal 1: New dedicated channel bandwidths are not considered for both BS and UE.

Observation 2: The configuration of SSB can be left to network and no specification impact is needed.

Observation 3: The impact to RF core requirements is very limited to support overlapping CA.

Observation 4: there is no impact on RAN1 and RAN2 of intra-band overlapping CA to support the irregular channel bandwidth except for UE capability signaling.

In summary, for the overlapping CA, the following benefits is identified,

- No new channel filter need to be designed

- CA framework from both standard and implementation perspectives can be reused and the impact to RF core is very limited

- There is no impact on RAN1 and RAN2 except for UE capability signaling.

- Support legacy UE to use smaller channel bandwidth in both sides

References

RP-202103, New SID: Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths, T-Mobile USA, Ericsson

R4-2101507, Consideration for overlapping UE channel bandwidths, Huawei

R4-2103264, Way forward on overlapping CBW method, Nokia

R4-2105419, Way forward on Evaluation of IrregularBW Approaches,