3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2110678

Electronic Meeting, 19– 27 May, 2021

Source:	Huawei, HiSilicon, Bell Mobility, Telus

Title:	TP for TR 38.717-04-02: CA_n5-n25-n66-n78

Agenda item:	8.14.2

Document for:	Approval

Background

This contribution provides text proposal for TR 38.717-04-02 to include CA_n25A-n71A-n78 as updated in the latest WID.

Text Proposal

---Start of changes---

5.1.x	CA_n5-n25-n66-n78

5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: Inter-band CA operating bands of CA_n5-n25-n66-n78

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n5-n25-n66-n78 | n5, n25, n66, n78 |

5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25A-n66A-n78A | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25(2A)-n66A-n78A | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25A-n66(2A)-n78A | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25A-n66A-n78(2A) | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25(2A)-n66(2A)-n78A | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25(2A)-n66A-n78(2A) | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25A-n66(2A)-n78(2A) | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25(2A)-n66(2A)-n78(2A) | CA_n5A-n25A CA_n5A-n66A CA_n5A-n78A CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

---End of changes---